3GPP TSG-RAN WG4 Meeting #117	R4-2521129

# Dallas, USA, Nov. 17-21, 2025

Source:	Ericsson

Title:	Simulation results for NR ATG enh demodulation

Agenda item:	6.7.4

Document for:	Information

# Introduction

In RAN4 #116bis meeting on Rel-19 ATG enhancement, companies have agreed most of parameters of downlink CA and MIMO, and all parameters for PUSCH demodulation.

In this contribution, we provided our initial simulation results to better facilitate the parameter assumption for the UE downlink CA and DL/UL MIMO requirements. For PUSCH 2Tx, both ideal and impairment results are delivered.

# Simulation

## 2.1	UE simulations

### 2.1.1	Downlink CA

In this section, we first exhibited the simulation assumption that we used for evaluation, then we shared our simulation results for all bandwidths.

For downlink CA simulations, we reuse the ATG channel model (AWGN+220Hz for FDD 15kHz，AWGN+500Hz for TDD 30kHz) on top of the common test parameters for CA below:

Table 2.1.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Frequency |  | Hz | 2.1G |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15kHz SCS: FR1.15-1 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases where the number of ACK/NACK to be transmitted on single PUCCH is 2 or less. PUCCH format 3 for cases where the number of ACK/NACK to be transmitted on single PUCCH is more than 2. |

Here we provided our simulation results for all FDD and TDD bandwidths.

For our results, rank 1 and legacy TDD pattern were assumed.

Table 2.1.1-2 Single carrier performance for FDD 15kHz SCS for CA configurations (2Rx)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 |  | 64QAM, 0.65 (MCS22) | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 10 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 15 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 20 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 25 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 30 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 35 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 40 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 45 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 50 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |

Table 2.1.1-3 Single carrier performance for FDD 15kHz SCS for CA configurations (4Rx)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 |  | 64QAM, 0.65 (MCS22) | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.8 |
| 10 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.6 |
| 15 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.8 |
| 20 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.6 |
| 25 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 30 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.6 |
| 35 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.6 |
| 40 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.6 |
| 45 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.8 |
| 50 |  | 64QAM, 0.65 | AWGN+220Hz Frequency offset | 2x4, ULA Low | 70 | 7.8 |

Table 2.1.1-4 Single carrier performance for TDD 30kHz SCS for CA configurations(2Rx)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 |  | 64QAM, 0.65 (MCS22) | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 10 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |
| 15 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 20 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 25 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.9 |
| 30 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 40 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 50 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 60 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 80 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 90 |  | 64QAM, 0.65 (MCS22) | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.6 |
| 100 |  | 64QAM, 0.65 (MCS22) | AWGN+500Hz Frequency offset | 2x2, ULA Low | 70 | 10.5 |

Table 2.1.1-5 Single carrier performance for TDD 30kHz SCS for CA configurations(4Rx)

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 |  | 64QAM, 0.65 (MCS22) | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.6 |
| 10 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 15 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 20 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 25 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 8.0 |
| 30 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 40 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 50 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 60 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 80 |  | 64QAM, 0.65 | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.6 |
| 90 |  | 64QAM, 0.65 (MCS22) | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |
| 100 |  | 64QAM, 0.65 (MCS22) | AWGN+500Hz Frequency offset | 2x4, ULA Low | 70 | 7.5 |

### 2.1.2	Downlink MIMO

In this section, we first exhibited the simulation assumption that we used for evaluation, then we shared our initial simulation results with candidate parameter assumptions.

Table 2.1.2-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Simulation results are summarized below:

Table 2.1.2-2 Evaluation cases(FDD)

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 2 | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 13.5 |
| 1-2 | 2 | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x2 | 70 | 22.46 |
| 2-1 | 2 | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 10.5 |
| 2-2 | 2 | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x4 | 70 | 19.45 |

Table 2.1.2-2 Evaluation cases(TDD)

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 2 | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 13.45 |
| 3-2 | 2 | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x2 | 70 | 22.45 |
| 4-1 | 2 | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 10.43 |
| 4-2 | 2 | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x4 | 70 | 19.43 |

# Conclusions

In this paper, we submitted our simulation results for DL CA and DL MIMO for ATG scenario.

# References

[1] 	R4-2514769, WF on NR_ATG_enh demodulation requirement, CMCC, RAN4 #116bis, Prague