3GPP TSG-RAN WG4 Meeting #117	R4-2521130

Dallas, Tx, USA, 17th – 21st November 2025

Agenda Item:	7.5.4

Source:	Ericsson

Title:	Discussion on demodulation and CSI reporting requirement for 6Rx with interference

Document for:	Discussion

# 1	Introduction

In RAN#109 meeting, a new WID on NR demodulation performance: Phase 6 was approved [1]. According to the WID, following are the objectives for UE part:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

With Rel-19 introducing UE RF enhancements supporting 6Rx reception, a consistent methodology for modeling both inter-cell interference and intra-cell inter-user interference becomes increasingly important. Existing Rel-17 and Rel-18 test models (INR-based interference representation) provide a mature framework for benchmarking but were defined mainly for 2/4/8Rx UE categories. It is natural to extend the interference mitigation solution to 6Rx UE/devices.

In this contribution, we provided our views on the scope of demodulation and CSI reporting requirements for two different interference scenarios, together with the discussion on the interference model and detailed parameter assumptions.

# 2	General

Coverage of UE types

It’s been agreed in Rel-19 6Rx WI to consider only one requirement to cover both handheld UE type and FWA type. Thus, for the extended topic: 6Rx with interference, we propose to reuse the above agreement and define only a single requirement for both handheld and FWA devices.

Proposal 1: Define only a single requirement for both handheld and FWA devices.

Test scope

Regarding the test scope, following the guidance of the WID, RAN4 will define PDSCH demodulation for the inter-cell interference scenario. As for the related CQI reporting requirements, a conclusion from WI Rel-19 NR demodulation performance: Phase 5 is needed that is whether to define CQI reporting requirements for 6Rx (without interference). As the latest agreement in Rel-19 6Rx WI, it’s been agreed to introduce CQI reporting requirements with 6Rx reception.

In this case, for the test scope, we propose the following:

Proposal 2: Define CQI reporting requirements for inter-cell interference scenario

Antenna correlation

It’s been agreed in Rel-19 6Rx WI to consider the MIMO correlation with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA. We are of the opinion to reuse the above agreement for the extended topic: 6Rx with interference. However, further evaluation is needed to see the performance in order to make sure that such high correlation will not impact too much while adding the interference.

Proposal 3: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA, and then make down-selection

# 3	Inter-cell interference

In this section, we discuss the PDSCH requirements for the scenario of inter-cell interference cancellation with 6Rx reception.

Following the objectives, we propose to take the legacy Rel-17 inter-cell interference parameter configurations as baseline and do the evaluation. Then, make adjustment accordingly to fit more for defining PDSCH requirement with inter-cell interference for 6Rx reception.

Following agreements have been made during the discussion in RAN4#116bis meeting [2]:

Test scenarios and interference profile - Agreement: - Cover both HomNet and HetNet scenarios - Interference modelling: Reuse the same interference modeling defined for Rel-17 2/4Rx and Rel-19 8Rx: - INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet (if covered) - INR1 = 7.58dB for HetNet (if covered) - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Random 16QAM modulated symbols for the interference cell(s) - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell). Rank for the serving cell - Agreement: - For initial evaluation purpose, cover rank 1,2 and 4 And make further selection based on simulation. Antenna configuration - Agreement: - For initial simulation purpose, 2T6R for rank 1/2 and 4T6R for rank 4

As for the rank, antenna, MCS and channel model, RAN4 can take the following existing requirements as a reference:

Table 2.3-1 Existing requirements in TS38.101-4

|  | Rank | MCS | Antenna | Propagation |
| --- | --- | --- | --- | --- |
| Rel-17 PDSCH requirements with inter-cell interference, 2/4Rx | 1 | 13 | 2x2, 2x4 ULA Low | TDLC300-100 |
|  | 1 | 13 | 2x2, 2x4 ULA Low | TDLA30-10 |
| Rel-19 PDSCH requirements with interference, 8Rx | 2 | 17 | 2x8, ULA low | TDLA30-10 |
|  | 4 | 13 | 4x8, ULA low | TDLA30-10 |
| Rel-19 PDSCH requirements, 6Rx | 4 | 17 | 4x6, Correlation TBD | TDLA30-10 |

In the meantime, we would prefer to consider only one case to cover both handheld UEs and FWA devices as RAN4 usually consider ULA low correlation for interference mitigation requirements.

Based on the above existing requirements and one-case principle, we propose at least the following cases for evaluation purpose, down-selection could be applied depending on the performance and further discussions.

Proposal 4: Consider following cases for initial evaluations:

| Eva. num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1,2 | 1 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 2x6, low | 70 |  |
| 3,4 | 2 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 2x6, low | 70 |  |
| 5,6 | 4 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 4x6, low | 70 |  |

UE capability and test applicability can be discussed and decided (if necessary) after RAN4 reaches agreements on the scope and basic parameter assumptions.

# 4	Intra-cell inter-user interference

In this section, we discuss the PDSCH requirements for the scenario of intra-cell inter-user interference cancellation with 6Rx reception.

Following agreements have been reached during the discussion in RAN4#116bis meeting [2]:

MU-MIMO Beamforming modeling - Agreement: - Reuse MU-MIMO Beamforming in TS 38.101-4 Annex B.4.2 Signal power assumptions and SNR assumptions - Agreement: - Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal - SNR = (STargetUE+ SInterfUE)/N Antenna configuration and rank for target + co-scheduled UE - Agreement: - For initial evaluation purpose, cover Rank 1+1 (with 2T6R) and Rank 2+2 (with 4T6R). - Select rank 2+2 if simulation results show feasibility Other test parameters and simulation assumptions - Agreement: - 1 Co-scheduled UE number - Precoding for the target UE: Random Single Panel Type I precoding - 2PRBs for PRB bundling size and precoding granularity - PDCCH and PDSCH allocation in time domain - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 - PDSCH allocation in frequency domain - Full PRB.

Except the agreements above, companies would also need to decide the propagation condition, precoding method for the co-scheduled UE and MCS for the target UE. Following the objective, we propose to take the legacy Rel-17 intra-cell inter-user interference parameter as the baseline and do the evaluation, then update it with proper parameter assumptions if needed for the purpose of introducing PDSCH requirement.

Specifically, to evaluate the both TDLA30-10 and TDLC300-100 for different MCS and rank combinations. Decisions can be made based on companies results in the following meetings.

For the propagation condition, option 3 in the following agreement can be assumed for the initial simulations.

Propagation condition - Candidate options: - Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

Proposal 5: Evaluate the performance for TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

Regarding the MCS for the target UE, we prefer to evaluate both option 1 and option 3 in the following agreement:

MCS for the target UE - Candidate options: - Option 1: MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case - Option 2: MCS 17 (Table 1) for 2+2 case - Option 3: Cover MCS 13 and MCS 17 for evaluation

Proposal 6: Evaluate the performance of MCS13, MCS17 and MCS19

# 5	CQI reporting requirements for inter-cell interference

If the CQI reporting requirements with interference can be agreed to be introduced, we propose to reuse the Rel-17 interference models and do the evaluations. We also propose to reuse the test metric for 2Rx, 4Rx and 8Rx, that is:

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 7: Reuse the Rel-17 interference models for initial evaluations

Proposal 8: Reuse the test metric for 2Rx, 4Rx and 8Rx

# 6	Summary

In this contribution, we shared our views on the test scope, interference model and parameter assumptions for inter-cell interference and intra-cell inter-user interference scenarios. Our proposals are summarized as follows:

Proposal 1: Define only a single requirement for both handheld and FWA devices.

Proposal 2: Define CQI reporting requirements for inter-cell interference scenario

Proposal 3: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA, and then make down-selection

Proposal 4: Consider following cases for initial evaluations:

| Eva. num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1,2 | 1 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 2x6, low | 70 |  |
| 3,4 | 2 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 2x6, low | 70 |  |
| 5,6 | 4 | 10 / 15 | MCS 17 + MCS 13(Table 1) | TDLA30-10 | 4x6, low | 70 |  |

Proposal 5: Evaluate the performance for TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

Proposal 6: Evaluate the performance of MCS13, MCS17 and MCS19

Proposal 7: Reuse the Rel-17 interference models for initial evaluations

Proposal 8: Reuse the test metric for 2Rx, 4Rx and 8Rx

# References

[1]	RP-252957: WID on NR demodulation performance: Phase 6, China Telecom, BT, RAN#109, Beijing, China

[2]	R4-2514823: WF on NR_demod_Ph6_Ph5, China Telecom, RAN4#116bis, Prague, October 2025

[3]	TS38.214