TSG-RAN Working Group 4 (Radio) meeting #117	  R4-2521132

Dallas, TX, USA, 17 – 21 November 2025

Source:	Ericsson

Title:	On the feasibility of n5-n8

Agenda item:	5.9.3

Document for:	Approval

# Background

In the TP [1] for introducing the UL CA_n5A-n8A in 38.793 is claimed “[…] that there is no RAN1/RAN2’s spec impacts beyond the introduction of UE capability for the proposed solution with non-concurrent n5 DL and n8 UL for both RF implementation 1 and RF implementation 2.”

It may not be that simple.

Considering the feasibility of the RF architecture and the complexity of n5 DL and n8 UL coordination, we propose that UL CA_n5A-n8A with its overlapping bands is not specified by means of a UE capability for non-simultaneous Rx-Tx between n5 DL and n8 UL, or ‘non-concurrent inter-band UL-DL operation’ in general.

# Support of the n5-n8 combination

The 38.872 [2] contains several candidate implementations of the n5-n8 band combination with non-simultaneous Rx-Tx between n5 DL and n8 UL. One of them is shown in Figure 1 below -- but is this implementation viable? The overlapping 5 DL filter on the main antenna would load (pull on) the n8 UL filter on the diversity antenna through coupling and distort the n8 UL passband even though simultaneous Rx-Tx would be avoided by scheduling. The coupling would obviously depend on the UE antenna implementation, but for low bands the coupling is often high between antennas.

Figure 1: a two-antenna implementation with a triplexer.

Conversely, if the UE is monitoring/receiving on the n5 DL, its passband would be distorted by the overlapping n8 UL filter unless there is a switching behavior specified for the architecture above. We observe

Observation 1: for a two-antenna implementation of the n5-n8, the overlapping 5 DL filter would load (pull on) the n8 UL filter through antenna coupling and distort the passband despite non-simultaneous Rx-Tx between n5 DL and n8 UL. The antenna coupling depends on the UE implementation but is often high for low bands.

In general, filters that overlap pull on each other, distort the pass bands and result in significant increase in insertion loss. The overlap in this case is > 50% of the filter passband. The same observation can be made for the other architectures assumed in 38.872.

The RFFE switching discussed in [1] occurs in the diversity branch of the architecture in Figure 1, presumably; “The filter design challenge however would be shifted to the diversity TRx path duplexer due to the wider band range to cover n5 DL and n8 UL”.

# The non-simultaneous Rx-Tx operation between n5 DL and n8 UL

A UE capability for non-simultaneous Rx-Tx between n5 DL and n8 UL could indeed be specified in 38.306 by RAN2, but the scheduler implementation in NW would be complex with uncertain gains of 2UL operation. For an n5 Pcell, RRC signaling and other common handling like paging and system information on the Pcell would have to be blocked during n8 UL transmissions. Likewise, blocking the n8 UL during n5 DL transmission would be non-trivial given the scheduling time offset before the transmission OTA.

In the example given in the TP [1] this also preceded byan “RFFE reconfiguration” after the n8 UL transmission.

An example of non-simultaneous Rx-Tx operation for n5-n8 is given in the TP [1]. Besides standard RRC reconfigurations for adding an n8 DL-only Scell and then a modification to add an UL, the procedure also includes an “RFFE reconfiguration” of the UE RF front-end that is assumed to occur during a slot before and after n8 UL transmissions:

Step 1: Access n5 as PCell

Step 2: RRC reconfiguration to Case 2: Add n8 with only n8-DL as SCell  UL Tx in SCell via n5-UL

  - RFFE configuration is #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx

Step 3: RRC reconfiguration to Case 1: PCell remains at n5, reconfigure SCell n8 by adding n8-UL

  - Freedom of RFFE configuration

    - Alt.#1: Not changed, remaining as #A: (n5-UL: Tx, n5-DL:Rx, n8-DL:Rx)

    - Alt#2: Or reconfigured to #B: (n5-UL:Tx, n8-UL:Tx, n8-DL:Rx)

Step 4: When n8-UL needs to be scheduled from Slot #N+k+2 to Slot #N+m (where m=18 for the illustration),

  - NW to choose a slot #N+k or before slot #N+k, where DCI issued to the UE that n8-UL transmission happens at slot #N+k+2

    - k is chosen

      - one slot switching period is large enough so the UE can make successful RFFE reconfiguration to Alt. #2

      - There will be no SSB/PDCCH/CSI-RS on n5-DL to be received by the UE between Slot #N+k+1 and #N+m+1 (where m=18 for the illustration)

    - The DCI also schedules n5-UL between Slot #N+k+1 and #N+m+1 (where m=18 for the illustration) via 1 TB mapping to multiple slots via features:

      - Repetitive (Rel-15)

      - Non-repetitive (Rel-17)

Hence n5-UL transmission during this period is not impacted.

  - UE to complete reconfiguration of RFFE between Slot #N-k and #N (k is chosen to guarantee that no n5-DL reception is required)

  - UE to make UL transmission at n8-UL at Slot #N until Slot #N+m

  - Once UE complete n8-UL transmission, reconfigure RFFE from #B to #A immediately with one slot switching period.

  - There is no uplink transmission on n8-UL for the UE between Slot #N and Slot #N+k

Step 5: Repeat Step 4 if new n8-UL transmission is required.

The UE is not expected to receive on n5 during the “RFFE reconfiguration” in the slot following the n8 UL transmission scheduled by DCI on n8 or n5 (cross-carrier and multi-slot). This appears like expected UE behaviour that would require RAN1 specification, not only a UE capability as claimed in [1]:

We make the following

Observation 2: the proposed non-simultaneous Rx-Tx between n5 DL and n8 UL is non-trivial; RRC signaling and other common handling on the n5 Pcell would have to be blocked during n8 UL transmissions, and blocking of the n8 UL during n5 DL transmission would be complicated by e.g. scheduling time offsets.

Furthermore, the proposed “RFFE reconfiguration” is not specified. The gain of the 2UL would be uncertain notwithstanding the problem with overlapping band filters. Hence, support of the feature is not only a question of defining a UE capability for non-simultaneous Rx-Tx:

Proposal 1: given the uncertain gain and performance of the proposed CA_n5A-n8A with 2UL operation without a specified switching behavior, a UE capability for non-simultaneous Rx-Tx support for this band combination (exceptional for FDD-FDD) is not specified.

# Another alternative for a shared n5-n8 network

A shared n5-n8 network with collocated n5 and n8 sites is nevertheless a great asset. One alternative is to support DL-only CA_n5A-n8A with CA_n5A in the UL in such a network, the CA users sharing the common network resources with non-CA users on n8. Hence the total offered UL traffic in the cells can be coordinated thus improving the system performance, but without the complex 2UL operation in the same UE.

The configuration CA_n5A-n8A with CA_n5A in the UL is feasible, also without LB-LB switching.

Observation 3: an alternative to the complex 2UL operation for a shared n5-n8 network with collocated sites is to support CA_n5A-n8A with CA_n5A in the UL, the CA users sharing the common network resources with non-CA users in band n8.

# Proposal

We observe that

Observation 1: for a two-antenna implementation of the n5-n8, the overlapping 5 DL filter would load (pull on) the n8 UL filter through antenna coupling and distort the passband despite non-simultaneous Rx-Tx between n5 DL and n8 UL. The antenna coupling depends on the UE implementation but is often high for low bands.

Observation 2: the proposed non-simultaneous Rx-Tx between n5 DL and n8 UL is non-trivial; RRC signaling and other common handling on the n5 Pcell would have to be blocked during n8 UL transmissions, and blocking of the n8 UL during n5 DL transmission would be complicated by e.g. scheduling time offsets.

Hence, support of the feature is not only a question of defining a UE capability for non-simultaneous Rx-Tx:

Proposal 1: given the uncertain gain and performance of the proposed CA_n5A-n8A with 2UL operation without a specified switching behavior, a UE capability for non-simultaneous Rx-Tx support for this band combination (exceptional for FDD-FDD) is not specified.

Nevertheless, a shared n5-n8 network with collocated n5 and n8 sites is a great asset:

Observation 3: an alternative to the complex 2UL operation for a shared n5-n8 network with collocated sites is to support CA_n5A-n8A with CA_n5A in the UL, the CA users sharing the common network resources with non-CA users in band n8.

# References

1.    R4-2513169, “TP to TR 38.793 to introduce UL CA_n5-n8”, CATT