3GPP TSG-RAN WG4 Meeting #117	                                   R4-2521144

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.10.3

Source: 	ZTE Corporation, Saneships

Title: 	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Approval

Introduction

In the RAN #103 meeting, the WID on several enhancements for NR and MR-DC RRM requirements was approval. There are two objectives need to be discussed in Rel-19, one is FR2-1 SSB based L3 measurement delay reduction for connected mode, and the other is Fast SCell activation for UE supporting Rel-18 EMR. In this meeting, we should discuss performance part on fast SCell activation for UE supporting R18 EMR from this meeting according to schedule [1].

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

Discussion

During the discussion on core part, RAN4 has agreed that both single direct activation and MAC CE based single SCell activation are supported at SCell addition. Meanwhile, MAC CE based PUCCH SCell activation is included. Therefore, RAN4 should discuss the test cases for the requirements we have defined on core part. Based on this, there are six test cases will be intriduced to verify UE’s behavior [2].

Issue 2-1-2: Applicability rules and number of test cases Agreements: - Further discussion applicability rules, taking the table containing TCs for EMR based SCell activation as reference. TC No. (CR No.); Scenario. 1; Normal SCell activation + FR1 + Configuration 1. Normal SCell activation + FR1 + Configuration 2. Normal SCell activation + FR1 + Configuration 3. 2; Normal SCell activation + FR2 + Configuration 1. Normal SCell activation + FR2 + Configuration 2. Normal SCell activation + FR2 + Configuration 3. 3; Direct SCell activation at SCell addition + FR1 + Configuration 1. Direct SCell activation at SCell addition + FR1 + Configuration 2. Direct SCell activation at SCell addition + FR1 + Configuration 3. 4; Direct SCell activation at SCell addition + FR2 + Configuration 1. Direct SCell activation at SCell addition + FR2 + Configuration 2. Direct SCell activation at SCell addition + FR2 + Configuration 3. 5; PUCCH SCell activation + FR1 + Configuration 1. PUCCH SCell activation + FR1 + Configuration 2. PUCCH SCell activation + FR1 + Configuration 3. 6; PUCCH SCell activation + FR2 + Configuration 1. PUCCH SCell activation + FR2 + Configuration 2. PUCCH SCell activation + FR2 + Configuration 3. Way forward: - Discuss the following applicability rules as a starting point: - Agree the following testing applicability rules: - Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped. - Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped. - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped. - Other applicability rules are not precluded. - Note: Any agreed testing applicability rules will be captured in clause A.3.X in 38.133.

During the disscussion in core part, the requirements have been modified for the case where the target SCell is unknown and meets the conditions and the requirenebts refer to 8.3.2A. Therefore, we think that both single direct activation and MAC CE based single SCell activation, should be considered in the design of test cases due to RAN4 needs to guarantee test coverage if UE may only supports some of scenarios. Meanwhile, some applicability rules can be defined and maybe it’s enough for UE to pass one of them due to the requirements of unknown SCell case are the same for these three scenarios and do not need the repeated validation.

The applicability rules mentioned in way forward looks fine for us. For these three scenarios, normal SCell activation is a baseline scenario, and direct SCell activation and PUCCH SCell activation are enhancement scenario. If UE only supports normal SCell activation, TC1 or TC2 should be used. If UE supports normal SCell activation and direct SCell activation or PUCCH SCell activation, the latter one should be tested and UE can pass the former one directly if UE behavior is right for the latter scenario. Similiar principle, for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped. Due to R18 early measurement report has been included in the R19 test case, corresponding R18 test case with valid report can be skipped if UE can pass R19 test case for fast SCell activation based on R18 EMR.

The following testing applicability rules are applied for Rel-19 fast SCell activation based on measurement in IDLE/INACTIVE mode

- Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

- Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

- Rule 3: for UE which can pass R19 test case for fast SCell activation based on measurement in IDLE/INACTIVE mode, corresponding test case with report can be skipped.

Conclusions

In this paper, we have some discussion on fast SCell activation for UE supporting Rel-18 EMR. And we have the following proposal:

Proposal 1: The following testing applicability rules are applied for Rel-19 fast SCell activation based on measurement in IDLE/INACTIVE mode

- Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped.

- Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped.

- Rule 3: for UE which can pass R19 test case for fast SCell activation based on measurement in IDLE/INACTIVE mode, corresponding test case with report can be skipped.

References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2514801, WF on NR_RRM_Ph5_Part2, CATT