# 3GPP TSG-RAN WG4 Meeting #117                                    R4-2521186

# Dallas, USA, Nov. 17-21, 2025

# Title: 	Discussion on UE and BS demodulation performance requirements for SBFD

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.13.4

# Document for:	Approval

# 1 Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The objectives related to RAN4 core and performance parts are listed.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles) - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] The objectives are as follows [RAN4]: - Specify BS RF conformance requirements to support SBFD operation at gNB - Specify necessary RRM performance requirements to support SBFD operation - Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation

In this contribution, we give some initial discussion on BS and/or UE demodulation performance requirements to support SBFD operation according to the last meeting agreements [2].

# 2 Discussion

BS demodulation requirements

How to model interference modeling for performance evaluation

- If RAN4 to define Rel-19 SBFD PUSCH requirement with UL muting only considering inter-site gNB-gNB interference, self-interference and intra-site inter sector interference are not taken into account. - For inter-site gNB-gNB interference modelling - Considering the RF requirement, the following INR values derived based on link budget analysis can be considered for evaluation. Based on evaluation results, RAN4 can further discuss whether to introduce the PUSCH requirement with UL muting. Table 2-1 Revised INR values for FR1 Conditions; FR1. Interf1 > Interf2; INR1 <= 9.2dB, INR2 <= -0.8dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1]} dB. Interf1 = Interf2; INR1 = INR2 <=3.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3]} dB. Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.. - Not pursue to derive INR based system level simulation for inter-site gNB-gNB interference modelling when defining PUSCH requirement with UL muting for SBFD in Rel-19.

Regarding how to model interference modeling, currently, considering the limited remain time for SBFD demod work, some companies provided a simple approach to derive the interference profile, link budget. Based on our understanding, we think that is more like a worst-case scenario assumption. Because in the gNB side, the coordination of gNBs or beam level coordination will be assumed for SBFD operation. From this point of view, the actual interference profiles will be less than the value of link budget. The motivation of system simulation level is to evaluate the interference profile based on the more comprehensive analysis and scheduling. However, the key issue is how to handle the beam coordination between gNBs. Currently, gNB-gNB co-channel interference also could be suppressed beam management, such as Tx beam nulling, etc. If RAN4 wants to evaluate the gNB-gNB interference, this beam coordination issue should be discussed. Different beam coordination means different interference level. As mentioned before, consider the limited time, which will be a challenge for the timeline. Therefore, we are fine to use this simple approach to derive the interference profiles.

Proposal 1. Propose to consider the link budget approach to derive the interference profiles if the SLS cannot be modeled.

Whether to introduce the PUSCH requirement with UL muting

- FFS on whether RAN4 introduce the PUSCH requirement with UL symbols muting.

Currently, companies have different understandings on interference profile modeling. We understand that the link budget approach is a simple approach to derive interference profiles when the SLS cannot be modeled exactly. Based on the last meeting simulation results, the performance gain of UL muting is very limited compared with the legacy approach. From the point of processing, different symbols are muted when BS suffered the interference. Thus, introducing the PUSCH requirements with UL muting in an interference scenario could be considered.

Proposal 2. Propose to introduce the PUSCH requirement with UL muting in an interference scenario.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of SBFD demodulation requirements, the conclusions are:

Proposal 1. Propose to consider the link budget approach to derive the interference profiles if the SLS cannot be modeled.

Proposal 2. Propose to introduce the PUSCH requirement with UL muting in an interference scenario.

# 4	Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei.

- R4-2515038, Way Forward for [116bis] [311] Rel-19 Demodulation_Part1, Samsung.