# 3GPP TSG-RAN WG4 Meeting #117                                    R4-2521191

# Dallas, USA, Nov. 17-21, 2025

# Title: 	Simulation results on performance requirements for 6Rx UE with interference

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.4

# Document for:	Discussion

# 1 Introduction

In RAN #109 meeting, the new WID of UE performance requirements for 6Rx UE devices (both handheld and FWA), which was approved in [1], wherein the objectives related to 6Rx UE are listed as below:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

In this contribution, we give some initial simulation results on the UE performance requirements for 6Rx, which including inter-cell interference cancellation based on the lasting meeting conclusion [2].

# 2 Discussion

In last meeting, RAN4 discussed whether introduce two set requirements for baseline and simplified receivers. Considering that there may be differences in performance of different receivers. The detail simulation assumptions as shown in following Table 2-1.

We evaluated HomNet and HetNet scenarios with different MCS, propagation conditions and different Rank, etc.

The SNR corresponding to 70% maximum throughput is shown in Table 2-2 and Table 2-3.

## Inter cell interference cancellation

Table 2-1: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled Disabled for HetNet |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for HomNet N/A for HetNet |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| INR |  | dB | N/A | 7.77 for HomNet 7.58 for HetNet | 2.29 for HomNet |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |

Table 2-2: SNR corresponding to 70% maximum throughput for HomNet scenarios

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel model | MMSE-IRC SNR (dB) | MMSE SNR (dB) | Gain MMSE-IRC vs MMSE (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 40M/30kHz | 2T6R | Rank1, MCS17 | TDLC 300-100 Low | 6.6 | 11.2 | 4.6 |
|  | 2T6R | Rank2, MCS17 | TDLA 30-10 Low | 10.9 | Inf | Inf |
|  |  |  | TDLA 30-10, α= 0 and β = 0.07179 | 18.0 | Inf | Inf |
|  | 4T6R | Rank 4, MCS 13 | TDLA 30-10 Low | 14.3 | Inf | Inf |
|  |  |  | TDLA 30-10, α= 0 and β = 0.07179 |  | Inf | Inf |

Table 2-3: SNR corresponding to 70% maximum throughput for HetNet scenarios

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel moel | MMSE-IRC SNR (dB) | MMSE SNR (dB) | Gain MMSE-IRC vs MMSE (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 40M/30kHz | 2T6R | Rank1, MCS17 | TDLC 300-100 Low | 5.2 | 10.3 | 5.1 |
|  | 2T6R | Rank2, MCS17 | TDLA 30-10 Low | 8.0 | Inf | Inf |
|  |  |  | TDLA 30-10, α= 0 and β = 0.07179 | 16.7 | Inf | Inf |
|  | 4T6R | Rank 4, MCS 13 | TDLA 30-10 Low | 12.1 | Inf | Inf |
|  |  |  | TDLA 30-10, α= 0 and β = 0.07179 | 19.9 | Inf | Inf |

## Intra cell interference cancellation

We evaluated intra cell scenarios with different MCS, propagation conditions and different Rank, etc.

The SNR corresponding to 70% maximum throughput is shown in Table 2-4.

The throughput curves are shown in Table 2-12 and Table 2-13 for MU-MIMO scenarios.

Table 2-4: SNR corresponding to 70% maximum throughput for MU-MIMO scenarios

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | MMSE-IRC SNR (dB) | MMSE SNR (dB) | Gain MMSE-IRC vs MMSE (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T6R | Random | TDLA30-10 | ULA Low | MCS 13 | 16QAM | 2.9 | Inf | Inf |
|  |  |  |  |  | MCS 19 |  | 7.6 | Inf | Inf |
| 2+2 | 4T6R | Orthogonal |  |  | MCS 13 |  | 8.5 | Inf | Inf |
|  |  |  |  |  | MCS 17 |  | 12.1 | Inf | Inf |
| 1+1 | 2T6R | Random | TDLA30-10 | ULA, α= 0 and β = 0.07179 | MCS 13 | 16QAM | 9.7 | Inf | Inf |
|  |  |  |  |  | MCS 19 |  | 14.6 | Inf | Inf |
| 2+2 | 4T6R | Orthogonal |  |  | MCS 13 |  | 16.2 | Inf | Inf |
|  |  |  |  |  | MCS 17 |  | 20.2 | Inf | Inf |

# 3 Conclusion

In this contribution, we give some initial simulation results for 6Rx with interference on inter cell and intra cell scenarios.

# 4 Reference

- RP- 252957, WID: NR demodulation performance: Phase 6.

- R4-2514823, WF on NR_demod_Ph6_Ph5.