3GPP TSG-RAN WG4 Meeting #117	R4-2521196

Dallas, USA, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on System parameters for US 4.9 GHz band

Agenda item:	5.11.2

Document for:	Approval

Introduction

RAN at #108 meeting agreed the WID on Introduction of NR TDD 4.9GHz band for US operation [1]. In RAN4#116, the initial study phase concluded that 3GPP would rely on the band manager described by FCC for the coexistence issues with incumbents and thus no specific 3GPP work would be required for them [2]. Since RAN4#116bis meeting, RAN4 has started working on the core part of the WI in the following.

Following the completion of the study phase, the core objectives of this WI are:

-	Specify a new NR TDD operating band for the frequency range, 4940-4990 MHz to support the following SCS and channel bandwidths:

| SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

- Specify band numbering, system parameters and RF characteristics for the channel bandwidths defined for the new band.

- Address potential BS and UE co-existence issues, if any.

  - Specify band specific BS emissions requirements if needed

  - Specify UE additional emission limits and A-MPR if needed

- Specify the BS, repeater and IAB RF requirements.

- Specify the UE RF requirements for the new band. Reuse NR band n79 UE hardware assumption if possible.

- Update the related technical specifications to include support for the new band

- CRs shall not be approved until ongoing litigation is resolved to avoid specification rework and potential NBC changes

Note: The new NR band will be introduced in a REL-independent way starting from REL-15.

In this contribution, we discuss the system parameters aspects for the US 4.9GHz band. The discussion and agreement made in RAN4#116-bis are found in [4][5][6].

Discussion

Band number

4940-4990 MHz overlaps with the existing NR bands n79. To accommodate specific FCC rules [3], new band number assignment is suggested. So far band number up to 113 (for LTE based 5G broadcast) is used. New band number n114 can be reserved for US 4.9 GHz band.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n114 | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |

A new band number n114 is reserved for the US 4.9GHz band for 4940-4990 MHz.

Channel raster

As SCS based raster is introduced for all bands above 2.4 GHz, the same should apply to the new US 4.9 GHz band in the following.

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

SCS based raster with NR-ARFCN from 729334 to 732666 are applicable for the US 4.9GHz band for 4940-4990 MHz.

Sync raster

As the minimum channel bandwidth of this band is 10 MHz as defined in WID [1], the sync raster step size must be 1 to support all the channel raster entries. SS block SCS and pattern should follow the common design with the existing FR1 TDD bands above 2.6 GHz (band n77/n78/n79), i.e., 30 kHz SCS and Pattern C.

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 30 kHz | Case C | 8850 – <1> – 8877 |

SS block SCS and pattern are proposed to be 30 kHz and Case C. GSCN range is from to 8850 to 8877 with step size of 1.

Conclusion

The following Observations and Proposals were made:

Proposal 1: A new band number n114 is reserved for the US 4.9GHz band for 4940-4990 MHz.

Proposal 2: SCS based raster with NR-ARFCN from 729334 to 732666 are applicable for the US 4.9GHz band for 4940-4990 MHz.

Proposal 3: SS block SCS and pattern are proposed to be 30 kHz and Case C. GSCN range is from to 8850 to 8877 with step size of 1.

- References

- RP-252929 Revised WID on Introduction of NR TDD 4.9GHz band for US operation, Fujitsu, RAN#109

- RP-252913 Status report for Introduction of NR TDD 4.9GHz band for US operation, Fujitsu, RAN#109

- R4-2509418 On 4940-4990 MHz band regulations for adjacent spectrum protection, Nokia, RAN4#116

- R4-2514445	Topic summary for [116bis][322] NR_TDD_4.9GHz_band, RAN4#116-bis

- R4-2515083	Ad-hoc meeting minutes for [116bis][322] NR_TDD_4.9GHz_band, RAN4#116-bis

- R4-2515103 Way Forward for [116bis][322] NR_TDD_4.9GHz_band, RAN4#116-bis