3GPP TSG-RAN WG4 Meeting # 117 	    R4-2521202

Dallas, US, 17 – 21 November 2025

Agenda Item:	6.11.5

Source: 	Ericsson

Title:	On core requirement for AI/ML based positioning

Document for:	Approval

# Introduction

In the last meeting core requirement for AI/ML positioning case 1 was discussed and the following agreement was reached  [1].

Agreement from RAN4#116bis for requirements for case 1

To be clarified in the requirements:

- Request location information message should refer to “case 1” positioning

- Reuse the legacy RSTD measurement delay for all the cases (with gap, w/o gap, PRS aggregation)

- Beam sweeping factor for FR2 including sweeping reduction factor

  - Take the legacy values

- Rx TEG: check on signalling, take the legacy(used in RSTD measurements) approach if it is signalled

- Cover all RRC states

In this contribution aspects related to the core requirement for AI/ML positioning case 1 are further discussed.

# Discussion

## RRC states

According to the UE feature list agreed in RAN1#122bis, AI/ML based positioning case 1 is supported by UE in RRC_CONNECTED state only. The agreement from last meeting where it was agreed to define core requirement for all RRC states therefore needs to be revised and should only define requirements for RRC_CONNECTED mode. The other parts of the agreement where the requirements are to be defined with PRS measurement is performed by UE without gap, with gap, and by aggregating PRS resources from multiple PFLs still remains valid and no changes are required for those parts of the agreement.

Observation 1: AI/ML positioning case 1 is supported by UE in RRC_CONNECTED mode only.

Proposal 1: Define core requirement for AI/ML positioning case 1 only for RRC_CONNECTED mode. PRS measurement with measurement gap, without measurement gap, and aggregation of PRS resources from multiple PFLs in RRC_CONNECTED mode are within the scope.

## UE features

Based on the latest UE feature list agreed in RAN1#122bis [2], the following observations can be made:

Observation 2: There is no overlap in terms of UE PRS processing capability between non-AI/ML and AI/ML based methods.

Observation 3: Supported number of samples for PRS measurement is not reported by UE to LMF.

Observation 4: Supported number of Rx beam sweeping factor for PRS measurement is not reported by UE to LMF.

Observation 5: Supported number of Rx TEGs for PRS measurement is not reported by UE to LMF.

Since supported number of samples, number of Rx beam sweeping factor, and number of Rx TEGs for PRS measurement are not reported by UE to LMF as its capability, LMF cannot configure UE, like in legacy, to perform PRS measurement with reduced number of samples, reduce Rx beam sweeping factor, and whether to measure same DL PRS resources from multiple Rx TEGs or to measure same DL PRS resource simultaneously from multiple Rx TEGs for AI/ML based positioning case 1. As these parameters are transparent from the signalling point of view, it is imperative from RAN4 point of view to consider Nsample = 4, NRxBeam = 1 in FR1 and NRxBeam = 8 in FR2 and to not consider impact of number of Rx TEGs on the measurement period requirement for AI/ML positioning case 1.

Proposal 2: Nsample = 4 is considered in the measurement period requirement for AI/ML positioning case 1.

Proposal 3: NRxBeam = 1 in FR1 and NRxBeam = 8 in FR2 is considered in the measurement period requirement for AI/ML positioning case 1.

Proposal 4: Impact of number of Rx TEGs on the measurement period requirement is not considered for AI/ML positioning case 1.

## Inference Delay

In RAN4#116 meeting it was agreed to define the reporting delay requirement for AI/ML positioning case. The agreement is copied below for reference.

Agreement from RAN4#116 [3]:

Take the framework of the existing reporting delay requirements

- Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report

- Check until next meeting on the number of samples needed for the measurements

In the last meeting it was agreed to define reporting delay requirement for cases where the PRS measurement is performed by UE within measurement gap, outside of the measurement gap and within PRS processing window (PPW), and when UE performs measurement by aggregating PRS resources in multiple PFLs to produce input for its AI/ML model to infer its location for reporting. In all these scenarios measurement delay can be different but the inference delay to account for the time taken by the AI/ML model to produce output as UE location should however be same.

Proposal 5: RAN4 to define one inference delay value that is applicable to AI/ML positioning case 1 when UE performs PRS measurement within measurement gap, outside of the measurement gap and within PPW, and when UE performs measurement by aggregating PRS resources in multiple PFLs.

# Summary

In this contribution, issues related to the core requirement for UE-based positioning case 1 are discussed. The observations and proposals below summarize the discussions presented in this paper.

Observation 1: AI/ML positioning case 1 is supported by UE in RRC_CONNECTED mode only.

Proposal 1: Define core requirement for AI/ML positioning case 1 only for RRC_CONNECTED mode. PRS measurement with measurement gap, without measurement gap, and aggregation of PRS resources from multiple PFLs in RRC_CONNECTED mode are within the scope.

Observation 2: There is no overlap in terms of UE PRS processing capability between non-AI/ML and AI/ML based methods.

Observation 3: Supported number of samples for PRS measurement is not reported by UE to LMF.

Observation 4: Supported number of Rx beam sweeping factor for PRS measurement is not reported by UE to LMF.

Observation 5: Supported number of Rx TEGs for PRS measurement is not reported by UE to LMF.

Proposal 2: Nsample = 4 is considered in the measurement period requirement for AI/ML positioning case 1.

Proposal 3: NRxBeam = 1 in FR1 and NRxBeam = 8 in FR2 is considered in the measurement period requirement for AI/ML positioning case 1.

Proposal 4: Impact of number of Rx TEGs on the measurement period requirement is not considered for AI/ML positioning case 1.

Proposal 5: RAN4 to define one inference delay value that is applicable to AI/ML positioning case 1 when UE performs PRS measurement within measurement gap, outside of the measurement gap and within PPW, and when UE performs measurement by aggregating PRS resources in multiple PFLs.

# References

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm Incorporated.

- R1-2507979, Updated RAN1 UE features list for Rel-19 NR after RAN1 #122bis, Moderators (AT&T, NTT DOCOMO, INC.).

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated.