3GPP TSG-RAN WG4 Meeting #117	R4-2521212

Dallas, Tx, USA, 17th – 21st November 2025

Source: 	Ericsson

Title:  	RRM core requirements and test framework for AI in beam management

Agenda Item:	6.11.4

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #116, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Core requirements

Issue 2-1: Measurement period for inference – case 2

Agreement: Measurement period for inference – case 2 K(number of samples)*[M]*N*P K as defined by RAN1

There is still a parameter M in bracket. M is regulated by R1, the spec. sets M = 1 when a higher-layer timeRestrictionForChannelMeasurement is present or for aperiodic CSI-RS; otherwise M = 3 for periodic / semi-persistent CSI-RS.

In conventional way of RRM requirements, M doesn’t affect measurement accuracy requirements, even it results in difference measurement delays. Accordingly, in prediction, the measurement delay may vary depending on the configured M value, while the accuracy of the measurements remains unaffected.

Proposal 1: For BM case 2, follow the conventional approach, M, as configured, is included in measurement delay formula.

Issue 2-6: KPIs for prediction

Agreement: - Regarding the metric for beam ID only prediction, where top-K predicted beam(s) are reported, RAN4 only specifies the requirements for the following scenario(s): - K=1 - In this case, the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

Regarding the leftover parameter ‘x’ dB, which essentially represents a margin factor, it may be influenced by measurement error as well as other prediction errors occurring during both training and inference phases. Therefore, it is expected that ‘x’ should be chosen to match or closely approximate the measurement error at ground-truth condition. While TR 38.843 references 1 dB as a margin for simulations, this value remains subject to doubt since no measurement error is taken into account.

Furthermore, it is worth noting that x dB here doesn’t consider Rx gain uncertainty, it is assumed that test can guarantee no Rx gain variation between measurement and prediction operation.

Proposal 2: Regarding the parameter x in beam prediction accuracy metrics, measurement error and Rx gain uncertainty shall be mitigated wherever possible.

Proposal 3: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of residual measurement error and other errors, [1] dB may be taken as a baseline.

Issue: Known conditions for TCI state switch

In draft CR R4-2514630, the known conditions for TCI state for beam management prediction

### 8.10.2X Known conditions for TCI state for beam management prediction The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19, to the completion of active TCI state switch, where the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19 is the RS in target TCI state or QCLed to the target TCI state, or the one of RS resource in resourcesForChannelPrediction-r19 is the RS in the target TCI state or QCLed to the target TCI state but not in the RS resource in resourcesForChannelMeasurement and UE supports [known TCI based on prediction capability] - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19 - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Otherwise, the TCI state is unknown.

A question shall be clarified is how to interpret the condition ‘The SSB associated with the TCI state remain detectable’. The RS resource for prediction may or may not be the SSB associated with the TCI state.

- If the RS resource for prediction is the SSB associated with the TCI state, the (predicted) SSB associated with the TCI state remain detectable based on prediction or predictable.

- If the RS resource for prediction isn’t the SSB associated with the TCI state, the (measured) SSB associated with the TCI state remain detectable based on measurement.

Proposal 4: RAN4 to confirm that the statement ‘TCI state remains detectable’ does not mean that the RS in the target TCI must be transmitted.

Proposal 5: RAN4 to confirm that in the statement ‘The TCI state remains detectable during the TCI state switching period, ‘detectable’ may be ‘predictable’.

## Testability and interoperability

Issue 2-9: Simplified Channel models

Agreement: RAN4 will further discuss the simplified Uma and UMi channel model by taking the following aspects into consideration - Commercial test system limitation, including the number of probes and the separation - Model simplification should not result in inference performance degradation beyond the margin when the training is done based on the original models. - The margin can be discussed and decided further. - Other aspects are not precluded.

The simplification of the CDL channel is achieved by constraining the directions Tx beams and and AOA/AOD. Observations regarding possible approaches to simplifying AOA/AOD can be found in R4-2500825, R4-2411254, and other relevant contributions. The simplified CDL channel consolidates similar AOA/AOD angles to enable spatial evaluation with a reduced number of probes.

Given that the maximal number of beams to be measured is 32, a important consideration is whether the 32 beams after simplified CDL channel can be produced by 8 or 12 probes thereby achieving the balance between performance and implementation complexity.

Proposal 6: RAN4 to check if the 32 beams after simplified CDL channel can be produced by [8] (if it is feasible to be realized in test bed) probes which may achieve the balance between performance and implementation complexity.

Proposal 7: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

# Conclusion

Proposal 1: For BM case 2, follow the conventional approach, M, as configured, is included in measurement delay formula.

Proposal 2: Regarding the parameter x in beam prediction accuracy metrics, measurement error and Rx gain uncertainty shall be mitigated wherever possible.

Proposal 3: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of residual measurement error and other errors, [1] dB may be taken as a baseline.

Proposal 4: RAN4 to confirm that the statement ‘TCI state remains detectable’ does not mean that the RS in the target TCI must be transmitted.

Proposal 5: RAN4 to confirm that in the statement ‘The TCI state remains detectable during the TCI state switching period, ‘detectable’ may be ‘predictable’.

Proposal 6: RAN4 to check if the 32 beams after simplified CDL channel can be produced by [8] (if it is feasible to be realized in test bed) probes which may achieve the balance between performance and implementation complexity.

Proposal 7: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

# References

- R4-2511890 WF on AI/ML, Qualcomm