3GPP TSG-RAN WG4 Meeting #117	R4-2521213

Dallas, Tx, USA, 17th – 21st November 2025

Source: 	Ericsson

Title:  	RRM performance requirements for AI in beam management

Agenda Item:	6.11.6

Document for:	Discussion

# Introduction

In this contribution present our views and proposals on the remaining open issues relevant to performance requirements.

# Discussion

In the core requirements, two aspects are introduced for beam management prediction: (1) the known conditions for TCI state switch, and (2) the measurement delay for prediction. Accordingly, performance requirements should consider test coverage for these two aspects.

For TCI state switch, legacy test cases primarily focus on MAC-CE–based active TCI state switches under known conditions. The same approach can be applied to TCI state switches based on prediction.

Proposal 1: Test TCI state switch with predicted known conditions, taking the below into account:

- Cover MAC-CE based active TCI state switch

- Prediction configuration comprises BM-case 1 and BM-case 2 (SSB->SSB)

Proposal 2: Test TCI state switch with predicted known conditions shall at least include 2 time periods:

- The first period is for measurement for prediction, no transmission of the Set-A beam (the target TCI state) occurs in the time period.

- Upon end of the first period the UE shall receive TCI state switch command and perform TCI state switch. Set-A beam (the target TCI state) shall be transmitted to the UE.

For L1-RSRP measurements used in beam management prediction, the parameter K represents the difference in measurement delay relative to the legacy case. Specifically, K=1 for [UE sided beam prediction for BM Case 1 for inference]. And for [UE sided beam prediction for BM Case 2 for inference], K is indicated by [UE capability on number of latest consecutive transmission occasion].

In general, two types of test cases are required:

- Testing the measurement accuracy and reporting delay of L1-RSRP measurements for prediction.

- Testing prediction accuracy based on measurement, including:

  - L1-RSRP prediction

  - Beam ID prediction

Proposal 3: Test cases shall consider at the least below:

- Test measurement accuracy and reporting delay of L1-RSRP measurements for Prediction.

- Test prediction accuracy based on measurement, and prediction covers the below options with respect to UE capability:

  - L1-RSRP prediction

  - Beam ID prediction

# Conclusion

Proposal 1: Test TCI state switch with predicted known conditions, taking the below into account:

- Cover MAC-CE based active TCI state switch

- Prediction configuration comprises BM-case 1 and BM-case 2 (SSB->SSB)

Proposal 2: Test TCI state switch with predicted known conditions shall at least include 2 time periods:

- The first period is for measurement for prediction, no transmission of the Set-A beam (the target TCI state) occurs in the time period.

- Upon end of the first period the UE shall receive TCI state switch command and perform TCI state switch. Set-A beam (the target TCI state) shall be transmitted to the UE.

Proposal 3: Test cases shall consider at the least below:

- Test measurement accuracy and reporting delay of L1-RSRP measurements for Prediction.

- Test prediction accuracy based on measurement, and prediction covers the below options with respect to UE capability:

  - L1-RSRP prediction

  - Beam ID prediction

# References

- R4-2511890 WF on AI/ML, Qualcomm