3GPP TSG-RAN WG4 Meeting #117	R4-2521220

Dallas, Tx, USA, 17th – 21st November 2025

Source: 	Ericsson

Title:  	Discussion on RRM core requirements on FR2-1 L3 measurement delay

Agenda Item:	4.15.1

Document for:	Discussion

# Introduction

At the last 3GPP RAN WG4 Meeting #116bis, the outcomes of the consensuses and the unresolved issues were documented in WF [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Topic: L3 measurement delay reduction by optimizing Rx beam sweeping factor

Issue 1-1-1: Condition of re-entering FBS after exiting

<Way Forward>: - Further discuss in next meeting considering the below options: - Option1: Define T2 for re-evaluation after the UE exits FBS mode. - Option2: Not to define T2 for re-evaluation after the UE exits FBS mode. - Option 2.1: Need to update core requirements. - Option 2.2: Not need to update core requirements.

After discussions in the last meeting, our view is that it is appropriate to first analyze the possible cases where the UE may re-enter FBS. These cases are presented below:

Case 1: UE re-evaluates FBS after quitting FBS due to not meeting FBS condition.

Case 2: UE re-evaluates FBS after quitting FBS due to expiry of the 90-second timer, while the FBS condition remained met throughout the 90 seconds.

First of all, we understand that a rational UE behavior is keeping the L3 measurement procedure as per legacy operation and comparing the measurement results against the FBS condition provided the UE supports FBS.

Observation 1: The UE continuously performs L3 measurements according to legacy procedures and compares the results with the FBS condition provided that the UE capability supports FBS.

If the above observation holds, Case 1 is straightforward. It indicates that the UE shall evaluate the FBS condition and enter FBS when the FBS condition is met. This behavior is consistent with the existing specification content, which is reproduced below for reference.

L3 fast Rx beam sweeping operation is activated for intra-frequency measurements and inter-frequency measurements with measurement gap provided that - UE is configured with triggering conditions indicated by fbs-ThresholdP-r19 and/or fbs-ThresholdQ-r19, and the configured triggering conditions are satisfied. …

Observation 2: The case that the UE re-evaluates FBS after exiting FBS due to the FBS condition not being met is already covered by the existing specification.

On the other hand, Case 2 presents a certain degree of ambiguity, as there is no explicit specification statement defining whether the UE shall re-enter FBS if the FBS condition remains met after the 90-second timer expires.

In other words, when the FBS activation criteria continue to be satisfied, it is currently up to UE implementation to determine whether and when to re-enter FBS, since no explicit requirement is defined in the specification.

We consider this implementation-specific behavior reasonable to some extent, even though it is outside of direct network control. The rationale is as follows:

As discussed in very early phase of this WI, FBS is intended to be enabled when the UE transitions from a non-cell-edge position to a cell-edge position, allowing the UE to provide 90 seconds of fast measurement results to support urgent mobility preparation.

If, after the 90-second period, the UE remains at the cell edge without moving back toward the non-cell-edge position and also without being triggered for handover, this implies that the UE is stationary at the cell edge and that the network has already obtained the relevant measurement results. In such a case, maintaining FBS operation is not considered urgent.

Observation 3: It is up to the UE to determine whether to re-enter FBS after exiting due to the 90 s expiry and the FBS condition remains met.

Proposal 1:  Re-entering FBS after exiting does not require any update to the core requirements.

Issue 1-1-4: Applicability of FBS when the target and serving carrier frequency are on different

<Way Forward>: - Further discuss the options: - Option 1: When the target and serving carrier frequency are different, FBS applies to target carrier only. - Option 2: When the target and serving carrier frequency are different, FBS applies to the serving carrier and target carrier. - Option 3: Introduce UE capability indicating UE can support FBS on both target and serving carrier frequency.

For the case where the target and serving carrier frequencies are different, the design objective is to accelerate measurement on the target carrier. It is acceptable if the serving carrier measurement is performed faster as well, even though a faster measurement on the serving carrier does not directly facilitate handover.

Proposal 2: We’re open to speeding up the serving carrier by FBS in the case of inter-frequency measurement.

# Conclusion

Observation 1: The UE continuously performs L3 measurements according to legacy procedures and compares the results with the FBS condition provided that the UE capability supports FBS.

Observation 2: The case that the UE re-evaluates FBS after exiting FBS due to the FBS condition not being met is already covered by the existing specification.

Observation 3: It is up to the UE to determine whether to re-enter FBS after exiting due to the 90 s expiry and the FBS condition remains met.

Proposal 1:  Re-entering FBS after exiting does not require any update to the core requirements.

Proposal 2: We’re open to speeding up the serving carrier by FBS in the case of inter-frequency measurement.

# References

- R4-2514800, WF for NR_RRM_Ph5_part1, Ericsson