3GPP TSG-RAN WG4 Meeting #117	R4-2521232

Dallas, USA, 17 November – 21 November 2025

Agenda Item:	6.2.3

Source:	Ericsson

Title:	UE demodulation requirements for non-colocated CA Phase 2

Document for:	Discussion

# Introduction

RAN4#116bis agreed with the WF on the UE demodulation requirements for the intra-band non-collocated EN-DC/NR-CA deployment Phasae2 [1], where RAN4 agreed to define PDSCH demodulation requirements for Type 4b UE with 2CC NR-CA scenario.

This contribution provides our simulation results to decide the MCS.

# Simulation results

Table 1 and Table 2 show our simulation results to list the MCS/rank candidates for Type 4b UE demodulation requirements. It is observed that the candidate pair is Rank 2 MCS 4 and Rank 4 MCS 21, to satisfy the MRTD within 25dB.

Table 1	PDSCH simulation results (for alignment) for CC#1.

| Rank | Propagation condition | Antenna configuration | MCS (64QAM table) | SNR to achieve 70% of maximum throughput (ideal results) |
| --- | --- | --- | --- | --- |
| 2 | Static channel in TS 38.101-4 B.1.1 | 2x4 | 4 | -4.3 |

Table 2	PDSCH simulation results (for alignment) for CC#2.

| Rank | Propagation condition | Antenna configuration | MCS (256QAM table) | SNR to achieve 70% of maximum throughput (ideal results) |
| --- | --- | --- | --- | --- |
| 4 | Static channel in TS 38.101-4 B.1.1 | 4x4 | 21 | 20.2 |
|  |  |  | 22 | 21.2 |
|  |  |  | 23 | 22.7 |
|  |  |  | 24 | 23.6 |
|  |  |  | 25 | 25.3 |
|  |  |  | 26 | 25.9 |

Observation 1: Candidate pair of Type 4b UE demodulation requirements for NR-CA is Rank 2 MCS 4 and Rank 4 MCS 21.

# Specification structure

In Rel-18 NonCol_intraB_ENDC_NR_CA, RAN4 added PDSCH demodulation requirements as a part of 2Rx CA demodulation requirements. For Type 4b UE, since it is assumed that the UE receive antennas are 4 for each carrier, RAN4 should add new clause “Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA” in 5.4A.3.x.

Proposal 1: RAN4 should add new clause “Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA” in TS 38.101-4 5.4A.3.x.

# Summary

Observations:

Observation 1: Candidate pair of Type 4b UE demodulation requirements for NR-CA is Rank 2 MCS 4 and Rank 4 MCS 21.

Proposals:

Proposal 1: RAN4 should add new clause “Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA” in TS 38.101-4 5.4A.3.x.

# References

- R4-2514791, “WF on NonCol_intraB_ENDC_NR_CA_Ph2_demod”

# Appendix

## Type 4 UE types

| UE Type | CC# | antenna / LNA | Mixer | Analog BB | #Rx | NRCA / ENDC | power imbalance | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 (NR) | 4 shared | 4 shared | 4 shared | 4Rx | NR-CA EN-DC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 (LTE/NR) |  |  |  | 4Rx |  |  |  |
| 2 | 1 (NR) | 2 | 2 | 2 | 2Rx | NR-CA EN-DC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 (LTE/NR) | 2 | 2 | 2 | 2Rx |  |  |  |
| 4a | 1 (NR) | 4 | 6 total | 4 | 4Rx | EN-DC | 25dB full range | Requires 6 antennas and FWA |
|  | 2 (LTE/NR) | 2 |  | 2 | 2Rx |  |  |  |
| 4b | 1 (NR) | 4 | 8 total | 4 | 4Rx | NR-CA EN-DC | 25dB full range | Requires 8 antennas and LNA |
|  | 2 (LTE/NR) | 4 |  | 4 | 4Rx |  |  |  |