3GPP TSG-RAN WG4 Meeting #117	R4-2521234

Dallas, USA, 17 November – 21 November 2025

Agenda Item:	7.9.3

Source:	Ericsson

Title:	Discussion on the 2-sided CSI

Document for:	Discussion

# Introduction

RAN4#116bis agreed with the way forward on AI/ML for NR air interface Phase 2 [1], where RAN4 agreed with the following terminologies:

Fully specified the model is defined as a specified structure with parameter in [TS]. Reference model means UE/NW is not mandated to implement it. Test decoder means TE vendor needs to implement it for testing purpose. Direct C in RAN1: fully specified reference encoder and decoder. Option 3 in RAN4: fully specified test decoder and the reference encoder is under discussion (it is agreed to have a fully specified reference encoder) Direction A 3a-1 in RAN1: specify the reference encoder structure. Presumably, NW will send the parameters to the UE.

The WF also recommended to discuss the following issues in RAN4#117.

- Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples

- Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions

- Proposals on model hyperparameters, considering in particular complexity, considering also quantization size

- Proposals and reasoning for scalability solutions

- Proposals and reasoning for training parameters

- Proposals for performance metrics for evaluating the different potential model structures

- Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

We first note that the agreed terminology on ‘Direction A 3a-1 in RAN1’ is not aligned with RAN1 understanding. In Direction A, option 3a-1, the NW-side will send the parameters to the UE-side, and whether the UE is involved or not, and how, is not yet decided. We following excerpt from TR 38.843, our highlighting, illustrates the point that the UE is not the main target for the parameter exchange.

- The parameter (3a) / model (5a) exchange originates from the NW-side and ends at the UE-side - Parameters (3a) / model (5a) received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing - Parameters (3a) / model (5a) exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both. - Option 3a-1 / 5a-1: Parameters (3a) / model (5a) exchanged from the NW-side to UE-side is CSI generation part.

Proposal 1: Correct the definition of terminology of “Direction A 3a-1 in RAN1” as follows: specify the reference encoder structure. Presumably, NW-side will send the parameters to the UE-side.

# Inter-vendor training collaboration

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

## RAN check point

According to the WID objective [2], RAN plenary needs feedback from RAN4 on the fully defined/specified reference model with RAN1 scalability study outcome taken into account. Note the other aspects such as specifications of standardized encoder model structure and dataset format/content will be responded by SA WG.

RAN4#116bis discussed how to respond to the RAN check point [1].

How to respond to the RAN checkpoint - Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model. - RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too. - Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective. - FFS on detailed steps to develop the model - RAN4 will send LS on RAN4’s agreement regarding the RAN checkpoint.

Based on the agreements above, RAN4 should respond to RAN plenary (and RAN1 in CC) as follows:

RAN4 concluded Option 3 is feasible to complete based on the SI conclusion. RAN4 agreed that, from the RAN4 perspective, the test decoder of Option 3 is same as the reference decoder of Direction C, and the reference encoder of Option 3 is the same as the reference encoder of Direction C. With this sense, RAN4 concluded that the fully defined/specified reference model is feasible. RAN4 will develop the test decoder and reference encoder with the following steps: - Step 1: Create dataset - Step 2: Align on the model backbone, model hyperparameters and the scalability solution. - Step 3: Specify the model. Where and how to specify may need further discussion in RAN4. - Note RAN4 needs to decide the detailed procedure in each step. When developing the test decoder and reference encoder, RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model. When RAN4 develops the reference encoder, it could possibly be considered by RAN1 as the standardized encoder model structure in Direction A sub-option 3a-1.

See the full text of draft LS in Appendix: draft LS to RAN on fully defined/specified reference model for two-sided CSI compression.

Proposal 2: RAN4 should send LS to RAN (and to RAN1 in CC) on fully defined/specified reference model for two-sided CSI compression.

# Interoperability of two-sided AI/ML models

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

## Model backbone

The test decoder and the reference encoder in Option 3 will have the same backbone. - It is FFS if a single test decoder will be specified. - The test decoder and the reference encoder can have different structures, e.g. # of layers Model backbone Transformer is assumed as the baseline. Only Quantized parameters are specified.

RAN4 agreed to use Transformer as the baseline model of CSI compression, however RAN4 has not discussed the detailed architecture (e.g., number of layers, etc) yet. We understand RAN1 is evaluating several Transformer options, however, since the test decoder and reference encode specified in RAN4 could be implemented on the commercial product, RAN4 should carefully choose the model backbone. Especially, RAN4 should consider the realistic quantization method because it affects to the real-time performance such as processing time, power consumption, and memory consumption.

Proposal 3: When RAN4 chooses the two-sided model backbone, RAN4 should consider the realistic quantization method.

## Model scalability

Unless the significant issues are identified, RAN4 is going to only discuss the scalability issues and options, which have been agreed in RAN1.

According to RAN1 discussion, they have considered the number of Tx ports, CSI feedback payload size, and bandwidth, for the model scalability.

- Number of Tx ports: 16 and 32 in Rel-19 SI. 32, 64, and 128 in Rel-20 WI.

- CSI feedback payload size: At least large CSI payload Z ≥ 230 bits per layer.

- Bandwidth: All subbands and a sunset of all the subbands

In RAN4#116bis, several companies proposed the model scalability and the moderator summary recommended as follows [3], based on the RAN1 study.

|  | Scalability solution |
| --- | --- |
| Choice of token/feature dimension (if Transformer is agreed) | - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. |
| Scalability over the feature dimension | - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) |
| Scalability over the token dimension | - Alt2: Padding at the input |
| Scalability over payload configurations | - Alt2: truncation/masking of the output linear layer output |

However if RAN4 use the different performance metric from RAN1 study, the conclusion could be different. Moreover, RAN1 did not study larger CSI-RS ports such as 128 in Rel-19 SI. Since RAN4 agreed to consider Transformer as the baseline, RAN4 may use the solution above as the initial assumption. However, RAN4 should also study the scalability solution depending on the performance metric.

Proposal 4: When RAN4 develops the Transformer based test decoder and reference encoder, use subband number as a token dimension, Tx port number as a feature dimension. As the initial assumption, use padding to support the scalability regarding the subband size and number of Tx ports. Other solutions should not be excluded at the current stage.

Proposal 5: When RAN4 develops the Transformer based test decoder and reference encoder, as the initial assumption, use truncation/masking of the output linear layer output to support the CSI feedback payload size scalability. Other solutions should not be excluded at the current stage.

## Dataset for model evaluation

RAN4 will discuss how to introduce the new mixed dataset for model structure evaluation purpose at the beginning of this WI, by taking into consideration of the scalability and/or various configurations. - Details are FFS including the channel model(s).

### Mixed dataset generation

In Rel-19 SI, RAN4 studied the feasibility of the interoperability of two-sided model, where RAN4 considered one scenario: 32Tx, 1 layer, 13 subbands, 32 latent values quantized with 2 bits (64bits), Dense Urban Macro. Therefore, companies only needed to generate one dataset.

In Rel-20 WI, one of the objectives in RAN4 is to specify the test decoder and reference encoder, according to RAN1 Direction C fully defined/specified reference model. Since the Direction C encoder/decoder could be implemented on the commercial product, the dataset used to train the models should consider the field scenarios as much as possible. Moreover, the dataset should also consider the scenarios used for the PMI reporting requirements, because the test decoder will be deployed in TE to perform the PMI reporting test for CSI compression. Although the PMI reporting requirements are discussed in the performance part, RAN4 should discuss the scenario (e.g., channel model, antenna configuration, subband size) earlier to generate the dataset to develop the test decoder.

Regarding the channel models, Rel-19 SI only considered Urban Macro. We don’t think it is enough to work in the commercial network. RAN4 should include more channel models such as Urban Macro, Urban Micro and Sub Urban. RAN4 also need to include TDL and CDL channel models considering the PMI reporting performance requirements.

Proposal 6: Dataset used for RAN4 test decoder training should be mixture of channel models such as Urban Macro, Urban Micro, Sub-urban, CDL and TDL.

### Alignment of dataset

In RAN4#116bis, it was proposed to study the method to align the dataset. We agree it is important to ensure the datasets from the companies should be ‘aligned’; however we have not studied what metric(s) should be used to judge the alignment of dataset for two-sided CSI compression model training.

There are several metrics listed in the topic summary such as Power Spectrum Entropy (PSE), Frequency Coherence, and Spatial Coherence. One simple way is, as we have done in Rel-19 SI, to cross-check the dataset each other, that is, train/validate the encoder/decoder with Company A’s dataset and test the model with Company B’s dataset. It will consume more time as the number of datasets are increased, but we should also consider it.

Proposal 7: RAN4 study the metric to ensure the alignment of dataset used for two-sided CSI compression models. RAN4 should also consider a simple cross-check approach such as using Company A’s dataset for model training/validation and using Company B’s dataset for model test.

## Training for test decoder and reference encoder development

Performance metrics

In Rel-19, RAN4 used SGCS of target CSI between encoder input and decoder output. We think it is a good start to develop the test decoder and reference encoder in Rel-20 WI also. RAN4 may also need to discuss the test metric of PMI reporting performance requirements with two-sided CSI compression model, for example, PDSCH throughput with follow PMI generated from construct target CSI.

Proposal 8: Use SGCS of target CSI between encoder input and decoder output as a starting point for test decoder and reference encoder development. RAN4 should also discuss the test metric used for the PMI reporting performance requirements with two-sided CSI compression before the performance part starts.

## How to capture the model and the dataset in 3GPP

For interoperability of two-sided models, RAN4 needs to capture the reference encoder and test decoder so that TE vendors can deploy the test decoder, possibly their own developed test decoder, without exchanging information between TE vendors and UE vendors. RAN4 also needs to capture the dataset where RAN4 trained the test decoder.

One way to capture the model/dataset is to store the model/dataset themselves (binary data) in 3GPP database (e.g., FTP server, 3GPP Forge). In Rel-19 SI, for example, RAN4 has used ONNX (Open Neural Network Exchange) [5] to exchange reference models.

We understand the ONNX is an open format to present machine learning models used in AI/ML area in 2025. However, since ONNX is developed by an open-source community, it is not ensured the community exists throughout the entire lifecycle of 5G.

Even if the community exists, 3GPP needs to be very careful to point towards the correct version and the version needs to not become obsolete the captured/standardized reference models. In case the ONNX community does not recommend using the current data format, 3GPP needs to update all the encoder/decoder models using new format, but it is not clear who is responsible for maintaining the models after the WI is complete.

Another way is to capture the model structure and dataset in the TR or TS. It could be possible to specify the model structure in the document (e.g., JSON-based serialized text data or data flow graph). On the other hand, it is not realistic to specify the model parameters (i.e., weights) and dataset in TS or in TR (MS Word document at least in NR specification) because it could be huge data. The parameters and dataset would then need to be stored in a separate file.

Table 1, Table 2, and Table 3 summarize our current views how to capture the models, model parameters, and datasets, respectively, in 3GPP.

Note the ‘3GPP server’ refers to 3GPP FTP server (www.3gpp.org/ftp/) for example.

Table 1: Options on how to capture the reference model structures

|  | How to specify | Where to store |
| --- | --- | --- |
| Option 1 | Open-source framework like ONNX | 3GPP server |
| Option 2 | Text data (e.g., JSON or data-flow graph) | TS or TR |

Table 2: Options on how to capture the reference model parameters

|  | How to specify | Where to store |
| --- | --- | --- |
| Option 1 | Open-source framework like ONNX (Together with model structure) | 3GPP server |
| Option 2 | Parameter data file (e.g., binary data or text data) | Parameter data format is specified in TS or TR Parameter data files are stored in 3GPP server |

Table 3: Options on how to capture the dataset

|  | How to specify | Where to store |
| --- | --- | --- |
| Option 1 | Open-source framework like NumPy | 3GPP server |
| Option 2 | Dataset data file (e.g., binary data or test data) | Dataset data format/content are specified in TS or TR Dateset data files are stored in 3GPP server |

Proposal 9: RAN4 should discuss how to capture the reference encoder and test decoder structures (i.e., backbone, numbers of layers, type of layers and all description of the model).

Proposal 10: RAN4 should discuss how to capture the reference encoder and test decoder parameters (i.e., model weights).

Proposal 11: RAN4 should discuss the dataset format and content for testability and interoperability purposes.

# Summary

Proposals:

Proposal 1: Correct the definition of terminology of “Direction A 3a-1 in RAN1” as follows: specify the reference encoder structure. Presumably, NW-side will send the parameters to the UE-side.

Proposal 2: RAN4 should send LS to RAN (and to RAN1 in CC) on fully defined/specified reference model for two-sided CSI compression.

Proposal 3: When RAN4 chooses the two-sided model backbone, RAN4 should consider the realistic quantization method.

Proposal 4: When RAN4 develops the Transformer based test decoder and reference encoder, use subband number as a token dimension, Tx port number as a feature dimension. As the initial assumption, use padding to support the scalability regarding the subband size and number of Tx ports. Other solutions should not be excluded at the current stage.

Proposal 5: When RAN4 develops the Transformer based test decoder and reference encoder, as the initial assumption, use truncation/masking of the output linear layer output to support the CSI feedback payload size scalability. Other solutions should not be excluded at the current stage.

Proposal 6: Dataset used for RAN4 test decoder training should be mixture of channel models such as Urban Macro, Urban Micro, Sub-urban, CDL and TDL.

Proposal 7: RAN4 study the metric to ensure the alignment of dataset used for two-sided CSI compression models. RAN4 should also consider a simple cross-check approach such as using Company A’s dataset for model training/validation and using Company B’s dataset for model test.

Proposal 8: Use SGCS of target CSI between encoder input and decoder output as a starting point for test decoder and reference encoder development. RAN4 should also discuss the test metric used for the PMI reporting performance requirements with two-sided CSI compression before the performance part starts.

Proposal 9: RAN4 should discuss how to capture the reference encoder and test decoder structures (i.e., backbone, numbers of layers, type of layers and all description of the model).

Proposal 10: RAN4 should discuss how to capture the reference encoder and test decoder parameters (i.e., model weights).

Proposal 11: RAN4 should discuss the dataset format and content for testability and interoperability purposes.

# References

- R4-2514614, “WF on R20 NR_AIML_air_Ph2”, Ericsson.

- RP-252445, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm.

- R4-2514530, “Topic summary for [116bis][123] NR_AIML_air_Ph2”, Moderator (Ericsson).

- R4-2411786, “Summary of e-mail discussion on AI/ML Model for CSI testing Option 3 feasibility study”, Moderator (Qualcomm).

- Open Neural Network Exchange, https://onnx.ai/

# Appendix: draft LS to RAN on fully defined/specified reference model for two-sided CSI compression

----------------------------------------------------- Beginning of draft LS ------------------------------------------------------------

Title:	LS on fully defined/specified reference model for two-sided CSI compression

Response to:	-

Release:	Rel-20

Work Item:	NR_AIML_air_Ph2

Source:	RAN WG4

To:	RAN

Cc:	RAN WG1

Contact person:	<name_of_tdoc_requestor>

<email_of_tdoc_requestor>

<phone_of_tdoc_requestor>

Attachments:	None

# 1	Overall description

RAN4 discussed the feasibility of the fully defined/specified reference model (“Direction C”) for two-sided CSI compression for the check-point in RAN#110, as mentioned in WID RP-252445.

In Rel-19 SI FS_NR_AIML_air_Ph2, RAN4 studied the interoperability of two-sided model, and concluded that “Option 3” as fully specified test decoder and reference encoder appears feasible.

In Rel-20 WI NR_AIML_air_Ph2, RAN4 concluded Option 3 is feasible to complete based on the SI conclusion. RAN4 agreed that, from the RAN4 perspective, the test decoder of Option 3 is same as the reference decoder of Direction C, and the reference encoder of Option 3 is the same as the reference encoder of Direction C. With this sense, RAN4 concluded that the fully defined/specified reference model is feasible.

RAN4 will develop the test decoder and reference encoder with the following steps:

- Step 1: Create dataset

- Step 2: Align on the model backbone, model hyperparameters and the scalability solution.

- Step 3: Specify the model. Where and how to specify may need further discussion in RAN4.

- Note RAN4 needs to decide the detailed procedure in each step.

When developing the test decoder and reference encoder, RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model.

When RAN4 develops the reference encoder, it could possibly be considered by RAN1 as the standardized encoder model structure in Direction A sub-option 3a-1.

# 2	Actions

To RAN

ACTION: RAN4 respectfully asks RAN to take into account the conclusion from RAN4 and to revise the WID of NR_AIML_air_Ph2.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN WG4 meeting #118		February 9, 2026 – February 13, 2026	Göteborg, SE

TSG-RAN WG4 meeting #118bis		April 13, 2026 – April 17, 2026	Malta, MT

----------------------------------------------------- End of draft LS ------------------------------------------------------------