# 3GPP TSG-RAN WG4 Meeting #117	R4-2521239

# Dallas, USA, Nov. 17-21, 2025

Source: 	OPPO

Agenda Item:	6.11.8

Title: 	Demodulation and CSI reporting requirements for CSI prediction

Document for:	Approval

Discussion

Updated simulation results for AI/ML based CSI prediction

Regarding the simulation assumptions for AIML-based CSI prediction, following aspects are updated in RAN4#116bis meeting,

- Remove the 50Hz doppler and remove the mixed Doppler assumption

- keep MCS 17 and MCS 19, remove MCS 13

- Use 2RX for FDD

- Use 4RX for TDD (update is allowed if any new agreement)

- Remove the assumptions on 32Tx ports

Accordingly, we conducted and updated Step-3 evaluations for the CSI prediction using the simulation assumptions(highlighted in yellow in Table 1). The simulation results are shown in Table 2.

Table 1: Updated Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Subcarrier spacing | 15kHz for FDD, 30kHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol for step-1 and step-2 simulation |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | 16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 2RX for FDD 4RX for TDD |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step-1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1 |
| Test setup(timeline) | Only for step-3 For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration |
| Beam steer approach | For step-3, randomization of the dual-cluster beam directions should be used as specified in AnnexB.2.3.2.3A in 38.101-4. The value of relative power ratio (p) shall be fixed as 1 during the test |
| SNR assumption | Only for step-3 Claimed by companies for step-3 first round simulation (TP as KPI) |
| CSI feedback overhead | PC7 as baseline |
| Baseline | Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements) |
| KPI | Step-1: - check SGCS1 before CSI feedback - SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-2: - check SGCS2 and SGCS3 - SGCS2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2) Step-3: - check SGCS1 and SGCS3, under practical channel estimation, corresponding to 90% of maximum TP with follow PMI using AI/ML-based CSI prediction - SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2) - check TPs after CSI feedback for both AI based CSI prediction and Random PMI with Rel-15 Type I single panel codebook, as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction γ1_AI/ML: AI/ML based CSI prediction is compared with Rel-15 Type I random PMI - check TPs after CSI feedback for both AI based CSI prediction and Rel-16 eType II codebook (sample and hold), as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction γ2_AI/ML: AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample and hold) - check TPs after CSI feedback Rel-16 eType II codebook (sample and hold) and Random PMI with Rel-15 Type I single panel codebook, as well as the SNR value corresponding to 90% of maximum TP of the Rel-16 eTypeII codebook. γ3_(sample & hold): Rel-16 eType II CB (sample and hold) is compared with Rel-15 Type I random PMI. - Companies are also encouraged to provide the TP curve with corresponding SNRs. - The test parameters for TPs simulation can be referred as 38.101-4 section of 6.3 - FDD with 16Tx2Rx: Table 6.3.2.1.8-1 - TDD with 16Tx4Rx: Table 6.3.3.2.8-1 |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. |
| Companies can provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models Companies are encouraged to clarify whether the SNR dataset for model training is per SNR or with an SNR range when providing the results of step-3 simulation |  |
| Clarifications on the SGCS calculation: SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel SGCS2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. |  |

Table 2. Simulation results(step-3) for AIML based CSI prediction, MCS=17 and 19

|  | gNB TX antennas | MCS | SNRue_AI/ML [dB] | SNR_Sample&Hold [dB] | γ1_(AI/ML) (compare with Rel-15 Type I random PMI) | γ2_(AI/ML) (compare with Rel-18 eType II Doppler CB) | γ3_(sample & hold) (compare with Rel-15 Type I random PMI) | AI/ML Channel prediction SGCS1 | AI/ML Channel prediction SGCS3 | Channel sample and hold SGCS1 | Channel sample and hold SGCS3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OPPO | 16 | 17 | 6.3 | 11 | [88.2] It is difficult for Rel-15 Type I random PMI to operate properly at SNRue_AI/ML | 1.7 | 5.5 | 0.935 | 0.84 | 0.75 | 0.635 |
|  |  | 19 | 7.7 | 12.9 | [88.8] It is difficult for Rel-15 Type I random PMI to operate properly at SNRue_AI/ML | 1.8 | 3.4 | 0.945 | 0.855 | 0.755 | 0.64 |
| MCS = 17 MCS=19 |  |  |  |  |  |  |  |  |  |  |  |

Regarding the simulation for AI/ML-based CSI prediction, the updated results are shown in Table 2 for MCS=17 and MCS=19.

Follow-up simulation and how to define requirement

By RAN4 #116bis meeting, RAN4 has 6 inputs for step-2 and 5 inputs for step-3, all for FDD. For the new updated step-3 results in RAN4#116bis meeting, which is more important to define the CSI prediction performance requirement, seems the variance among different companies is still quite heavy, potentially due to the generated wireless channel by different companies (e.g., different simulation platform), the utilized model(e.g., different backbone, detailed design), the channel estimation approach.

- Regarding the 20Hz assumption on Doppler spread, for the utilized models, we have compared different model architectures ranging from simple MLP to complex Mixer-Transformer structure. All achieved relatively good channel prediction results under the current simulation assumptions.

- Regarding channel estimation part, following assumptions are added and utilized:

Step-3: - check SGCS1 and SGCS3, under practical channel estimation, corresponding to 90% of maximum TP with follow PMI using AI/ML-based CSI prediction - SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2)

From the simulation results, SGCS3 under practical channel estimation shows some performance loss compared to SGCS3 under ideal channel estimation. However, from our understanding, the drop is consistent with what is expected from a practical estimation approach.

- Regarding the generated wireless channel, it may be a cause for the significant discrepancies observed in the current simulation results. Based on our simulations, under the 20Hz Doppler spread, the CSI prediction model predicts the next instant's channel quality quite good even at lower SNR levels. This leads to a situation that it is difficult for Rel-15 Type I random PMI to operate properly at SNRue_AI/ML as shown in Table 2. Aside from the performance advantage of the model design itself (as we compared various models from simple to complex), the underlying reason may be the inherent predictability of the generated wireless channel itself.

The inherent predictability of the generated wireless channel itself is a likely cause for the significant discrepancies observed in the current simulation results.

For step-3, under the condition of TX=16 and rank=2, the throughput performance obtained by randomly selecting a Type I codebook is relatively poor at SNRue_AI/ML

Considering above analysis and the simulation results already provided in RAN4#116bis meeting by different companies, for follow-up simulations to define CSI prediction requirements, two approaches could be considered:

  - Approach 1: Based on current simulation results, excluding results that show significant discrepancies with others, attempt to define a minimum performance requirement for CSI prediction that is acceptable to companies.

  - Approach 2: Attempt to construct a reference dataset for CSI prediction simulation. Based on this reference dataset, generate new simulation results, and then define the minimum performance requirement for CSI prediction acceptable to companies based on the new results.

Regarding the reference dataset, it may include:

  - The wireless channel H to be utilized for downlink transmission at slot n.

  - The predicted PMI obtained by NW (after performing practical channel estimation, channel prediction and CSI reporting), to be utilized for NW precoding at slot n.

  - A randomly selected Type I codebook set for comparison purpose.

By using the reference datasets provided by companies, RAN4 could form a mixed reference dataset for CSI prediction. Subsequently, based on this mixed reference dataset, companies could evaluate the throughput performance for assessing CSI prediction.

To define CSI prediction requirements, two approaches could be considered:

- Approach 1: Based on current simulation results, excluding results that show significant discrepancies with others, define a minimum performance requirement.

- Approach 2: Construct a reference dataset for CSI prediction simulation, generate new simulation results, and then define the minimum performance requirement for CSI prediction.

Regarding the reference dataset for CSI prediction, it may include:

- The wireless channel H to be utilized for downlink transmission at slot n.

- The predicted PMI obtained by NW (after performing practical channel estimation, channel prediction and CSI reporting), to be utilized for NW precoding at slot n.

- A randomly selected Type I codebook set for comparison purpose.

By using the reference datasets provided by companies, RAN4 could form a mixed reference dataset for assessing the CSI prediction requirements.

Conclusion

In this contribution, we have discussed the demodulation and/or CSI reporting requirements for CSI prediction, and got the following proposals:

- Observation 1: 	Regarding the simulation for AI/ML-based CSI prediction, the updated results are shown in Table 2 for MCS=17 and MCS=19.

- Observation 2: 	The inherent predictability of the generated wireless channel itself is a likely cause for the significant discrepancies observed in the current simulation results.

- Observation 3: 	For step-3, under the condition of TX=16 and rank=2, the throughput performance obtained by randomly selecting a Type I codebook is relatively poor at SNRue_AI/ML

- Proposal 1: 	To define CSI prediction requirements, two approaches could be considered:

- Approach 1: Based on current simulation results, excluding results that show significant discrepancies with others, define a minimum performance requirement.

- Approach 2: Construct a reference dataset for CSI prediction simulation, generate new simulation results, and then define the minimum performance requirement for CSI prediction.

- Proposal 2: 	Regarding the reference dataset for CSI prediction, it may include:

- The wireless channel H to be utilized for downlink transmission at slot n.

- The predicted PMI obtained by NW (after performing practical channel estimation, channel prediction and CSI reporting), to be utilized for NW precoding at slot n.

- A randomly selected Type I codebook set for comparison purpose.

- Proposal 3: 	By using the reference datasets provided by companies, RAN4 could form a mixed reference dataset for assessing the CSI prediction requirements.

Reference

R4-2511796 WF on updated simulation assumptions for AI based CSI prediction

RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2