# 3GPP TSG-RAN WG4 Meeting #117		  R4-2521253

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.7.4

Source:		Qualcomm Incorporated

Title:			Discussion on UAV UE RF requirements

Document for:	Approval

# 1  Introduction

New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) has been approved in RAN#109 [1] with the objective of analyzing the coexistence for UAV with higher transmit power.

- Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

In RAN4#116bis, the UAV UE RF requirements has been initially discussed, and the agreements are captured in WF [2]. In this paper, we provide our views on UAV UE RF requirements.

# 2  Discussion

For UAV UE RF requirements, it is important to first resolve how to define maximum output power for UAV higher power UE, as the framework of defining maximum output power will influence the development of other RF requiremetns.

In [2], the following agreements were reached regarding the maximum output power:

Issue 3-2-1: Maximum output power - Agreements: - RAN4 to specify maximum output power for UAV higher power UE considering the following two options: - Option 1: Introduce the new capability for high power UAV to indicate the rated maximum output power (Similar as ATG) - Option 2: Introduce a new power class for UAV UE for up to 33dBm max total conductive output power for the example bands (Similar as legacy TN UE power class definition) - Option 3: Other options are not precluded.

In Table 1, we provide the comparison of Option 1 and Option 2.

Table 1: Comparison between Option 1 and Option 2

|  | TN UE | ATG |
| --- | --- | --- |
| UE power class | PC3/2/1.5/1 | No power class definition. Rated MOP indicated/declared by UE [23dBm, 40dBm] |
| UE antenna type | Agnostic | Two antenna types are defined in 3GPP - Omni-directional - Phase array Two separate requirements are defined two types including min output power, maximum input level, etc. |
| Frequency error | No pre-compensation | SIB22, agt-gNB-Location-r18 |
| ACLR | 30dB for PC3, 31dB for PC2/1.5, [37dB] for PC1 | 30dB |
| ACS | 33dB | 33dB |
| MPR/A-MPR | Specify MPR/A-MPR | No MPR/A-MPR |
| Note: there is ongoing work on whether it is feasible to relax ACLR requirements for PC1 CPE |  |  |

First, the target maximum output power for Rel-20 UAV UE is up to 32 dBm, which is very close to the current maximum output power for PC1. Meanwhile, UAV UE does not perform frequency and/or timing pre-compensation; therefore, there is no need to define a new SIB as in ATG. Consequently, a higher-power UAV UE in Rel-20 appears quite similar to TN HPUE.

With that, we propose the following:

Proposal 1: RAN4 to consider introducing power class for UAV UE such as PC2/PC1.5/PC1 with up to XdBm maximum total conducted output power for the example bands (FFS on X).

Proposal 2: RAN4 to further discuss whether UE RF requirements can be defined in an antenna-type-agnostic manner.

# 3. Conclusion

In summary, we have following proposals:

Proposal 1: RAN4 to consider introducing power class for UAV UE such as PC2/PC1.5/PC1 with up to XdBm maximum total conducted output power for the example bands (FFS on X).

Proposal 2: RAN4 to further discuss whether UE RF requirements can be defined in an antenna-type-agnostic manner.

# 4  References

- RP-252908, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), RAN #109, September 2025

- R4-2515159, Way Forward for [116bis][319] NR_UAV, RAN4 #116bis, October 2025