3GPP TSG-RAN WG4 Meeting # 117						R4-2521255

Dallas, 17th – 21st November 2025

Agenda item:			7.11.3

Source:	Qualcomm Incorporated

Title:	Discussion on Ambient IoT Coexistence Assumptions in Outdoor

Document for:	Approval

# 1  Introduction

In RAN4 meeting #116bis, the ambient IoT coexistence scenarios and assumptions were initially discussed and the agreement for calibration was summarized in [1]. Based on the assumptions, we conducted the system level simulation, and identified some assumptions requires further discussion.

This paper provides our considerations on the coexistence assumptions for outdoor ambient IoT deployments.

# 2  Discussion for co-existence assumptions

## 2.1 Ambient IoT system performance

Based on the system assumptions agreed in [1], the ambient IoT system performance without adjacent channel interference was evalauted. The results, shown in Figures 1 and 2, include device 2b and device c under ISDs of 750m and 500m.

Figure 1. R2D SINR performance in standalone mode with reader Tx power 46dBm

Figure 2. D2R SINR performance

The performance curves for all four cases are nearly identical, indicating that device type and ISD variations have minimal impact due to the system being interference limited. Increasing transmit power also raises interference levels. For robust system performance, both R2D and D2R links require sufficient SINR. Although the working SINR threshold is still under discussion in RAN1, conservative reference values can be applied: a minimum of 6 dB for R2D links and 12 dB for D2R links. Under current deployment conditions, this results in a high outage rate—exceeding 50%.

Observation 1: R2D and D2R SINR performance show no significant difference between ISDs of 500m and 750m for device 2b and device c. More than 50% of devices experience outages under current deployment conditions.

Proposal 1: Interference mitigation methods, such as TDM and FDM, should be considered for ambient IoT systems in outdoor scenarios to reduce intra-system interference. If RAN4 agrees to adopt interference mitigation methods, an LS should be sent to RAN1 to assess any potential impact on RAN1

## 2.2 RF modeling for ambient IoT standalone mode

The ACLR and ACS values for device 2b and device c are proposed in [1] for calibration purposes. However, the modelling of ACLR and ACS has not been clearly addressed, particularly for multiuser scenarios and cases where the aggressor and victim bandwidths are asymmetric.

The relationships between aggressor and victim bandwidths for uplink (UL) and downlink (DL) cases are illustrated in Figure 3 and Figure 4.

Figure 3. the victim and aggressor bandwidth diagram for UL

Figure 4. the victim and aggressor bandwidth diagram for DL

For ACLR and ACS modeling of standard UE and BS, the typical assumptions can be reused for coexistence evaluations. ACLR is modeled using a three-step approach: [x, x+13, x+20], while ACS is modeled as a flat value. When NR acts as the aggressor with a larger bandwidth than ambient IoT, the ACLR can be adjusted using the scaling factor:

FACLR=10×log10​(BAggressor/Bvictim​​)

where BAggressor and BVictim are the bandwidths of the aggressor and victim, respectively. This scaling method is referenced in TR36.942 Chapter 5.1.1.4.1.

For ACLR and ACS modeling of device 2b and device c, only the in-band mode is considered in Release 19; thus, standalone mode is not discussed in detail. The D2R link bandwidth is only 15 kHz, which is very narrow. Two options are proposed for further discussion in RAN4:

- Option 1: Reuse the NB IoT and IoT NTN assumptions in previous RAN4 coexistence studies. When the aggressor has a narrower bandwidth, a symmetrical ACLR value is sufficient for coexistence. In this case, ACLR is flat/identical for an adjacent channel of 10 MHz (DL and UL, 3 UEs), and FACLR=0. Under these assumptions, the ACLR and ACS values are calculated and presented in Table 1.

Table 1. ACLR and ACS values for ambient IoT standalone mode

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| D2R | NR UL | For device 2b, ACLR = 20dB for UE 1, UE2, and UE 3 For device c, ACLR = 25dB for UE1, UE2, and UE 3 | ACS = 33 dB for UE 1, UE 2, and UE3 |
| NR UL | D2R | Bandwidth scale factor = 10*log10(3.3MHz/15KHz) = 23dB ACLR for UE 1 = 30+ 23 = 53dB ACLR for UE2 = 30+13+ 23 = 66dB ACLR for UE 3 = 30+20+23 = 73dB | ACS = 46dB for UE1, UE2 and UE3 |
| R2D | NR DL | ACLR = 45dB | ACS = 46dB |
| NR DL | R2D | Bandwidth scale factor = 10*log10(10MHz/180KHz) = 17dB ACLR = 45+ 17 = 63dB | for device 2b, ACS = 30dB for device c, ACS = 35dB |

- Option 2: As RF design for device 2b and device c is still in the early stages, filter design and performance remain unclear. Leakage to UEs at the far end of spectrum may be trivial. For the coexistence study in Rel-20, only one UE is proposed for NR UL, with a UE bandwidth of 3.3 MHz considered for coexistence analysis.

Figure 5: the victim and aggressor bandwidth diagram for UL (alternative option)

Proposal 2: For the ACLR and ACS modeling for standard TN UE and BS in ambient IoT standalone mode, the typical assumptions can be reused for coexistence evaluation.  ACLR should be modelled using a three-step approach: [x, x+13, x+20], while ACS can be modelled as a flat value. Bandwidth scaling factor is considered.

Proposal 3:  For ACLR modelling of devices 2b and c, two options can be considered for coexistence in ambient IoT standalone mode. RAN4 should further evaluate which approach is preferable.

- Option 1: Reuse the NB IoT and IoT NTN assumptions in previous RAN4 coexistence studies. ACLR is flat/identical for an adjacent channel of 10 MHz (1UE in DL and 3 UEs in UL).  The calculation is presented in Table 1.

- Option 2: As RF design for device 2b and device c is still in the early stages, filter design and performance remain unclear. Leakage to UEs at the far end of spectrum may be minimal. For this study, only one UE is proposed for NR UL, with a UE bandwidth of 3.3 MHz considered for coexistence analysis.

## 2.3 RF modeling for ambient IoT in-band mode

For the ALCR and ACS modelling in ambient IoT in-band mode, the coexistence study in Rel-19 can be used as reference.

Proposal 4: For ambient IoT device and reader, applying the same ACLR and ACS modelling for standalone mode and in-band mode.

Proposal 5: For standard TN UE and BS, the IBE modelling can be used, and the ACS can be assumed very large. The ACLR and ACS modelling for different coexistence cases are summarized in Table 2.

Table 2. ACLR and ACS modeling for ambient IoT in-band mode

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| D2R | NR UL | For device 2b, ACLR = 20dB for UE 1, UE2, and UE 3 For device c, ACLR = 25dB for UE1, UE2, and UE 3 | Assuming ACS is very large, the A-IoT RB can be removed. |
| NR UL | D2R | Legacy UE IBE. Sector 6.4.2.3 of TS 38.101. Legacy UE IBE requirement is related to EVM requirement, to simplify the calibration, it's suggested to directly use QPSK, i.e. 17.5% EVM for IBE requirement. | ACS = 46dB for UE1, UE2 and UE3 |
| R2D | NR DL | ACLR = 45dB | Assuming ACS is very large, the A-IoT RB can be removed. |
| NR DL | R2D | Considering BS IBE Option 1: 30dBc Option 2: 17dBc Companies to check the leakage of legacy NR BS. | for device 2b, ACS = 30dB for device c, ACS = 35dB |

# 3  Conclusion

In summary, we have following observations and proposals.

Observation 1: R2D and D2R SINR performance show no significant difference between ISDs of 500m and 750m for device 2b and device c. More than 50% of devices experience outages under current deployment conditions.

Proposal 1: Interference mitigation methods, such as TDM and FDM, should be considered for ambient IoT systems in outdoor scenarios to reduce intra-system interference.

Proposal 2: For the ACLR and ACS modeling for standard UE and BS, the typical assumptions can be reused for coexistence evaluation.  ACLR should be modelled using a three-step approach: [x, x+13, x+20], while ACS can be modelled as a flat value. Bandwidth scaling factor is considered.

Proposal 3:  For ACLR modelling of devices 2b and c, two options can be considered for coexistence. RAN4 should further evaluate which approach is preferable.

- Option 1: Reuse the NB IoT and IoT NTN assumptions in previous RAN4 coexistence studies. ACLR is flat for an adjacent channel of 10 MHz (DL and UL 3 UEs).  The calculation is presented in Table 1.

- Option 2: As RF design for device 2b and device c is still in the early stages, filter design and performance remain unclear. Leakage to UEs at the far end of spectrum may be minimal. For this study, only one UE is proposed for NR UL, with a UE bandwidth of 3.3 MHz considered for coexistence analysis.

Table 1. ACLR and ACS values for coexistence

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| D2R | NR UL | For device 2b, ACLR = 20dB for UE 1, UE2, and UE 3 For device c, ACLR = 25dB for UE1, UE2, and UE 3 | ACS = 33 dB for UE 1, UE 2, and UE3 |
| NR UL | D2R | Bandwidth scale factor = 10*log10(3.3MHz/15KHz) = 23dB ACLR for UE 1 = 30+ 23 = 53dB ACLR for UE2 = 30+13+ 23 = 66dB ACLR for UE 3 = 30+20+23 = 73dB | ACS = 46dB for UE 1, UE 2, and UE3 |
| R2D | NR DL | ACLR = 45dB | ACS = 46dB |
| NR DL | R2D | Bandwidth scale factor = 10*log10(10MHz/180KHz) = 17dB ACLR = 45+ 17 = 63dB | for device 2b, ACS = 30dB for device c, ACS = 35dB |

Proposal 4: For ambient IoT device and reader, applying the same ACLR and ACS modelling for standalone mode and in-band mode.

Proposal 5: For standard TN UE and BS, the IBE modelling can be used, and the ACS can be assumed very large. The ACLR and ACS modelling for different coexistence cases are summarized in Table 2.

Table 2. ACLR and ACS modeling for ambient IoT in-band mode

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| D2R | NR UL | For device 2b, ACLR = 20dB for UE 1, UE2, and UE 3 For device c, ACLR = 25dB for UE1, UE2, and UE 3 | Assuming ACS is very large, the A-IoT RB can be removed. |
| NR UL | D2R | Legacy UE IBE. Sector 6.4.2.3 of TS 38.101. Legacy UE IBE requirement is related to EVM requirement, to simplify the calibration, it's suggested to directly use QPSK, i.e. 17.5% EVM for IBE requirement. | ACS = 46dB for UE1, UE2 and UE3 |
| R2D | NR DL | ACLR = 45dB | Assuming ACS is very large, the A-IoT RB can be removed. |
| NR DL | R2D | Considering BS IBE Option 1: 30dBc Option 2: 17dBc Companies to check the leakage of legacy NR BS. | for device 2b, ACS = 30dB for device c, ACS = 35dB |

# 4  References

[1] R4-2515152 WF for [116bis][321] Study of A-IoT for outdoor_RAN4