3GPP TSG RAN4 Meeting #117						    	         R4-2521273

Dallas, USA, Nov 17-21, 2025

Agenda item:	7.7.3

Source:	Huawei, HiSilicon

Title:	Discussion on UAV co-existence evaluation

Document for:	Discussion

Background

The new WID for enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) was approved at RAN#109 [1]. In RAN4#116bis, the discussion on co-existence evaluation was initiated, and the agreements were captured in [2]. This document provides our further thinking on the co-existence evaluation aspect.

Discussion

Co-existence scenario

In RAN4#116bis meeting, 8 coexistence scenarios were discussed. The scenarios 4 and 8 could be considered as high priority, as they are directly related to the major objective of this WI, i.e. the higher UE power class.

The scenarios 1, 2, 5 and 6 are checking the performance of UAV BS and TN UE, which is not relevant with current WI. We propose to remove these scenarios.

The scenarios 3 and 7 are also proposed by some company. These scenarios are related to UAV UEs, but only for the Rx performance. We don’t think these scenarios are relevant to current WI since the objective consists only the increased transmit power. These scenarios might worth studying but the WID needs to be revised first to include the Rx perspective.

Table 1: Scenarios for UAV coexistence study

|  | Aggressor |  | Victim |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | deployment scenario UL/DL | System BW duplex mode | deployment scenario UL/DL | System BW duplex mode | Simulation frequency | ACLR/ACS requirements |
| 1 | TN UL | 100MHz TDD | UAV UL | 100MHz TDD | 4 GHz | UAV BS ACS |
| 2 | UAV DL | 100MHz TDD | TN DL | 100MHz TDD | 4 GHz | UAV BS ACLR |
| 3 | TN DL | 100MHz TDD | UAV DL | 100MHz TDD | 4 GHz | UAV UE ACS |
| 4 | UAV UL | 100MHz TDD | TN UL | 100MHz TDD | 4 GHz | UAV UE ACLR |
| 5 | TN UL | [20MHz] FDD | UAV UL | [20MHz] FDD | 2 GHz | UAV BS ACS |
| 6 | UAV DL | [20MHz] FDD | TN DL | [20MHz] FDD | 2 GHz | UAV BS ACLR |
| 7 | TN DL | [20MHz] FDD | UAV DL | [20MHz] FDD | 2 GHz | UAV UE ACS |
| 8 | UAV UL | [20MHz] FDD | TN UL | [20MHz] FDD | 2 GHz | UAV UE ACLR |

Proposal 1: The co-existence study only considers scenarios 4 and 8 with UAV UL as aggressor.

Deployment scenario

In RAN4#116bis meeting, UMa and RMa are considered as candidate deployment scenarios for coexistence analysis. Given the main target is to study the impact from increased UE transmit power, we think worse case is RMa since the TN BS will be receiving higher interference power from UAV UEs through LOS channel.

Proposal 2: RMa is used for evaluating the scenarios 4 and 8 with UAV UL as aggressor.

Network layout

In RAN4#116bis meeting, it’s discussed whether to consider co-located or non-collocated network layout. The agreed BS deployment is 19BS sites, and ISD 2~4km for UAV and 500~1732m for TN. Below figures shows the layout of ISD 2km for UAV+500m for TN, and ISD 4km for UAV and 1732m for TN. The figures show that only single pair of BS could be collocated. Therefore, we could consider non-collocated case as default assumption, even if a subset of BSs is collocated.

Proposal 3: Non-collocated case is used for coexistence evaluation. A subset of BSs might be collocated.

UE antenna orientation

In ATG discussion, the UE antenna orientation is assumed as ‘Single UE panel deployed on the abdomen of the airplane facing downwards and with the longest dimension of the array aligned with the direction of the flight route’. For UAV, we also need to discuss the assumption of UE antenna orientation. Currently the UAV array antenna is assumed as 2x2 and there is no longest dimension. We would like to propose ‘Single UE panel deployed on the bottom of the UAV UE facing downwards’.

Proposal 4: The UAV UE antenna orientation is assumed as ‘Single UE panel deployed on the bottom of the UAV UE facing downwards’.

Conclusion

This document provides our view on the UAV co-existence evaluation. Following proposals and observations are for RAN4 discussion.

Proposal 1: The co-existence study only considers scenarios 4 and 8 with UAV UL as aggressor.

Proposal 2: RMa is used for evaluating the scenarios 4 and 8 with UAV UL as aggressor.

Proposal 3: Non-collocated case is used for coexistence evaluation. A subset of BSs might be collocated.

Proposal 4: The UAV UE antenna orientation is assumed as ‘Single UE panel deployed on the bottom of the UAV UE facing downwards’.

References

[1] RP-252954, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), CMCC

[2] R4-2515159, Way Forward for [116bis][319] NR_UAV, Qualcomm, CMCC