3GPP TSG-RAN WG4 Meeting #117	R4-2521322

Dallas, USA, 17th – 21st November 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR MIMO Phase 5 demodulation requirements

Agenda item:	6.12.3

Document for:	Discussion

# Introduction

In RAN#116bis meeting, companies discussed on 3-antenna-port codebook-based transmissions for BS demodulation requirements. In this paper, we present our view on the open issues, along with UE demodulation requirements.

# BS Demodulation requirements

Open issue:

FFS on whether to introduce test applicability rule for 3Tx

In the previous meeting, companies agreed to define PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions [1] and all simulation assumptions have been agreed. The only remaining open issue is as follows:

2.1 Discussion

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

The test applicability rule for 4Tx is given as follows in TS 38.141-1:

Based on our understanding, similar test applicability can be defined for 3Tx antenna tests.

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 3 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

Proposal 1: Introduce test applicability rule for 3Tx as follows:

Further, since all simulation assumptions are agreed, we have provided simulations results in our other contribution [2] for alignment.

# UE Demodulation requirements

3.1	CSI reporting requirements

3.1.1	Refined Type I Single-Panel Codebook (typeI-SinglePanel-r19)

Issue 1-2-2: Test metric for 64 CSI-RS ports Proposals: - Option 1: Modify the legacy test metric to following: - Option 1A: X=5dB γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNR_ue +[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.. - Option 2: Consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed

RAN4#116bis agreed with the detailed simulation parameters for Rel-19 typeI-SinglePanel codebook with 64 CSI-RS ports, as shown in Table 1.

Table 1	Configurations of PMI reporting requirements with Rel-19 refined Type-I single panel codebook.

| Test case | CSI-RS ports (N1, N2) | Port mapping method (portMappingMethod) | Codebook mode (typeI-codebookMode-r19) | CMR configuration (K and nrofPorts per CSI-RS resource) | Number of slots for CSI-RS transmission | CSI-RS configuration and density |
| --- | --- | --- | --- | --- | --- | --- |
| 64 ports | 64 (8, 4) | Method 1 | Mode A | K=2 32 CSI-RS ports | One slot | Row: 17 Density: 1 |

Figure 1 and Figure 2 show our simulation results for 64 CSI-RS ports with FDD and TDD, respectively. For 64 CSI-RS ports scenario, one open issue is how to set the PMI reporting performance requirements for larger CSI-RS ports.

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 1: PMI reporting with Rel-19 typeI-SinglePanel for 64 CSI-RS ports (FDD).

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 2: PMI reporting with Rel-19 typeI-SinglePanel for 64 CSI-RS ports (TDD).

Table 2 summarizes the simulation results, where Gamma is throughput ratio between follow PMI and random PMI where SNR corresponds to 90% of the maximum throughput with follow PMI. We also show alternative throughput ratio where random PMI is measured at the SNR corresponds the 90% of the maximum throughput with follow PMI with the offset of XdB. We call it ‘Gamma-XdB’ on the table, where X = [3,5,7].

Table 2	Summary of Rel-19 TypeI PMI 64 CSI-RS ports reporting simulation results.

| FDD/TDD | UE Rx | MCS/rank | SNR to achieve 90% of the peak with follow PMI | Throughput ratio between follow PMI and random TypeI PMI (Gamma) | Throughput ratio with SNR offset of 3dB for random TypeI PMI (Gamma-3dB) | Throughput ratio with SNR offset of 5dB for random TypeI PMI (Gamma-5dB) | Throughput ratio with SNR offset of 7dB for random TypeI PMI (Gamma-7dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 2Rx | 22/rank2 | 9.6 | 11.7 | 5.9 | 4.3 | 3.2 |
|  | 4Rx | 22/rank2 | 2.7 | 30.7 | 10.3 | 6.7 | 4.6 |
| TDD | 2Rx | 22/rank2 | 9.6 | 8.3 | 4.7 | 3.5 | 2.8 |
|  | 4Rx | 22/rank2 | 2.1 | 18.6 | 8.9 | 5.7 | 4.1 |

Since the existing Rel-15 TypeI PMI reporting requirements with 32 CSI-RS ports sets gamma=5.0 for 2Rx and gamma=7.0 for 4Rx, we propose to set X=5 to define the typeI-SinglePanel-r19 PMI reporting requirements with 64 CSI-RS ports.

Proposal 1: For typeI-SinglePanel-r19 PMI reporting requirements with 64 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput obtained at SNRue+XdB with random precoding, where X=5dB.

3.1.2	Refined Type I Single-Panel Codebook (typeI-SinglePanel-r19)

Simulation results

Figure 3 and Figure 4 show our simulation results of Rel-19 eTypeII with 64 Tx ports for FDD 15kHz and TDD 30kHz, respectively. Table 3 also summarizes the simulation results of Rel-19 eTypeII 64 ports.

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 3: Rel-19 eTypeII 64 CSI-RS ports simulation results (FDD).

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 4: Rel-19 eTypeII 64 CSI-RS ports simulation results (TDD).

Table 3	Summary of Rel-19 eTypeII PMI 64 CSI-RS ports reporting simulation results.

| FDD/TDD | UE Rx | MCS/rank | SNR to achieve 90% of the peak with follow PMI | Throughput ratio between follow PMI and random TypeI PMI (Gamma) | Throughput ratio with SNR offset of 5dB for random TypeI PMI (Gamma-5dB) | Throughput ratio with SNR offset of 7dB for random TypeI PMI (Gamma-7dB) | Throughput ratio with SNR offset of 10dB for random TypeI PMI (Gamma-10dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 2Rx | 22/rank2 | 3.6 | 111.3 | 15.6 | 8.9 | 5.0 |
|  | 4Rx | 22/rank2 | 0.1 | 103.7 | 12.0 | 7.8 | 4.3 |
| TDD | 2Rx | 22/rank2 | 7.4 | 13.6 | 4.9 | 3.7 | 2.6 |
|  | 4Rx | 22/rank2 | 1.7 | 20.1 | 6.1 | 4.3 | 2.9 |

Same as Rel-19 TypeI 64 CRSI-RS ports simulation results, RAN4 should consider adding offset to SNR when TE measures the random PMI throughput, because the throughput ratio becomes very high.

Since the existing Rel-16 eTypeI PMI reporting requirements with 16 CSI-RS ports sets gamma=2.2 for 2Rx/4Rx, we propose setting X=10 to define the eTypeII-r19 PMI reporting requirements with 64 CSI-RS ports.

Proposal 2: For eTypeII-r19 PMI reporting requirements with 64 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput obtained at SNRue+XdB with random precoding with Rel-19 Type I codebook, where, X=10dB.

Codebook subset restriction

RAN4 PMI reporting test configures the codebook subset restriction to limit the reported PMI corresponding to a certain rank, e.g., rank 2. For Rel-19 eTypeII codebook, RAN1 has redefined new codebook subset restriction bitmap (typeII-CBSR-r19), which restricts the reported beam(s). On the other hand, the reported rank is configured by typeII-RI-restriction-r19.

The bitmap length of typeII-CBSR-r19 is given by N1N2/(X1(c)X2(c)), where X1(c) and X2(c) are configured by higher parameters valueOfX1-typeII-CBSR-r19 and valueOfX2-typeII-CBSR-r19, respectively. For example, in the case of 64 CSI-RS ports with N1,N2=(8,4), the candidates of X1(c),X2(c) are (1,1), (2,1), (2,2), (4,1), (4,2), according to TS 38.214 Table 5.2.2.2.5a-1.

For RAN4 eTypeII-r19 reporting requirements, we should configure to report all the beams without overhead reduction. We therefore propose to configure X1(c), X2(c)=(1,1), which means the bitmap length of typeII-CBSR-r19 is 32 (= 8 x 4 / (1 x 1)), and the corresponding bitmap is 0xFFFF FFFF.

Proposal 3: For eTypeII-19 PMI reporting requirements with 64 CSI-RS ports, configure typeII-CBSR-r19 = 0xFFFF FFFF, and (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) = (1, 1).

# Conclusion

4.1	BS Demodulation Requirements

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 3 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

Proposal 1: Introduce test applicability rule for 3Tx as follows:

4.2	UE Demodulation Requirements

Proposal 1: For typeI-SinglePanel-r19 PMI reporting requirements with 64 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput obtained at SNRue+XdB with random precoding, where X=5dB.

Proposal 2: For eTypeII-r19 PMI reporting requirements with 64 CSI-RS ports, use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput obtained at SNRue+XdB with random precoding with Rel-19 Type I codebook, where, X=10dB.

Proposal 3: For eTypeII-19 PMI reporting requirements with 64 CSI-RS ports, configure typeII-CBSR-r19 = 0xFFFF FFFF, and (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) = (1, 1).

# References

- R4-2514806, “WF on NR_MIMO_demod”, Samsung.

[2]  R4-2521323, “Simulation results for FR1 3Tx demodulation requirements”, Ericsson,  RAN4#117, November 2025.