3GPP TSG-RAN WG4 Meeting #117							   R4-2521327

Dallas, USA, Nov. 17-21, 2025

Title: 	On accuracy of CSI-RS based L1 measurement for SBFD operation

Source: 	OPPO

Agenda item:	6.13.3

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF on SBFD RRM performance requirements was approved [1].

In this contribution, we will provide our views on RRM performance impact for SBFD operation.

Discussion

### Issue 2-1-1: Measurement accuracy for CSI-RS based L1 measurements - Background - Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - Agreement from RAN4#114bis - Note: The following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - Way forward - Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB) - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), - for a UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB apply. - Option 2: accuracy requirements based on 24 RB apply. - for UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB apply

For CSI-RS L1 measurement with puncturing in frequency domain, we prefer RAN4 to define relaxed accuracy requirements based on 24 RBs for case 3, regardless of UE indicating CSI processing time scaled by 2 or not.

Proposal 1: For case 3, prefer to define relaxed accuracy requirements based on 24 RBs, regardless of UE indicating CSI processing time scaled by 2 or not.

Based on the agreed simulation assumption, the L1 measurements for both 24RBs and 48 RBs are simulated and the accuracy results are summarized in Table 1 below. About 0.6~0.8 dB performance loss is observed for 24 RBs compared with 48 RBs.

Table 1. Simulation results for CSI-RS L1-RSRP accuracy in Case 3

|  | SCS | CSI-RS (RB) | 5% | 95% | max (5%, 95%) |
| --- | --- | --- | --- | --- | --- |
| FR1 | 15kHz | 24 | -1.73 | 1.30 | 1.73 |
|  |  | 48 | -1.15 | 0.91 | 1.15 |
|  | 30kHz | 24 | -1.77 | 1.32 | 1.77 |
|  |  | 48 | -1.07 | 0.91 | 1.07 |
| FR2 | 120kHz | 24 | -1.81 | 1.32 | 1.81 |
|  |  | 48 | -1.09 | 0.98 | 1.09 |

Observation 1: For CSI-RS L1 measurement, 0.6~0.8 dB performance loss is observed for 24 RBs compared to 48 RBs.

To be conservative, we propose to introduce 1dB margin for the relaxed accuracy requirement, based on legacy CSI-RS based L1-RSRP accuracy requirements defined in clause 10.1.19.2 in TS38.133.

Proposal 2: For accuracy requirement for CSI-RS L1 measurement with 24 RBs in case 3, consider 1dB relaxation compared to the legacy accuracy with 48 RBs.

Conclusions

In this paper we provided our views on RRM impact for SBFD operation.

Proposal 1: For case 3, prefer to define relaxed accuracy requirements based on 24 RBs, regardless of UE indicating CSI processing time scaled by 2 or not.

Observation 1: For CSI-RS L1 measurement, 0.6~0.8 dB performance loss is observed for 24 RBs compared to 48 RBs.

Proposal 2: For accuracy requirement for CSI-RS L1 measurement with 24 RBs in case 3, consider 1dB relaxation compared to the legacy accuracy with 48 RBs.

Reference

- R4-2515054, WF on SBFD RRM performance