3GPP TSG-RAN WG4 Meeting #117	R4-2521328

Dallas, USA, Nov. 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TC of OD-SSB based deactivated SCell measurement - case 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN4 #116bis, RAN4 agreed to introduce the NES Test case of OD-SSB based deactivated SCell measurement in Case 1 in FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the test case of deactivated SCell measurement in Case 1 based on OD-SSB to verify UE behaviour in FMW and relevant to T2. TC7; Intra-frequency measurement (Deactivated SCell) based on OD-SSB; Deactivated SCell measurement; Interruption during deactivated SCell measurement; OD-SSB periodicity 10ms; OD-SSB Case 1 FMW; UE behaviour relevant to T2 FR1; OPPO. |
| Consequences if not approved: |  | The related test case is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.x.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------- CHANGE #1 ------------------------------

A.6.6.x.1	OD-SSB based deactivated SCell measurement under non-DRX mode in FR1 (OD-SSB Case 1)

A.6.6.x.1.1	Test Purpose and Environment

The purpose of this test is to verify that the OD-SSB based deactivated SCell measurement are within the requirements stated in clause 9.17.5.3 and 9.17.5.4.

The supported test configurations are shown in table A.6.6.x.1.1-1 below. The test parameters are given in table A.6.6.x.1.1-2 and cell-specific parameters in table A.6.6.x.1.1-3 below.

There are two NR carriers, each with one cell. Both cells have constant signal levels and the SCell activation command is not configured throughout the test.  Measurement gap or DRX is not configured, and measCycleSCell is configured as 160 ms.

At the start of T1, the UE is in connected mode to Cell 1. During time duration T1, the UE is configured to add Cell 2 as a deactivated SCell without a first SSB transmission, and then the first activation of OD-SSB transmission is indicated on Cell 2. At the end of T1, OD-SSB transmission is indicated to be deactivated.

During T2, after processing of OD-SSB deactivation command, UE does not measure on Cell 2. UE only needs to pass one of the two cases (case A and B) depending on the duration of T2. At the end of T2 and before the start of T3, UE is indicated the second activation of OD-SSB transmission at slot n.

During T3, after the slot n+ THARQ+3ms+TprocessingNR slot lengthwhen UE completes the command of OD-SSB activation, UE performs the deactivated SCC measurement as follows.

- Case A: T2 is 320ms (less than 5*measCycleSCell in FR1)

For Case A, during T3, UE performs the legacy deactivated SCC measurement and report (based on TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-3, 9.17.5.3-5 and 9.17.5.4-2 for FR1), until UE is indicated the second deactivation of OD-SSB transmission.

The test under case A consists of 3 successive time periods, with duration of T1, T2 and T3, respectively.

- Case B: T2 is 960ms (larger than 5* measCycleSCell in FR1)

For Case B, during T3, UE performs the deactivated SCell measurement during FMW (based on TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-1, 9.17.5.3-2 and 9.17.5.4-1 for FR1), until UE reports the deactivated Scell measurement results.

After that, during T4, UE performs the legacy deactivated SCC measurement and report (based on TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.17.5.3-3, 9.17.5.3-5 and 9.17.5.4-2 for FR1), until UE is indicated the second deactivation of OD-SSB transmission.

The test under case B consists of 4 successive time periods, with duration of T1, T2, T3, and T4, respectively.

The UE is required to make correct and timely reporting during T3 and T4. This test case verifies that the UE reporting is within the delay requirement and measurement accuracy is tested in other test cases.

In addition, this test also verifies the requirements of PCell interruption due to measurements on deactivated Scell during T3 and T4. The UE is allowed with up to 0.5% probability of missed ACK/NACK rate for PCell in standalone NR specified in clause 8.2.2.2.

Table A.6.6.x.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, PCell with 10 MHz and SCell with 10 MHz bandwidth, both FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, PCell with 10 MHz and SCell with 10 MHz bandwidth, both TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, PCell with 40 MHz and SCell with 40 MHz bandwidth, both TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.3.x.1-2: General test parameters for deactivated SCell measurements in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | Not configured | Non-DRX |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | s | 0.32(case A) 0.96(case B) |  |
| T3 | s | 3 (case A) 0.5(case B) |  |
| T4 | s | 2.5 (case B) |  |

Table A.6.6.x.1-3: Cell specific test parameters for NR PCell

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |
|  | Config 3 |  | Note 7 |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1Note 5 |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| reportConfigType |  |  | periodic |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |
|  | Config 3 |  | 10 |  |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |  |
|  | Config 3 |  | 5 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |
|  | Config 3 |  | -101 |  |  |
|  |  | dB | 17 |  |  |
|  |  | dB | 17 |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |
|  | Config 3 |  | -84 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36 MHz | -58.96 |  |  |
|  | Config 3 | dBm/ 38.16 MHz | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | - | 2x2 Low |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 [26] clause 5.2.2.5. |  |  |  |  |  |

Table A.6.6.x.1-4: Cell specific test parameters for NR SCell

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |
| BWchannel | ConfigSCell 1,2 | MHz | Note 7 |  |  |
|  | ConfigSCell 3 |  | Note 7 |  |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1Note 5 |  |  |
|  | ConfigSCell 3, |  | OP.1 Note 6 |  |  |
| OD-SSB Configuration | ConfigSCell 1,2 |  | SSB.x1 FR1 |  |  |
|  | ConfigSCell 3 |  | SSB.x4 FR1 |  |  |
| CSI-RS configuration for CSI reporting Note 8 | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |
| reportConfigType |  |  | N/A |  |  |
| reportQuantity |  |  | N/A |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |
|  |  | dB | 17 | 0 | 17 |
|  |  | dB | 17 | 0 | 17 |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -87 | 0 | -87 |
|  | ConfigSCell 3 |  | -84 | 0 | -84 |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 | 0 | -87 |
| Io Note3 | ConfigSCell 1,2 | dBm/ 9.36 MHz | -58.96 | 0 | -58.96 |
|  | ConfigSCell 3 | dBm/ 38.16 MHz | -52.87 | 0 | -52.87 |
| Propagation condition |  | - | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[18] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |  |

A.6.5.3.x.2	Test Requirements

For Case A, the UE shall send one measurement report for the deactivated SCell, with a measurement reporting delay less than 2080 ms from the beginning of time period T3.

The UE is required to successfully detect PSS/SSS for Cell 2 within

Ceil(5 x Kp) x measCycleSCell x CSSFintra;

The UE is required successfully read the SSB index for Cell 2 within

Ceil(3 x Kp) x measCycleSCell x CSSFintra;

The UE is required to successfully measure on Cell 2 within

Ceil(5 x Kp) x measCycleSCell x CSSFintra;

Where

-	measCyclePSCell = 160 ms,

-	Kp = 1,

-	CSSFintra = 1.

This sums up as 2080 ms in total, consisting of 800 ms of PSS/SSS detection, 480 ms of SSB index detection and 800 ms of measurement time.

For Case B, the UE shall send one measurement report for the deactivated SCell, with a measurement reporting delay less than 130 ms from the beginning of time period T3. The UE shall send another measurement report for the deactivated SCell, with a measurement reporting delay less than 2080 ms (as calculated for Case A above) from the beginning of time period T4.

The UE is required to successfully detect PSS/SSS for Cell 2 within

Ceil(M x Kp) x OD-SSB periodicity x CSSFintra;

The UE is required successfully read the SSB index for Cell 2 within

Ceil(3 x Kp) x OD-SSB periodicity x CSSFintra;

The UE is required to successfully measure on Cell 2 within

Ceil(M x Kp) x OD-SSB periodicity x CSSFintra;

Where

-	OD-SSB periodicity = 10 ms,

-	Kp = 1,

-	CSSFintra = 1.

-	M = 5.

The sums up as 130 ms in total, consisting of 800 ms of PSS/SSS detection, 480 ms of SSB index detection and 800 ms of measurement time

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

The same test requirements for interruptions during measurements on deactivated NR SCell in FR1 can apply, as defined in clause A.6.5.2.1.2. The UE shall be continuously scheduled on PCell during T3 and T4. During the time duration T3 and T4 the UE shall transmit at least 99.5 % of ACK/NACK on PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

-------------------- END OF CHANGE 1--------------------------