# 3GPP TSG-RAN WG4 Meeting #117	                                                        R4-2521344

# Dallas, USA, November 17th– 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on test case for EMR based normal SCell activation in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The requirements for EMR based normal SCell activation are introduced in Rel-19 and the corresponding test cases should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the test case for EMR based normal SCell activation in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for EMR based normal SCell activation delay requirements in FR2 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.7.5.3.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.7.5.3.X	EMR based SCell activation of unknown SCell in FR2

A.7.5.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the EMR (early measurement reporting) based SCell activation delay is within the requirements stated in clause 8.3.2A, when the SCell in FR2 is unknown by the UE at the time of activation.

In the test, there are two cells: NR Cell 1 as PCell in FR2 on NR RF channel 1 and NR Cell 2 as SCell in FR2 on NR RF channel 2. The supported test configurations are shown in table A.7.5.3.X.1-1 below. The general test parameters are described in Tables A.7.5.3.X.1-2, and cell specific test parameters are described in Tables A.7.5.3.X.1-3. OTA related test parameters are shown in table A.7.5.3.X.1-4 below.

The test consists of 6 successive time periods, with time duration of T1, T2, T3, T4, T5 and T6 respectively.

During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2. UE is configured with early measurement reporting for Cell 2 in:

measIdleCarrierListNR-r16 for UE supporting measValidationReportEMR-r18 or idleInactiveNR-MeasReport-r16 only, or

measReselectionCarrierListNR-r18 for UE supporting measValidationReportReselectionMeasurements.

Beam level reporting for early measurements is configured. The time point when UE receives RRC_Release message from the TE defines as the starting point of T2.

At the beginning of T2, Cell 2 becomes detectable however no cell reselection is being performed. Signal level of Cell 2 is set to fixed value according to Table A.7.5.3.X.1-4. The duration of T2 is set to fixed value according to the Table A.7.5.3.X.1-2.

At the beginning of T3, the signal level of the Cell 2 is set to another value according to the table A.7.5.3.X.1-4. The duration of the T3 equals to measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 depending on the UE capabilities of the UE under test.

During T2 and T3, UE is in RRC_IDLE mode.

The time when TE sends the paging message is defined as the starting point of T4. During T4, the UE shall send a valid measurement report with SSB index of Cell 2 to the PCell.

At the beginning of T5 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. UE is only monitoring the PCC and shall be continuously scheduled in the PCell when UE is connected to PCell. Then the test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T6.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

Table A.7.5.3.X.1-1: Supported test configurations for EMR based SCell activation in FR2

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.3.X.1-2: General test parameters for EMR based SCell activation in FR2

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels are used for this test. RF channel number 1 is in band 1 and RF channel number 2 is in band 2, where bands 1 and 2 are inter-band CA operating bands in FR2 as specified in Table 5.2A.2-1 in TS 38.101-2 [19]. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | s | ≤8 | A random value from 0s to 8s |
| T331 | s | 30 |  |
| T1 | s | 10 | During this time UE connects to PCell and early measurement report on the carrier frequency of Cell 2 is configured. |
| T2 | s | 30 | During this time UE is in RRC_IDLE state and Cell 2 becomes detectable. |
| T3 | s | 5 | During this time UE determines the validity of reported idle/inactive and reselection measurements. |
| T4 | s | 1 | During this time UE shall send a valid measurement report with SSB index of Cell 2 to the PCell. |
| T5 | s | 4 | During this time SCell (Cell 2) is configured via RRC signalling. |
| T6 | s | 1 | During this time UE completes the activation of SCell (Cell 2). |
| Filter coefficient |  | 0 | L3 filtering is not used |
| measIdleValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportEMR-r18 is tested in this test. |
| measReselectionValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportReselectionMeasurements is tested in this test. |
| measIdleDuration-r16 | s | 60 | Value for timer T331 for UE performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 10 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.7.5.3.X.1-3: Cell specific test parameters in Idle and Connected mode for EMR based SCell activation in FR2

| Parameter |  | Unit | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 | T1 | T2 | T3 | T4 | T5 | T6 |
| NR RF Channel Number |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |  |
| Duplex mode |  |  | TDD |  |  |  |  |  | TDD |  |  |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  |  | TDDConf.3.1 |  |  |  |  |  |
| BWchannel |  | MHz | 100: NPRB,c = 66 |  |  |  |  |  | 100: NPRB,c = 66 |  |  |  |  |  |
| BWP BW |  | MHz | 100: NPRB,c = 66 |  |  |  |  |  | 100: NPRB,c = 66 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |  |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  | TCI.State.0 |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | OP.1 |  |  |  |  |  | OP.1 |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  |  |  |  |  | NA |  |  |  |  |  |
| RMSI CORESET Parameters |  |  | CR.3.1 TDD |  |  |  |  |  | NA |  |  |  |  |  |
| Dedicated CORESET Parameters |  |  | CCR.3.1 TDD |  |  |  |  |  | NA |  |  |  |  |  |
| CSI-RS configuration |  |  | NA |  |  |  |  |  | CSI-RS.3.1 TDD Note 2 |  |  |  |  |  |
| CSI reporting periodicity Note 3 |  |  | NA |  |  |  |  |  | 5 |  |  |  |  |  |
| SSB parameters |  |  | SSB.1 FR2 |  |  |  |  |  | SSB.1 FR2 |  |  |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | SMTC.1 |  |  |  |  |  | SMTC.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |  |  | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: CSI-RS for CSI measurement is (re)configured in the next DL slot after slot n+ TFirstSSB_MAX + 15*TSMTC_MAX during T2. NOTE 3: L1-RSRP measurement and reporting are configured to the the UE prior to the start of time period T1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.3.X.1-4: OTA related test parameters for EMR based SCell activation in FR2

| Parameter |  | Unit |  |  | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 | T1 | T2 | T3 | T4 | T5 | T6 |
| Angle of arrival configuration |  |  | Setup 3 defined in clause A.3.15.1 |  |  |  |  |  | Setup 3 defined in clause A.3.15.1 |  |  |  |  |  |
|  |  |  |  |  | AoA1 |  |  |  |  |  | AoA2 |  |  |  |  |  |
| Assumption for UE beams Note 7 |  |  | Rough |  |  |  |  |  | Rough |  |  |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15 kHz | -104.7 |  |  |  |  |  | -104.7 | -104.7 | -104.7 | -104.7 | -104.7 | -104.7 |
| Note 1 | Config 1,2,3 | dBm/SCS | -95.7 |  |  |  |  |  | -95.7 | -95.7 | -95.7 | -95.7 | -95.7 | -95.7 |
|  | Config 1,2,3 | dB | 7 |  |  |  |  |  | -infinity | 7 | -0.5 | -0.5 | -0.5 | - 0.5 |
|  | Config 1,2,3 | dB | 7 |  |  |  |  |  | -infinity | 7 | -0.5 | -0.5 | -0.5 | -0.5 |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS | -88.7 |  |  |  |  |  | -infinity | -88.7 | -96.5 | -96.5 | -96.5 | -96.5 |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz | -58.92 |  |  |  |  |  | -66.68 | -58.92 | -69.3 | -69.3 | -69.3 | -69.3 |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Void NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: Void NOTE 6: Void NOTE 7: Information about types of UE beam is given in clause B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.7.5.3.X.2	Test Requirements

During T6, the UE shall start reporting CSI after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot n+THARQ+3 msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed.

During T6, the UE shall be able to send CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3.2A, where

-		THARQ is defined in Table A.7.5.3.X.1-2

-		Tactivation_time = TFirstSSB_MAX + 15*TSMTC_MAX + max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), which allows 350 ms.

- For this test case, TFirstSSB_MAX=TSMTC_MAX=Trs=20 ms.

-		TCSI_Reporting = 10 ms

-		NR slot length is 0.125 ms for this test case.

The observed SCell activation delay fulfiiling the SCell activation delay requirements specified in clause 8.3.2A in TS 38.133 is counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90 %.

<End of Change 1>