3GPP TSG-RAN WG4 Meeting #117	                                                                                   R4-2521347

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	7.12.3

Source: 	OPPO

Title: 	Discussion on RAN4 requirements for RRM measurement prediction

Document for:	Approval

- Introduction

In last meeting, the requirements for AI mobility including measurement prediction and indirect event prediction were discussed. The agreements and open issues are captured in the approved WF [1]. In this contribution, we will further discuss the identified requirements and open issues for measurement prediction as captured in the topic summary [2] and give our proposals.

- Discussion

2.1 Scenarios and sub-use cases

In last meeting, it was agreed that the following scenarios can be discussed for the potential RRM requirements and the frequency domain prediction for FR2 is FFS.

Issue 1-2-2: Scenarios and sub-use cases Online agreements on Tuesday: Agreement: RAN4 to discuss the potential RRM requirements for cell-level based prediction for the following scenarios: Evaluation scenario; Methodology. FR1 to FR1 intra-frequency temporal domain case A; intra-cell. FR1 to FR1 intra-frequency temporal domain case B; Intra-cell. FR1 to FR1 inter-frequency (frequency domain) (co-located case); Inter-cell. FR2 to FR2 intra-frequency temporal domain case A; Intra-cell. FR2 to FR2 intra-frequency temporal domain case B; Intra-cell. FFS whether to discuss the requirements for the following scenario: Evaluation scenario; Methodology. FR2 to FR2 inter-frequency (frequency domain) (co-located case); Inter-cell.

Theoretically, we think the mechanism of frequency domain prediction for FR1 and FR2 should be similar, but there are also some specific aspects we can see e.g., beam configurations, pathloss characteristics, limited coverage, that may impact the performance and the applicability of frequency domain prediction. On the other hand, according to the discussions in Rel-19 beam management, many issues regarding FR2 OTA testing are not resolved and may need further studies in future releases. To reduce the workload and avoid the same situation, we think the FR2 to FR2 frequency domain prediction can be deprioritized.

Proposal 1: RAN4 to deprioritize the scenario of FR2 to FR2 inter-frequency (frequency domain) prediction (co-located case).

2.2 Measurement and prediction related requirements

Based on the proposals in last meeting, the following requirements has been proposed for measurement prediction:

Issue 2-1-1: What kind of core requirements to be defined Further discuss based on the open issues in [1] and the following way forward: - Discuss whether the follow requirements need to be defined: - Measurement delay/period requirements (OPPO, CMCC, ZTE, Samsung, QC) - Measurement and scheduling restrictions in OW/PW (QC, Samsung) - Prediction delay requirements (OPPO) - Prediction reporting delay requirements (OPPO, HW, QC) - OW/PW length (HW, LGE, Samsung) - Data collection delay requirements (Ericsson) - SCell related requirements (Samsung, QC) - UE mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements (QC)

In our understanding, most of the above requirements existing in legacy need to be discussed e.g., measurement/reporting related requirements, scheduling restrictions, SCell related requirements and UE capabilities.  But for OW/PW length and data collection delay requirements, we think further clarifications are needed.

Regarding OW/PW length, based on the conclusions in study item, it is up to UE’s implementation to decide on OW length and the PW length is up to NW configuration. So we think OW and PW length is not a core requirement for measurement prediction. But if the length has impact on the performance requirements, RAN4 can further consider whether to define them as the side conditions.

##### 6.1.2.1.2 Inference configuration and report Existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report. When a full inference configuration is received, UE shall maintain it until it is released by network explicitly. Following inference parameters can be configured to UE for inference and assessing applicability: - PW length for intra-frequency temporal domain case A; - Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction; - Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement); - Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available); - Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction; - Optional associated ID. It is up to UE’s implementation to decide on model related choices including cluster-based vs single-cell-based approach, RRM sub-use cases and OW length.

Regarding the data collection for UE sided model, we understand the logged data is for UE sided model training. When UE receives the log configuration, it will log the data on its own side, but when and how to transfer it to server has not been discussed and decided in RAN2. On the other hand, the logged data is still L1 or L3 measurement, the corresponding delay will be defined in the measurement period, and we think we do not need to define additional delay requirements for data collection. But considering the data collection procedure is led by RAN2 and RAN2 has not started the normative work, we are also fine to wait for more progress from RAN2.

Regarding the prediction delay requirements and prediction reporting delay requirements, we have provided the analysis in [3] in last meeting and illustrated the procedures as the following figure which is same as the mechanism in beam management.

Figure 1: illustration of prediction reporting delay

And in last meeting, the following agreements were made in Rel-19 NR_AIML_air for beam management:

RAN4#116bis meeting： Issue 2-1: Measurement period for inference – case 2 Agreement: Measurement period for inference – case 2 K(number of samples)*[M]*N*P K as defined by RAN1 Issue 2-2: Reporting delay Agreement for reporting delay: Reuse d’ as to be agreed by RAN1 d’ to be added to the measurement period to reflect the extra time needed by the UE to perform inference Depending on the RAN1 specifications, d’ shall not be double counted in the overall delay for periodic, aperiodic or semi-persistent reporting Final specifications to be agreed after RAN1 includes d’ in the specs RAN1#121 meeting: Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively. - Detailed values of d and d’ can be further discussed in UE feature.

According to RAN1 agreements, the inference delay for BM-Case 1 and BM-Case 2 are defined as d and d’ respectively which is UE capability. And the detailed values are still under discussion. Based on the new RAN4 agreements and endorsed draft CR [4], the inference delay is included in measurement period requirements to reflect the extra time for inference. Since the inference delay depends on the AI model implementation, we think it should be similar for at least measurement related use cases. And the time used in beam management i.e., d’ as defined in RAN1 can be also applied to AI mobility. The framework of measurement period in BM-Case 2 can be the starting point for temporal domain prediction in AI mobility. With this understanding, the prediction delay is not needed to be defined separately and the illustration of prediction reporting delay can be updated as below.

Figure 2: illustration of prediction reporting delay (updated)

Proposal 2: RAN4 to discuss the following core requirements for measurement prediction:

    - Measurement delay/period requirements

    - Measurement and scheduling restrictions in OW/PW

    - Prediction reporting delay requirements

    - SCell related requirements

    - UE capability on the number of cells and/or frequency layers for actual measurements and inference-based measurements

Proposal 3: RAN4 to discuss whether the OW/PW length should be the side condition of the core requirements.

Proposal 4: RAN4 not to define the delay requirements for data collection.

Proposal 5: For inference delay, reuse the conclusion in beam management, i.e., d’ as defined in RAN1 can be added in the measurement period requirements.

In legacy measurements, the reporting delay requirements are defined for event triggered reporting and the first report in event triggered periodic measurement reporting. The reporting delay excludes the delay which caused by no UL resources being available for UE to send the measurement report on and excludes the delay for L3 filtering, i.e., only includes the delay for cell identification.

### 9.2.4 Measurement Reporting Requirements #### 9.2.4.1 Periodic Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). #### 9.2.4.2 Event-triggered Periodic Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event-triggered periodic measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.2.4.3. #### 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra_with_index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap. A cell is detectable only if at least one SSBs measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

For AI mobility, we think the same definition of reporting delay as legacy measurement can be reused, i.e., the prediction reporting delay is defined as the time between an event that will trigger a prediction report and the point when the UE starts to transmit the report over the air interface. According to this definition, the time for the first available reporting resource is not needed and the delay is same as the measurement period requirement.

Proposal 6: RAN4 to define prediction reporting delay for event triggered reporting and the first report in event triggered periodic reporting.

Proposal 7: For the prediction reporting delay requirements to be defined, the time for the first available reporting resource and the time for L3 filtering are not included.

For the measurement period except inference delay in above requirements, i.e., duration T1 as illustrated in figure 2, it is related to the different scenarios including temporal domain prediction case A and case B, and frequency domain prediction.

For temporal domain measurement prediction case A illustrated in Figure 3 as below, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW. And the samples with predicted results will be still actually measured.

Figure 3: Example of intra-frequency temporal domain case A

Since the measurements are not reduced and the measurement behaviour is same as legacy for case A, we understand the legacy measurement requirements can be reused.

Proposal 8: For temporal domain prediction case A, the legacy measurement period requirements can be reused.

For temporal domain prediction case B illustrated in Figure 4 and Figure 5 as below, measurement results in PW are predicted by historical measurement result(s) in OW, in which either continuous samples (Skipping pattern example 1) or non-continuous samples (Skipping pattern example 2) are allowed. The samples with predicted results will be skipped when performing measurement. And the measurement reduction rate is defined as MRRT assuming same length of measurement time instances, where,

- MRRT = skipped measurement time instances / total measurement time instances

Figure 4: Skipping pattern example 1 of intra-frequency temporal domain case B

Figure 5: Skipping pattern example 2 of intra-frequency temporal domain case B

Among the above examples, the NW transmission on the SSB/SMTC are periodical, but UE may have different skipping patterns for different MRRT or even same MRRT for case B. And for the measurement configuration of case B, RAN2 has the following agreements:

Following inference parameters can be configured to UE for inference and assessing applicability: - PW length for intra-frequency temporal domain case A; - Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction; - Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement); - Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available); - Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction; - Optional associated ID.

It means that the inference configuration of SSB resources is from NW transmission perspective rather than UE measurement perspective, i.e., NW will configure the available SSB resources but UE may not measure all the resources and can skip some resources based on UE implementation of temporal prediction case B. This solution can accommodate the legacy UE which does not support AI mobility. Based on the above SI conclusion, we understand the NW transmission and UE measurement/prediction for Case B can be illustrated as case B-1 and case B-2 as below, i.e., the NW will configure and transmit all the available resources periodically no matter they are for measurement or prediction and indicate UE to report the results for each resource. And if UE supports AI mobility, the reported results of some resources will be based on prediction.

Figure 6:  Case B-1 (skipping pattern example 1)

Figure 7: Case B-2 (skipping pattern example 2)

Based on the illustration of case B-1 and case B-2, the framework of the measurement period of case B can be similar as legacy, i.e., Tmeasurement is defined as Kscaling*M*max(MGRP, SMTC periodicity) or Kscaling*M*SMTC periodicity, where M is the number of samples to meet the accuracy requirements and Kscaling includes at least the scaling factor due to resource collision, gap sharing or searcher sharing which is similar as legacy. The value of M needs to be further discussed.

Initially, we think there are two approaches to define the number of samples. The first one is to define M as a fixed value including the total number of samples for both measurement and prediction, e.g., the legacy number of samples i.e., 5 samples is used as starting point. In this case, during each measurement period, only part of the samples (M1) are based on actual measurements while the others (M2) are based on prediction. Theoretically, the scaling factor due to collision or gap sharing or searcher sharing will only apply to the occasions for actual measurement, and an additional factor related to prediction implementation such as MRRT needs to be considered, e.g., M1 = N * M where N is a scaling factor related to the MRRT for AI mobility case B and N<1. But this approach will impact the measurement period of other frequency layers due to the skipped occasions by prediction. And the formula of measurement period will be complicated while the performance gain is limited. To reduce the impact on non-predicted measured carrier and reduce the complexity, we think the legacy requirements can still apply without differentiating the number of samples for measurement and prediction.

The second one is to define a fixed number of samples for actual measurement, e.g., if the legacy number of samples (i.e., Mlegacy = 5) are needed for actual measurement, the total number of samples M used in measurement period can be defined as M = N * Mlegacy where N is a scaling factor related to the MRRT for AI mobility case B and N>1.

Proposal 9: For temporal domain prediction case B, the measurement period requirements is defined as Tmeasurement = Kscaling*M*max(MGRP, SMTC periodicity) or Kscaling*M*SMTC periodicity. The value of Kscaling and M need to be further discussed.

For frequency domain measurement prediction, only co-located scenario is included, i.e., the cell to be measured and cell to be predicted are located in the same sector of either serving site or same neighbouring site. And for the prediction method, both signal cell prediction and cluster-based prediction are considered. Based on the above RAN2 agreements on inference parameters, network will configure the measured frequency carrier and predicted frequency carrier information for inter-frequency prediction. With this information, UE behavior on the configured measured frequency carrier can be same as legacy and the existing requirements can be reused.

Proposal 10: For frequency domain prediction, the legacy measurement requirements can be reused by clarifying in CSSF that the frequency layers configured for prediction are not counted.

2.3 LCM related requirements

According to the WID, for the LCM related discussion in AI mobility, the LCM framework for Rel-19 AI/ML for NR air interface WI can be reused as much as possible. Based on the discussion in Rel-19 AI/ML for NR air interface, the following agreements are reached and no other delay requirements except AI/ML functionality activation delay are defined.

Issue 1-1-3: Whether to create delay requirements for functionality activation/switching/fallback Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI Switching delay and Fallback delay: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion. General LCM principles: Agreement: - No need to create deactivation requirements for AI functionality LCM for beam management: Agreement: - Do not define performance monitoring delay requirements for LCM purpose - If the monitoring delay is identificated as critical in the future, this issue can be revisited. No agreements on performance monitoring accuracy. Issue 2-3: Reporting delay requirements for monitoring Agreement: Do not define any explicit requirements for reporting delay of RS-PAI performance monitoring metric.

We think the same conclusions can be reused for AI mobility except the activation delay and performance monitoring. For AI/ML functionality activation, before we reuse the conclusion, the scenario and procedure of activation need to be clarified. For performance monitoring, since the metrics can be different from beam management, we can postpone the discussion and wait for RAN2 progress.

Proposal 11: Do not define deactivation/switching/fallback delay requirements for AI functionality.

Proposal 12: For AI functionality activation delay and performance monitoring, wait for RAN2 progress on LCM framework.

2.4 Simulation assumption

2.4.1 Measurement error

According to the discussions in study phase, simulation is needed to define requirements of AI mobility and the measurement error on AI model input should be taken into account. And for the measurement error, the following assumptions are agreed as baseline:

- For BB error, use link level simulation to generate L3-RSRP difference as baseband error.

- For RF error model, use a truncated Gaussian distribution.

  - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

In legacy accuracy requirements, the baseband error derived from LLS and the RF margin are included. According to the discussions and agreements in Rel-15 [5][6][7] as cited below, 2.5dB RF margin is used in FR1 and 4dB RF margin is used in FR2 for L1-RSRP and L3-RSRP measurement.

R4-1904820:

R4-1809602 RAN4-AH-1807 Meeting report R4-1809356 Ad hoc minutes for NR RRM measurement accuracy and mapping table CR- rev Cat: () v Source: Qualcomm Abstract: This contribution provides the way forward on Discussion: Agreement: - RSRP accuracy for FR2: +/-[6]dB - The number could be revisited and revised in the future RAN meetings. Verizon: we can use Nokia’s number. Qualcomm: we can do it better. Ericsson: for LTE, we start with +/-6dB and had a long discussion. We would like to avoid the same situation. Decision: Approved WG4 Meeting AH-1807 R4-1809356 Proposals: Absolute RSRP: Nokia +-6dB (R4-1808790) QC: +-4.5dB (R4-1809123) Intel: FFS (R4-1808719) Relative RSRP(intra or inter): QC: +-4.5dB (R4-1809123) Discussion: Intel: what is the RF margin assumed? Nok: +-4dB as proposed earlier by QC, we would support tighter requirement as proposed by QC in this meeting QC: +-2.5dB, same calibration accuracy as FR1 is achievable

For AI mobility, since the AI model input is same as legacy L3-RSRP measurement, we think the legacy RF margin assumption can be reused, i.e., the variance of truncated Gaussian distribution can be 2.5dB for FR1 and 4dB for FR2.  The other issue for RF error model is whether to consider different parameters for different frequency layers. In reality, we can understand that the instantaneous RF error for different frequency layers or different time may be different, but we think we do not need to model such details in the simulation. And on the other hand, even if we use the same model for different frequencies, the actual added error on each sample can be different. So we think single model of Gaussian distribution is enough.

Proposal 13: Use the following assumption for RF error model in simulation and same variance can be used for different frequency layers:

- Truncated Gaussian distribution under ±4 dB RF error (μ=0,  σ2=4) for FR2

- Truncated Gaussian distribution under ±2.5 dB RF error (μ=0,  σ2=2.5) for FR1

Regarding the baseband error, based on the agreements, it will be derived based on link level simulation. Following the approaches in legacy, L1 filtering (i.e., number of samples) is included while L3 filtering is not included. And the accuracy is related to the SINR side condition. We understand the same procedures can be reused for AI mobility and the number of samples used in L1 filtering can be further discussed. Since the reference point for ideal dataset and added baseband error should be aligned, with this LLS approach, the ideal SS-RSRP dataset will also include L1 filtering and the assumption for number of samples should be same. According to such understanding, we provide the link level simulation assumption and the template of results collection for baseband error derivation in [8][9].

Proposal 14: For link level simulation which is used for deriving BB error, L1 filtering should be included while L3 filtering is not included. And the baseband error will be added on the ideal dataset after L1 filtering.

2.4.2 Simulation procedure:

In last meeting, the following agreement was reached to use the simulation procedures in AI/ML beam management as starting point for AI mobility evaluation.

Issue 1-3-1: Simulation assumption for measurement prediction Agreement: - Take the simulation procedures used in AI/ML beam management as starting point for the simulation discussion of measurement prediction and [indirect measurement event prediction] to evaluate the predicted L3-RSRP accuracy considering the impact of measurement error.

Referring to the simulation in beam management [10], we think the following procedures can be used as starting point for AI mobility simulation:

- Companies to generate ideal intra-frequency or inter-frequency SS-RSRP dataset from the SLS assumption. - Use the LLS assumptions to generate intra-frequency SS-RSRP difference as the baseband errors. - Use the Simulation assumptions for RF error to generate RF errors. - Add the measurement error into the ideal SLS dataset in 1st step to derive the dataset with measurement errors. - Both baseband errors and RF errors are included. - Measurement error assumptions follow 2nd step and 3rd step. - Add L3 filtering to the dataset with added measurement error generated from the 4th step. - Use subset samples of the dataset generated from the 5th step for training models. - Filter UEs in the other subset samples of the dataset with added measurement error (non-overlapped samples with the subset dataset in 6th step) based on SINR value (e.g., SINR>= -6dB as the starting point) to derive the dataset for inference. - Use the dataset with measurement error in 7th step for inference to evaluate the performance metrics (predicted L3-RSRP) assuming that the ground truth is with the measurement error. Note: Intra-frequency or inter-frequency SS-RSRP in 1st and 2nd step means the Cell level RSRP measurement without L3 filtering, which is same as that in RAN4 legacy SS-RSRP accuracy requirements.

For the dataset format, we understand it can be sample-by-sample manner. And there are some issues specific for AI mobility to be resolved which are mainly for measurement error addition, e.g.,

- Where to add the measurement error? Before or after L1 filtering?

- Whether to add different measurement errors based on different SINR values?

- How to decide the value to be added on each sample?

- Whether to align the L1 filtering approach (sample number and sliding approaches)?

- Whether to consider different L3 filter coefficient?

- Whether to filter the training/inference dataset with SINR > =[-6dB]?

- Whether and how to get ground truth with measurement error?

- Whether to configure multiple Tx beams for FR1 and FR2 respectively?

- Other issues…

To resolve the above issues related to L1/L3 filtering, we use the following measurement model defined in TS 38.300 as an illustration:

Figure 8: Measurement Model in TS 38.300

In legacy measurement requirements, the baseband accuracy is based on the simulation results at point B, i.e., after L1 filtering with 3 or 5 samples. To reuse the legacy link level simulation as much as possible, we think we can generate the ideal SS-RSRP dataset at point B using SLS and then add the measurement error including baseband and RF error at point B to get the L1 dataset. After that, the L3 filtering can be added to get the final dataset for training and inference. With this understanding, we illustrate the simulation procedure as below which can be used as the reference for the simulation.

Figure 9: Illustration of simulation procedure for measurement prediction

Proposal 15: Use Figure 9 as starting point for the simulation procedure of measurement prediction.

The next issue is how to add the measurement error in step 4. For RF error, with the agreed Gaussian distribution, we think we can derive a set of RF error values and add each value to the corresponding sample of the dataset.

For baseband error, according to the experience in legacy link level simulation, the accuracy is relevant to the SINR side condition. And since the dataset derived based on SLS simulation includes the samples under different SINR, we need to discuss whether and how to add different errors based on SINR values on each sample. We think the following options can be considered:

- Option 1: Single dataset based on the minimum SINR (e.g., -6dB).

- Option 2: Group the dataset based on different SINR range.

Option 2 is close to reality but will highly increase the simulation workload and the boundary for each group also need much discussions. And by using option 1, it means we will use the maximum measurement error and the legacy accuracy value can be reused. Since we are defining the minimum requirements, we think using option should be OK.

For both option 1 and option 2, the following approaches can be considered to add the measurement error on each sample in each group:

- Option A: Add a random error within [-X, +X] dB on each sample.

- Option B: Add error with Gaussian distribution under X dB (μ=0,  σ2=[X]) on each sample.

- Option C: Perform multiple simulations and add the errors derived from LLS on each sample.

Proposal 16: Discuss whether to add different baseband errors on different group of datasets.

- Option 1: Add one set of measurement errors on single dataset based on the minimum SINR (e.g., -6dB).

- Option 2: Group the dataset based on different SINR range and add different sets of measurement error on each group.

Proposal 17: For each group of datasets, consider the following approaches to add the measurement error on each sample:

- Option A: Add a random error within [-X, +X] dB on each sample.

- Option B: Add error with Gaussian distribution under X dB (μ=0,  σ2=[X]) on each sample.

- Option C: Perform multiple simulations and add the errors derived from LLS on each sample.

For the definition of ground truth, there are two alternatives left based on the study item discussion, where alt1 is to use the expected measurement results calculated based on transmission power and channel modelling and alt2 is to use the reported measurement results under certain conditions.

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

For alt1, the advantage is that it is easily calculated and TE side can clearly know the expected results at each timing point, but the issue is that TE does not know when the measurement is performed or predicted. Therefore, TE doesn’t know which one to be compared which may request additional report from UE on the time stamp. The other issue is that the performance requirements may be used in model monitoring in LCM procedure, we understand it may cause problem since UE cannot acquire the expected measurement results in real LCM implementation.

For alt2, according to the existing measurement model, from inputs at point A, UE needs to experience L1 filter, beam consolidation/selection and L3 filter to get L3 measurement results at point C in which the latter two parts are RRC configured. Based on the legacy procedure, when the SINR is high enough, the UE measurement results are approximate to the ideal values. The advantage is that this can accommodate different UE implementation e.g., L1 filtering, and can be aligned with model training and inference procedure. To define the prediction accuracy and testing, certain channel modelling, beam consolidation configuration and L3 filter configuration can be considered as possible side conditions. The issue for alt2 is that UE may not perform actual measurement on certain occasions in some of the prediction cases, but we think this can be resolved by setting additional procedure in the testing. The other concern is that both ground truth and report are from UE which makes UE possible to cheat in testing. But we understand the actual measurement used as ground truth still needs to meet the legacy requirements, and cheating in the testing is not benefit for UE since it may lead bad performance in real practice. So we do not need to assume that UE will cheat in the testing.

From the analysis above, we prefer to use alt2 as baseline to define the ground truth.

Proposal 18: Use Alt2 (i.e., reported measurement value under certain conditions) to define the ground truth for prediction accuracy in FR1.

Based on the above discussion, the simulation assumption for measurement prediction is provided in [8] as starting point.

- Conclusions

In this contribution, we discussed the RAN4 requirements for RRM measurement prediction and got the following observations and proposals:

Proposal 1: RAN4 to deprioritize the scenario of FR2 to FR2 inter-frequency (frequency domain) prediction (co-located case).

Proposal 2: RAN4 to discuss the following core requirements for measurement prediction:

    - Measurement delay/period requirements

    - Measurement and scheduling restrictions in OW/PW

    - Prediction reporting delay requirements

    - SCell related requirements

    - UE capability on the number of cells and/or frequency layers for actual measurements and inference-based measurements

Proposal 3: RAN4 to discuss whether the OW/PW length should be the side condition of the core requirements.

Proposal 4: RAN4 not to define the delay requirements for data collection.

Proposal 5: For inference delay, reuse the conclusion in beam management, i.e., d’ as defined in RAN1 can be added in the measurement period requirements.

Proposal 6: RAN4 to define prediction reporting delay for event triggered reporting and the first report in event triggered periodic reporting.

Proposal 7: For the prediction reporting delay requirements to be defined, the time for the first available reporting resource and the time for L3 filtering are not included.

Proposal 8: For temporal domain prediction case A, the legacy measurement period requirements can be reused.

Proposal 9: For temporal domain prediction case B, the measurement period requirements is defined as Tmeasurement = Kscaling*M*max(MGRP, SMTC periodicity) or Kscaling*M*SMTC periodicity. The value of Kscaling and M need to be further discussed.

Proposal 10: For frequency domain prediction, the legacy measurement requirements can be reused by clarifying in CSSF that the frequency layers configured for prediction are not counted.

Proposal 11: Do not define deactivation/switching/fallback delay requirements for AI functionality.

Proposal 12: For AI functionality activation delay and performance monitoring, wait for RAN2 progress on LCM framework.

Proposal 13: Use the following assumption for RF error model in simulation and same variance can be used for different frequency layers:

- Truncated Gaussian distribution under ±4 dB RF error (μ=0,  σ2=4) for FR2

- Truncated Gaussian distribution under ±2.5 dB RF error (μ=0,  σ2=2.5) for FR1

Proposal 14: For link level simulation which is used for deriving BB error, L1 filtering should be included while L3 filtering is not included. And the baseband error will be added on the ideal dataset after L1 filtering.

Proposal 15: Use Figure 9 as starting point for the simulation procedure of measurement prediction.

Figure 9: Illustration of simulation procedure for measurement prediction

Proposal 16: Discuss whether to add different baseband errors on different group of datasets.

- Option 1: Add one set of measurement errors on single dataset based on the minimum SINR (e.g., -6dB).

- Option 2: Group the dataset based on different SINR range and add different sets of measurement error on each group.

Proposal 17: For each group of datasets, consider the following approaches to add the measurement error on each sample:

- Option A: Add a random error within [-X, +X] dB on each sample.

- Option B: Add error with Gaussian distribution under X dB (μ=0,  σ2=[X]) on each sample.

- Option C: Perform multiple simulations and add the errors derived from LLS on each sample.

Proposal 18: Use Alt2 (i.e., reported measurement value under certain conditions) to define the ground truth for prediction accuracy in FR1.

- References

R4-2514827 WF on NR_AIML_Mob_RRM, RAN4#116bis

R4-2513532 Topic summary for [116bis][226] NR_AIML_Mob_RRM, RAN4#116bis

R4-2513097 Discussion on RAN4 requirements for RRM measurement prediction, RAN4#116bis

R4-2514630 Draft CR on RRM core requirements for support of AIML for NR Air Interface for beam management, RAN4#116bis

R4-1904820 WF on L1-RSRP accuracy requirements

R4-1809602 RAN4-AH-1807 Meeting report

R4-1809356 Ad hoc minutes for NR RRM measurement accuracy and mapping table

R4-2521345 Simulation assumption for measurement prediction, RAN4#117

R4-2521346 Template of simulation results collection for AI mobility, RAN4#117

R4-2511797 WF on updated simulation assumption for AI based beam management, RAN4#116