3GPP TSG-RAN WG4 Meeting #117	R4-2521354

# Dallas, USA, 17 - 21 Nov, 2025

Title: 	Discussion and simulation results on NR NTN Phase 3 demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	6.19.4

Document for:	Discussion

Background

During RAN4#116bis meeting, Way Forward [1] for Non-Terrestrial Networks (NTN) for NR Phase 3 was approved. In this contribution, we share our view on NR NTN Phase 3 demodulation performance requirements.

Discussion

UE side

Channel Model for requirements

Way Forward Option 1: Define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model Option 2: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenario - Companies are encouraged to provide simulation results in the next meeting with fixed frequency offset 200Hz and timing offset=[29Tc] to derive the additional margin

NTN for RedCap/eRedCap UE is the parallel Rel-19 WI with NGSO time varying channel model that is still under discussion with open issues left, it is contradictory with RAN4 work principle if RAN4 works on two parallel topics that have dependency. Rel-19 NTN for RedCap/eRedCap WI has been discussed for several meetings and is near to be completed in next March, i.e. only two meetings are left. Interesting companies have spent much efforts for simulation alignment with NTN channel model based on the agreed simulation assumptions reached in the previous meetings, it is unreasonable and unacceptable to change the channel model in the last two meetings and ask companies to re-simulate all cases for alignment. What’s more, the reason behind to approve Rel-19 WI NR_IoT_NTN_req_test_enh-Perf is to consider and respect interesting companies’ simulation results in the previous release and agree to work on a new WI in new release for the NTN time varying channel model study.

Agree Option 1, i.e. define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model.

Simulation results

RedCap 1Tx-1Rx, 10MHz/15kHz, FDD/HD-FDD

Table 2.1.2.1-1 Simulation results for RedCap

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52PRB | NTN-TDLA100-200 | 16 | 1x1 | 1.1 |
| 2 | MCS4, 52PRB | NTN-TDLC5-200 | 32 | 1x1 | 0.2 |
| 3 | MCS13, 52PRB | NTN-TDLC5-200 | 16 | 1x1 | 8.0 |

Table 2.1.2.1-2 FRC for RedCap FDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | Test 1/2 | Test 3 |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 13 |  |  |  |
| Modulation |  | QPSK | 16QAM |  |  |  |
| Target Coding Rate |  | 0.30 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 4096 | 13064 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | CBs | 1 | 2 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 13104 | 26208 |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 13728 | 27456 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 3.891 | 12.411 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table 2.1.2.1-3 FRC for RedCap HD-FDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | Test 1/2 | Test 3 |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 13 |  |  |  |
| Modulation |  | QPSK | 16QAM |  |  |  |
| Target Coding Rate |  | 0.30 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 2664 | 8456 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 4096 | 13064 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 16 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 1 | 2 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 1 | 2 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 13104 | 26208 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 8736 | 17472 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 13728 | 27456 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 2.786 | 8.876 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

eRedCap 1Tx-1Rx/2Rx, 10MHz/15kHz, FDD/HD-FDD

Without baseband bandwidth reduction capability

Table 2.1.2.2.1-1 Simulation results for eRedCap w/o baseband bandwidth reduction

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | 1.1 |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 | 8.0 |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 | - |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 | 4.8 |

Table 2.1.2.2.1-2 FRC for eRedCap FDD w/o baseband bandwidth reduction

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | Test 1/3 | Test 2/4 |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 40 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 13 |  |  |  |
| Modulation |  | QPSK | 16QAM |  |  |  |
| Target Coding Rate |  | 0.30 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 4096 | 9992 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | CBs | 1 | 2 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 13104 | 20160 |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 13728 | 21120 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 3.891 | 9.492 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table 2.1.2.2.1-3 FRC for eRedCap HD-FDD w/o baseband bandwidth reduction

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | Test 1/3 | Test 2/4 |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 52 | 40 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 13 |  |  |  |
| Modulation |  | QPSK | 16QAM |  |  |  |
| Target Coding Rate |  | 0.30 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 2664 | 6400 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 4096 | 9992 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 16 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 1 | 1 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 1 | 2 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 13104 | 20160 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 8736 | 13440 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 13728 | 21120 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 2.786 | 6.776 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

With baseband bandwidth reduction capability

Table 2.1.2.2.2-1 Simulation results for eRedCap w/ baseband bandwidth reduction

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR |
| --- | --- | --- | --- | --- | --- |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 | 1.4 |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 | 7.6 |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 | -2.6 |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 | 4.7 |

Table 2.1.2.2.2-2 FRC for eRedCap FDD w/ baseband bandwidth reduction

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | Test 1/3 | Test 2/4 |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 25 | 25 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 19 | 19 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 13 |  |  |  |
| Modulation |  | QPSK | 16QAM |  |  |  |
| Target Coding Rate |  | 0.30 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 2024 | 6272 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | Bits | 16 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slots i = 1,…, 19 | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 6300 | 12600 |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 6600 | 13200 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 1.923 | 5.958 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table 2.1.2.2.2-3 FRC for eRedCap HD-FDD w/ baseband bandwidth reduction

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | Test 1/3 | Test 2/4 |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Number of allocated resource blocks | PRBs | 25 | 25 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 8 | 8 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames | Slots | 15 | 15 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 13 |  |  |  |
| Modulation |  | QPSK | 16QAM |  |  |  |
| Target Coding Rate |  | 0.30 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | 12 | 12 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 1256 | 3968 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 2024 | 6272 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 16 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | Bits | 16 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | CBs | 1 | 1 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 6300 | 12600 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} | Bits | 4200 | 8400 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,9,12,…,19} | Bits | 6600 | 13200 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 1.364 | 4.243 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

SAN side

Simulation results

Here we provide the updated simulation results based on the simulation assumption in WF[1].

Table 2.2.1-1: Simulation results for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 1 | 0 | -6.9 |
|  |  |  |  |  |  |  | 1 | -6.9 |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 1 | 0 | -9.5 |
|  |  |  |  |  |  |  | 1 | -9.5 |

Table 2.2.1-2: Simulation results for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 1 | 0 | -6.9 |
|  |  |  |  |  |  |  | 1 | -6.9 |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 1 | 0 | -9.5 |
|  |  |  |  |  |  |  | 1 | -9.5 |

Table 2.2.1-3: Simulation results for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 1 | 0 | -6.9 |
|  |  |  |  |  |  |  | 1 | -6.9 |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 1 | 0 | -9.6 |
|  |  |  |  |  |  |  | 1 | -9.6 |

Table 2.2.1-4: Simulation results for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 1 | 0 | -6.9 |
|  |  |  |  |  |  |  | 1 | -6.9 |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70% | TBD | 1 | 0 | -9.6 |
|  |  |  |  |  |  |  | 1 | -9.6 |

Proposals

In this contribution, we share our view on NR NTN Phase 3 demodulation performance requirements. Our observations and proposals are:

UE side

- Agree Option 1. i.e. define demodulation performance requirements for Redcap/eRedCap UEs with NTN channel model.

Reference

R4-2515038, Way Forward for [116bis][311] Rel-19 Demodulation_Part1, RAN4#116bis, Ericsson