3GPP TSG-RAN WG4 Meeting #117	R4-2521386

Dallas, USA, Nov 17 – 21, 2025

Agenda Item:	7.9.2

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects and workplan for AI-CSI compression

Document for:	Discussion and Decision

# Introduction

According to new WI on AI/ML for NR air interface [1], the core part work of interoperability and RRM requirement in RAN4 includes:

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability o The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we discuss general aspects and workplan for AI-CSI compression.

# Discussion on general aspects and work plan

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

According to the new WID, for inter-vendor training collaboration, there are two directions - namely Direction C and Direction A, in the scope. In Direction C, a fully defined/specified reference model is adopted as the approach for inter-vendor training collaboration. Given that RAN4 achieved significant progress in the feasibility study of aligning reference models among companies, RAN4 becomes the leading group for specifying reference models in R20.

Unlike RAN4 legacy work on defining requirements based on the assumptions specified in RAN1 and RAN2 specifications – the reference models to be developed by RAN4 in R20 would also serve as means to ensure interoperability between UE and NW in real-network deployments. To elaborate further, if a UE or NW implements these specified models, the other side could derive models compatible with them, thereby enabling.

In RAN4 legacy, requirements are derived for minimum requirements, while for real-world deployments, advanced methods are usually employed for better performance. Thus, there is a gap between the motivations for deriving models solely to define test cases and those for deriving models to support implementation in real-world deployments.

When defining minimum requirements, AI models with lower but reasonable complexity could be considered. In contrast, for models to be implemented in real networks to support inter-vendor training collaboration, AI models are intended to achieve the highest possible performance.

In R19, it was observed that it is feasible to derive a common decoder with sufficiently high complexity that is compatible with different encoders (varying in complexity and backbone) (added parentheses to streamline modifier placement). Therefore, it is possible to define a test decoder that is compatible both with the AI models used for minimum requirement definition and with the AI models for inter-vendor training collaboration in real networks.

Proposal 1: RAN4 defines a test decoder that is compatible with encoders with different complexities/backbones.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: RAN4 defines a test decoder which is compatible with encoders with different complexities/backbones.

# References

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, 3GPP RAN #108, 2025.