3GPP TSG-RAN WG4 Meeting #117	R4-2521390

Dallas, US, 17th – 21st November 2025

Agenda item:	8.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Views on 6G waveform

Document for:	Discussion

- Introduction

In RAN4#116bis, we started to discuss the 6G system parameter. At that meeting, the approach to the discussion was agreed upon as outlined in the WF [1], and some items are dependent on the progress of RAN1. This contribution describes the views on the waveform that should be considered by RAN4 based on the progress made by RAN1 at the previous meeting.

- Discussion

According to R4-2514650[1], we agreed the direction of the 6G system parameters as below.

- Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement PA model - Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

The study of this item depends on the progress in RAN1. In the previous meeting, RAN1 reached the following agreements regarding waveform.

Agreement For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER A realistic PA model should be used When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. FFS: Other evaluation metrics Note: Companies to report how to calculate the Tx power gain, modulation and coding Agreement Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

RAN1 agreed on the definition of Net Gain in the previous meeting. However, aligning the interpretation of each component—Tx power gain relative to the reference and SNR degradation relative to the reference—is also important for RAN4 in the context of evaluating RF performance metrics. For example, Tx power gain relative to the reference can quantify the improvement in transmit power due to techniques such as PAPR reduction, although the contributing factors to transmit power gain should be further discussed. Similarly, SNR degradation relative to the reference can quantify the loss in receive quality under specific channel conditions, which need to be considered in the evaluation. RAN4 considers it necessary to clarify these constituent parameters.

Observation 1: The constituent parameters of Net Gain, which is a candidate evaluation metric—namely Tx power gain relative to the reference and SNR degradation relative to the reference—still require further discussion within RAN4.

Proposal 1: It is necessary for RAN4 to discuss how the parameters Tx power gain relative to the reference and SNR degradation relative to the reference, which constitute the candidate evaluation metric Net Gain, should be defined.

RAN1 has agreed to evaluate DFT-s-OFDM for uplink with more than 1 layer. It is considered that the number of layers may impact RF performance. For example, as the transmit signal becomes more complex, phase and amplitude errors across layers may accumulate, potentially degrading EVM. Additionally, increasing the number of layers requires more transmit antennas, power amplifiers, and digital processing, which may lead to increased power consumption. RAN4 should consider the RF performance impact associated with the number of layers and share the findings with RAN1 for alignment.

Observation 2: Increasing the number of layers may degrade EVM due to signal complexity and raise power consumption due to additional RF and baseband resources.

Proposal 2: RAN4 needs to discuss the RF impact of the number of layers in case of DFT-s-OFDM for UL and share the findings with RAN1.

- Conclusion

In this contribution, we offer our RAN4 views on 6G waveform as follows.

Observation 1: The constituent parameters of Net Gain, which is a candidate evaluation metric—namely Tx power gain relative to the reference and SNR degradation relative to the reference—still require further discussion within RAN4.

Proposal 1: It is necessary for RAN4 to discuss how the parameters Tx power gain relative to the reference and SNR degradation relative to the reference, which constitute the candidate evaluation metric Net Gain, should be defined.

Observation 2: Increasing the number of layers may degrade EVM due to signal complexity and raise power consumption due to additional RF and baseband resources.

Proposal 2: RAN4 needs to discuss the RF impact of the number of layers in case of DFT-s-OFDM for UL and share the findings with RAN1.

References

[1] 3GPP, R4-2514650, “WF on 6GR system parameter”, Feature lead (Huawei, HiSilicon).