3GPP TSG-RAN WG4 Meeting #117	 R4-2521401

Dallas, USA, 17th – 21st November, 2025

Source:	Nokia

Title:	Support of 3MHz for (e)RedCap devices

Agenda item:	7.4.3.1

Document for:	Discussion

Introduction

During the last RAN4 WG meeting, following agreements were made, reproduced below for ease of readability [1]. In this document, we present our views on the remaining issues required for completion of this work.

# Workplan #### Issue 1-1: Work plan for UE RF support of less than 5MHz channel bandwidth for (e)Redcap It is agreed that the UE RF part of workplan in R4-2513987. # Spec impact #### Issue 2-1: Spec impact on 5.3I Channel bandwidth It is agreed to removing the sentence “3MHz channel bandwidth is not applicable for (e)RedCap UE in the current release.” #### Issue 2-2: Spec impact on 5.4I Channel arrangement It is agreed to no spec impact on 5.4I Channel arrangement #### Issue 2-3: Spec impact on 6.2I Transmitter power It is agreed that PC2 will not be specified for 3MHz (e)RedCap in this WI. #### Issue 2-4: Spec impact on 7.3I Reference sensitivity for (e)RedCap It is agreed to define the REFSENS requirement for (e)Redcap with 2Rx FD-FDD, 1Rx FD-FDD, 2Rx HD-FDD and 2Rx HD-FDD #### Issue 2-5: Spec impact on FRC It is agreed that no new FRCs is needed for 3MHz (e)RedCap # REFSENS requirements #### Issue 3-1: REFSENS for FD-FDD with 2Rx for Redcap It is agreed to Re-use the 2 RX FD-FDD REFNSENS for non-redcap UE for Redcap Operation band; 3MHz REFSENS with FD-FDD with 2Rx. n26; -99.7. n28; -100.2. n85; -99.2. n100; -102.2. n106; -99.2. #### Issue 3-2: REFSENS for HD-FDD with 2Rx for Redcap It is agreed that for 2Rx HD-FDD configuration, the 3MHz channel bandwidth REFSENS requirements can be derived based on the following formula: 2Rx HD-FDD REFSENS_5MHz – 10log10(25/15) dB = 2Rx HD-FDD REFSENS_5MHz – 2.2 dB Based on the approach above, it is agreed to adopt the following REFSENS for HD-FDD with 2Rx for Redcap as below: Operating band; 3MHz REFSENS with HD-FDD with 2Rx. n26; -100.5. n28; -101.5. n85; -100. n100; -102.2. n106; -100*. *Note: for n106, due to the missing of 5MHz REFSENS, it is agreed to adopt 0.8 dB HD-FDD tightening factor compared to 3MHz FD-FDD REFSENS in issue 3-1. Issue 3-3: REFSENS relaxation for 1Rx in FD-FDD and HD-FDD for Redcap It is agreed to use the same sensitivity allowance value as 5MHz CBW, e.g., 2.5 dB, relaxation with respect to 2 Rx FD-FDD and 2Rx HD-FDD REFNSENS Operating band; Channel bandwidth (MHz); ΔR1R (dB). TDD band; 5, 10, 15, 20; 2.5. FDD band; 3,5; 2.5. FDD band; 10, 15, 20; 3.0. Issue 3-4: REFSENS value for eRedcap It is agreed to use the same value of REFSENS requirements as Redcap for eRedcap. Issue 3-5: REFSENS RB allocation configuration for eRedcap FFS how to capture RB allocation configuration in spec for eRedcap # Release independent and UE capability Issue 4-1: Whether 3MHz UE CBW should be optional or mandatory in Rel-20 for (e)Redcap FFS whether 3MHz should be optional or mandatory in Rel-20 for (e)Redcap Issue 4-2: Release independent of 3MHz UE CBW for (e)Redcap It is agreed 3MHz UE CBW is release independent from Rel-18 for (e)RedCap UE. Issue 4-3: UE capability FFS in next meeting what UE capabilities of (e)Redcap need to be updated to support 3MHz and sent LS to RAN2.

Discussion

Optionality of 3 MHz

3 MHz channel bandwidth was introduced back in Release 18 and has been optional even in Release 19. Given that two releases have passed since its introduction, in our opinion 3 MHz should be mandatory from Release 20 and onwards.,  Further, n106, which one of the exemplary band for this work only supports 3 MHz channel bandwidth, it seems apt that 3 MHz becomes mandatory.

3 MHz channel bandwidth shall be made mandatory from Release 20.

Release independence

RedCap was introduced and is release independent from Release 17. Similarly, eRedCap was introduced and is release independent from Release 18. Further, 3 MHz is release independent from Release 18 only. Thus, (e)RedCap with 3 MHz can be release independent from Release 18 only.

3 MHz for (e)RedCap shall be release independent from Release 18.

LS to RAN2

In the previous meetings, some companies have identified some 3 MHz capabilities which specifically preclude (e)RedCap devices from 3 MHz operation. However, the list is not complete and since the specifications in question are under RAN2 control, they should have the discussion to identify and implement what is needed for implementing this feature. To facilitate this, we have drafted a LS to be sent to RAN2.

Agree to send the LS presented in R4-2521402.

Conclusion

The following Observations and Proposals were made:

Proposal 1: 3 MHz channel bandwidth shall be made mandatory from Release 20.

Proposal 2: 3 MHz for (e)RedCap shall be release independent from Release 18.

Proposal 3: Agree to send the LS presented in R4-2521402.

- References

- R4-2515101, WF for [116bis][323] NR_less_than_5MHz_RedCap_UERF WID, Sony, RAN4#116bis, Prague, CZ, Oct 135 – Oct 17, 2025.