3GPP TSG-RAN WG4 Meeting #117	R4-2521404

Dallas, USA, November 17 – 21, 2025

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Agenda item:	4.14.1

Document for:	Discussion

Introduction

During the maintenance phase for Rel-19 NR demodulation performance Phase 5: BS Demodulation MMSE-IRC, RAN4 still allows companies to align the simulation results and discuss the results as stated in the way forward of RAN4#116bis discussions in [1] as below:

Alignment on simulation results for BS MMSE-IRC requirements - Way forward - Keep the way forward from RAN4#116 and continue the discussions in RAN4#117 - Make decision on final SNR requirements in RAN4#117

As stated above, RAN4 will keep the way forward from RAN4#116 in [2], which is:

- Wayfoward: - For this meeting, use the latest SNR values for requirement with all [ ] removed. - Companies can further align simulation results in the maintenance part. - Further discuss the solution in the maintenance part if the results are not well-aligned.

In addition to the above-mentioned way forwards, [2] captured also early observations from few companies on the simulation results collected in RAN4#116 in [3].

This contribution provides our brief observations on the simulation results collected in RAN4#116 for consideration in the discussions during RAN4#117, if any updated simulation results from companies are made available in RAN4#117.

Discussion

The collected simulation results in [3] shows some cases of the ideal simulation results with span more than 2 dB. Specifically, for the 8Rx case, from 16 cases, all of them have span more than 2 dB, ranging from the minimum of 2.49 dB to the maximum of 6.42 dB. One of the observations documented in [2] is as follows:

Regarding companies may still use different MMSE-IRC implementation, our simulation results shared in the last meeting show that: for 2Rx and 4Rx, there is no performance difference if companies use different MMSE-IRC implementation. Therefore at least for 2Rx and 4Rx, we do not think that will be the reason if the results are not aligned.

Referring to such an observation from the observer’s own simulation results, different MMSE-IRC implementations are expected to show performance differences in 8Rx case. This is somehow reflected by the collected simulation results in [3] for 8Rx, in which all 16 cases have span more than 2 dB.

In Rel-19 BS MMSE IRC, the Interference-to-Noise-Ratio (INR) is derived from the Dominant-Interference-Proportion (DIP) from LTE, which was a result from system-level simulations from a group of contributing companies as documented in TR 36.884 [4]. Interestingly, if one would consider only the results of that group of companies which have contributed to TR 36.884 [4], the results for 8Rx are quite aligned with span less than 2 dB for all the 16 cases.

Regardless of whether there are companies updating their simulation results in RAN4#117, as stated in the WF [1], RAN4 should make decision on final SNR requirements in RAN4#117. In case there are any updates, a discussion during RAN4#117 is needed for alignment and for removing the outliers if the span of the updated ideal results is still more than 2 dB. The updated SNR requirements can then be integrated to the corresponding CRs submitted in RAN4#117 for agreements, namely [5][6][7].

RAN4 to conclude on the final SNR requirements and to capture them in CRs submitted in RAN4#117, if any changes on the final SNR requirements are made.

Conclusion

In this contribution, we provide our views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Proposal 1: RAN4 to conclude on the final SNR requirements and to capture them in CRs submitted in RAN4#117, if any changes on the final SNR requirements are made.

- References

- R4-2514823, WF on NR_demod_Ph6_Ph5, 3GPP RAN4#116, China Telecom, Prague, Czech Republic, October 13-17, 2025

- R4-2512606, Way Forward for [116][321] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#116, China Telecom, Bengaluru, India, August 25-29, 2025

- R4-2509405, Simulation results summary for BS requirement with MMSE_IRC receiver_v5_Samsung, 3GPP RAN4#116, Samsung, Bengaluru, India, August 25-29, 2025

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, V13.1.0, 09-2016

- R4-2521405, CR on 38.104 for Rel-19 BS MMSE-IRC, 3GPP RAN4#117, Nokia, Dallas, USA, November 17 – 21, 2025

- R4-2521406, CR on 38.141-1 for Rel-19 BS MMSE-IRC3GPP RAN4#117, Nokia, Dallas, USA, November 17 – 21, 2025

- R4-2521407, CR on 38.141-2 for Rel-19 BS MMSE-IRC3GPP RAN4#117, Nokia, Dallas, USA, November 17 – 21, 2025