3GPP TSG RAN WG4 Meeting #117		R4-2521418

Dallas, TX, U.S.A, 17th – 21st November 2025

Source: 		Rohde & Schwarz

Title: 			Discussion on channel model for AI/ML based beam management

Agenda Item: 	6.11.4

Document for: 	Discussion

# Introduction

In the last RAN4 meeting (#116bis) AI/ML BM testability aspects were treated and the WF in [18] included the following agreements:

Issue 2-8: Test system setup

Agreement:

A multi-AoA IFF based test setup with 4 active cross-polarized probes placed in the single plane (xz plane). The relative angular separation between neighboring probes is 30º, 60º, and 60º, respectively.

Issue 2-9: Simplified Channel models

Agreement:

RAN4 will further discuss the simplified Uma and UMi channel model by taking the following aspects into consideration

- Commercial test system limitation, including the number of probes and the separation

- Model simplification should not result in inference performance degradation beyond the margin when the training is done based on the original models.

  - The margin can be discussed and decided further.

- Other aspects are not precluded.

In this document we focus on the definition of a candidate simplified channel as suggested above in Issue 2-9, to fit to the test system agreed above in Issue 2-8 (simultaneous cross polarized 4AoA based on CATR, xz plane, relative angular separation respectively 30º, 60º, and 60º). Basically, we summarize our proposal explained in detail in our previous contributions [13 to 16] and compare the approach followed, with an alternative one presented in [19].

# Discussion

For defining a simplified CDL based channel model which is supported by the multi-AoA IFF based test setup, we should focus on matching the OTA characteristics between the simplified channel and the setup.

Two approaches were captured in the WF in [18] and are further analyzed herein using the UMi CDL-C channel model Table 7.2.2-3 from TR 38.827 [3] as a reference (which is the same as the one captured in Annex D.1 of TS 38.151 [4]) depicted below:

Table 7.2.2-3: Channel model parameters for UMi CDL-C at 28 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.25 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 3 | 13.314 | -3.4684 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 4 | 13.974 | -5.2294 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -2.5215 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 7 | 38.688 | -2.2185 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 8 | 39.36 | -3.9794 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 9 | 39.504 | -7.4215 | 17.3861 | 84.3647 | 100.4513 | 69.2454 |
| 10 | 47.61 | -7.1215 | -37.5865 | 92.0623 | 98.5616 | 66.7349 |
| 11 | 49.278 | -10.7215 | 20.2226 | -97.7585 | 100.6231 | 72.0348 |
| 12 | 56.016 | -11.1215 | -50.6106 | 78.4702 | 98.218 | 64.4337 |
| 13 | 73.71 | -5.1215 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -6.8215 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| 15 | 130.224 | -8.7215 | -43.1797 | 102.4645 | 98.1225 | 64.7824 |
| 16 | 162.63 | -13.2215 | 29.2116 | 67.2359 | 100.2604 | 92.467 |
| 17 | 255.534 | -13.9215 | 27.8133 | 34.5731 | 98.4852 | 65.6889 |
| 18 | 276.018 | -13.9215 | 23.6584 | 48.5813 | 98.1416 | 68.7572 |
| 19 | 329.412 | -15.8215 | -52.5282 | 36.4455 | 97.9698 | 59.1339 |
| 20 | 336.462 | -17.1215 | 25.0168 | 52.6729 | 100.7376 | 65.3402 |
| 21 | 378.39 | -16.0215 | 25.4562 | 49.8296 | 98.1225 | 58.4365 |
| 22 | 398.244 | -15.7215 | 30.7697 | 46.4316 | 98.1034 | 65.2705 |
| 23 | 422.562 | -21.6215 | 35.9234 | 30.759 | 100.4513 | 62.6903 |
| 24 | 519.138 | -22.8215 | -61.2775 | 69.2469 | 100.9476 | 61.993 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

## Simplification based on cluster illumination

During the last RAN4 meeting (#116bis) an alternate simplification method was presented in R4-2514404 [19], based on the concept that the cluster illumination per BS beam in its codebook would help reduce the AoA/ZoA angular spread and thus accommodate a sparse probe layout like the one agreed in the WF [18].

This first assumption supporting this concept in [19] is that “the 24 clusters are distributed across the entire channel environment around the gNB”.Refering to the above Table 7.2.2-3 from TR 38.827 [3], this doesn’t seem true since the AoDs spread over approximately the same range as the BS codebook assumptions (i.e. maximum ±60º in azimuth).

Observation 1: (Simplification based on illumination) The AoD spread in the channel model has approximately the same range as the BS codebook assumptions (i.e. maximum ±60º in azimuth), thus all clusters are illuminated by Set A.

The second assumption in [19] is that each BS beam has a very limited HPBW (i.e. 11.25º), and thus the set of illuminated clusters is very limited. As per our understanding this is only true for the beam(s) pointing towards the main cluster as can be seen in the Spherical Power Spectrum (SPS) plots presented last year in [9] and shown in Figure 1 below. Figure 2 highlights the selected Set B beams among 128 beams in the BS codebook.

Figure 1 – SPS at the UE end for Set B with 8 BS beams for case 1 in Figure 4 (Reference [9])

Figure 2 – Set B with 8 BS beams for case 1 (red highlight) out of 128 Set A beams, formed by 16 beams in azimuth [-60°,60] and 8 beams in elevation [-25°,25°] (Reference [9])

Referring to Figure 1 above, the plot for Beam #87 which is the beam pointing towards the strongest cluster, as well as Beam #55 (same azimuth, but pointing to a higher elevation), are the only ones with limited illuminated AoA spread with about ~100º. For any other BS beams, the sidelobes have also a quite relevant impact on the illuminated clusters and the overall AoA spread cannot be simulated with a sparse probe layout. This effect remains even if reducing the number of the BS beams in Set A (e.g. 32 beams) as long as the ±60º coverage in azimuth is considered for the BS codebook.

Observation 2: (Simplification based on illumination) The complete AoA spread in the channel model after BS illumination cannot be simulated with a sparse probe layout.

In the same way, referring to Figure 1 above, even if a power truncation (e.g. > -10dB) is applied after BS illumination, the overall spread becomes impractical.

Observation 3: (Simplification based on illumination) Removing the weak clusters after BS illumination (e.g. > -10dB) does not reduce the overall AoA spread.

Another option suggested in [19] is that the test is performed in sequential steps where the UE is rotated so that different AoA/ZoA regions are presented to the UE during the inference period. From TE perspective this could be considered for further analysis, but overall it sounds unrealistic to define a UE Test Function to enable such sequential testing over a long time (e.g. in hundreds of seconds) for a feature that is supposed to react quickly to the environment.

Observation 4: (Simplification based on illumination) It is unrealistic and not aligned with the test scope to perform the AI-ML BM test case in sequential manner by means of a UE Test Function.

## Simplification based on weak cluster elimination

As already presented in our previous contributions in [13 – 16], using the UMi CDL-C channel model Table 7.2.2-3 from TR 38.827 [3] as a reference, one option to implement further simplifications is as follows:

- The clusters that have the same AoAs can be merged. To do so, the delay for the first tap is kept and the power of the three taps with the same AoD/AoAs is combined, resulting in +3dB for clusters #2 and #6 in the example for UMi CDL-C.

- The weak clusters falling outside the UE’s spherical coverage (i.e. <-10dB) are expected to have a limited impact on the beam management and can be removed, following Observation 8 to Observation 10 in [17], in order to reduce the number of clusters.

Implementing #1 results in a power scaling of all clusters. This is done by adjusting the power for the strongest cluster (e.g. #6 for UMi CDL-C) to 0dB and reducing the remaining clusters by the difference to the strongest cluster. With this scaling, there are more clusters falling below -10dB from the strongest cluster and then they are also removed. The resulting channel model parameters for UMi CDL-C are presented in Table 1. The same approach can be applied to channel models other than UMi CDL-C.

Table 1 – Channel model parameters for UMi CDL-C at 28 GHz 
after merging clusters with similar AoAs and removing weak clusters

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

The maximum angular spread required to emulate the down selected clusters in Table 1 is close to 150°, which can be covered by the probe layout of the proposed multi-AoA IFF based setup agreed in the WF in [18].

Observation 5: (Simplification based on weak clusters) CDL channel model can be simplified by merging the clusters that have the same AoA and removing the weak clusters falling outside the UE’s spherical coverage (i.e. < -10dB) resulting in the simplified channel model in Table 1.

Observation 6: (Simplification based on weak clusters) The required total AoA spread of 150° of the simplified channel in Table 1 can be emulated with the multi-AoA IFF based probe layout.

Looking at further channel model simplifications to fit the multi-AoA IFF based system, there is one clear change to be implemented: the elevation spread (i.e. ZoA) and the intra-cluster elevation angle spread (i.e. CZSA) must be flattened, to fit the probe layout placed in a single plane. Further discussion could determine whether a flat 90º (i.e. as done for FR1 MIMO OTA as described in TR 38.827 [3]) should be used, or it could be possible to select the weighted average of the ZoA for the remaining clusters (i.e. 74.51º). In Table 2, the weighted average is used for further discussion. It can be decided later whether this single elevation is tested in the chamber by selecting the appropriate DUT testing conditions/orientations.

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Related to how this channel model can be implemented in the candidate test system regarding signal to probe allocation, we see the 3 following possibilities, which are also illustrated in Figure 3:

- Option A: Using the simplified channel model in Table 2, the strongest cluster (i.e. cluster #6) is aligned to one of the available probes (i.e. 30º, Figure 3). In this approach, like done for MIMO OTA, the test system calculates the weights to simulate the AoAs in between the probes (i.e. reflectors). With this option, there is no need to further modify the channel model table. On the other hand, the main drawback is that there are several clusters falling outside the probe spread.

- Option B: Using the simplified channel model in Table 2, the mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes (i.e. 75º, Figure 3). With this option, there is no need to further modify the channel model table, and all clusters (nearly) fall into the probe spread.  The main drawback is that the strongest cluster fall between probes separated 60º apart.

- Option C: Using Option B as starting point, AoAs in the channel model parameter table are modified to align with the closest probe location. Here, every cluster will be aligned with one probe location and allows the intra-cluster angle spread (i.e. CASA) to be set to 0º for a very much simplified channel model. This results in the simplified channel model in Table 3 (probe alignment in Figure 3).

Table 3 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 0 | 0.5726 | 0 | 7 |  |

Observation 7: (Simplification based on weak clusters) The simplified channel model parameters for UMi CDL-C can be implemented in the multi-AoA IFF based test system, with 3 possible options (Figure 3): Option A: with flattened elevation (Table 2) and strongest cluster aligned to one probe; Option B:  with flattened elevation (Table 2) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 3).

In all 3 cases, even though intra-cluster angle spread and/or intra-cluster elevation angle spread must be reduced, the doppler per cluster will be emulated by the test system. Also it has to be noted that all probes in the system are used to simultaneously simulate all the clusters (with some weight), and therefore all probes contribute to simulate each BS beam.

Observation 8: (Simplification based on weak clusters) The simplified channel model parameters for UMi CDL-C do not affect the doppler per cluster, i.e. the latter is still emulated by the multi-AoA IFF based test system.

Observation 9: (Simplification based on weak clusters) In the multi-AoA IFF based test system, all the probes are used to simultaneously simulate all the clusters (with some weight), and therefore all probes contribute to simulate each BS beam.

Proposal 1: (Simplification based on weak clusters) Consider the channel models described in Table 2 (Option A/B) and Table 3 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM.

Please note that in the resulting alignment of the clusters with respect to the example probe layout presented in Figure 3 – , the channel model cluster AoAs have been transformed to the chamber probe coordinates, and that the strongest cluster (i.e. cluster #6) has been highlighted with a different marker (▲).

Figure 3 – Simplified UMi CDL-C at 28 GHz alignment to the multi-AoA IFF based system

As presented in our previous contributions in [8-16], the degradation introduced in the inference performance by these simplified channel models with 6 clusters compared to the complete channel model with 24 clusters is negligible.

It must be noted that the system level simulations (i.e. BS incl. Codebook, Channel Model, UE incl. Codebook and AIML Perf) results presented in our previous contributions [8-16] include the BS beam illumination towards the 24 or 6 clusters, for the complete or simplified channel models respectively.

Observation 10: (Simplification based on weak clusters) Based on preliminary simulations with 8 Set B & 32 Set A beams, there is only a negligible inference performance degradation (deviation of max ~1%) for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for the multi-AoA IFF based test system (Options A/B/C - 6 clusters).

Nevertheless, as mentioned before, for finalizing the evaluation of the channel for the AI/ML testing, further comprehensive simulations are required by UE/chipset vendors, in order to include also real UE performance aspects.

Proposal 2: (Channel Evaluation) RAN4 to start the evaluation of CDL-C simplified channel models with 6 Clusters in Table 2 and Table 3 through comprehensive AI/ML BM simulations, by comparing the prediction performance between simulations with the full channel and the simplified channel, when using the same training model and performance metric.

# Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: (Simplification based on illumination) The AoD spread in the channel model has approximately the same range as the BS codebook assumptions (i.e. maximum ±60º in azimuth), thus all clusters are illuminated by Set A.

Observation 2: (Simplification based on illumination) The complete AoA spread in the channel model after BS illumination cannot be simulated with a sparse probe layout.

Observation 3: (Simplification based on illumination) Removing the weak clusters after BS illumination (e.g. > -10dB) does not reduce the overall AoA spread.

Observation 4: (Simplification based on illumination) It is unrealistic and not aligned with the test scope to perform the AI-ML BM test case in sequential manner by means of a UE Test Function.

Observation 5: (Simplification based on weak clusters) CDL channel model can be simplified by merging the clusters that have the same AoA and removing the weak clusters falling outside the UE’s spherical coverage (i.e. < -10dB) resulting in the simplified channel model in Table 1.

Observation 6: (Simplification based on weak clusters) The required total AoA spread of 150° of the simplified channel in Table 1 can be emulated with the multi-AoA IFF based probe layout.

Observation 7: (Simplification based on weak clusters) The simplified channel model parameters for UMi CDL-C can be implemented in the multi-AoA IFF based test system, with 3 possible options (Figure 3): Option A: with flattened elevation (Table 2) and strongest cluster aligned to one probe; Option B:  with flattened elevation (Table 2) and mid-point of the total angular spread of the clusters is aligned to the mid-point of the spread of the probes; Option C: with flattened elevation, AoAs aligned to probe layout and no intra-cluster angle spread (Table 3).

Observation 8: (Simplification based on weak clusters) The simplified channel model parameters for UMi CDL-C do not affect the doppler per cluster, i.e. the latter is still emulated by the multi-AoA IFF based test system.

Observation 9: (Simplification based on weak clusters) In the multi-AoA IFF based test system, all the probes are used to simultaneously simulate all the clusters (with some weight), and therefore all probes contribute to simulate each BS beam.

Observation 10: (Simplification based on weak clusters) Based on preliminary simulations with 8 Set B & 32 Set A beams, there is only a negligible inference performance degradation (deviation of max ~1%) for Top 3 & Top 1 beams between the cases with full CDL channel model (Reference - 24 clusters) and simplified CDL channel model suitable for the multi-AoA IFF based test system (Options A/B/C - 6 clusters).

Proposal 1: (Simplification based on weak clusters) Consider the channel models described in Table 2 (Option A/B) and Table 3 (Option C) as candidates for CDL-based simplified channel models for multi-AoA testing of AI/ML BM.

Table 2 – Channel model parameters for UMi CDL-C at 28 GHz with flat ZoA

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -134.4434 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 129.1633 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -152.8206 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | 164.1145 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 93.1719 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -112.0441 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 0 | 7 |  |

Table 3 – Channel model parameters for UMi CDL-C at 28 GHz
 with AoA aligned to probe layout and no intra-cluster angle spread

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4318 | -30.4353 | -114.436 | 98.9242 | 74.51134 |
| 2 | 12.594 | -1.2500 | -20.9269 | 125.5639 | 99.1915 | 74.51134 |
| 5 | 13.056 | -5.5318 | -28.0782 | -174.436 | 99.5732 | 74.51134 |
| 6 | 38.196 | 0.0000 | -11.6982 | -174.436 | 99.306 | 74.51134 |
| 13 | 73.71 | -8.1318 | -33.911 | 95.5639 | 100.165 | 74.51134 |
| 14 | 78.498 | -9.8318 | -37.5066 | -114.436 | 100.2604 | 74.51134 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 0 | 0.5726 | 0 | 7 |  |

Proposal 2: (Channel Evaluation) RAN4 to start the evaluation of CDL-C simplified channel models with 6 Clusters in Table 2 and Table 3 through comprehensive AI/ML BM simulations, by comparing the prediction performance between simulations with the full channel and the simplified channel, when using the same training model and performance metric.

# References

- 3GPP TR 38.843 V18.0.0, “Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”

- 3GPP TR 38.901 V19.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”

- 3GPP TR 38.827 V16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”

- 3GPP TS 38.151 V18.4.0, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”

- 3GPP TS 38.101-4 V18.8.0, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”

- 3GPP TS 38.133 V19.1.0, “NR; Requirements for support of radio resource management”

- 3GPP TS 38.508-1 V19.0.0, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- R4-2409739, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2411587, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112, August 2024

- R4-2416391, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112bis, October 2024

- R4-2418941, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #113, November 2024

- R4-2501726, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2504268, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #114bis, April 2025

- R4-2506981, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #115, May 2025

- R4-2511597, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #116, August 2025

- R4-2514248, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #116bis, October 2025

- R4-2500403, “Testability for AI/ML beam management”, Qualcomm, 3GPP TSG RAN WG4 Meeting #114, February 2025

- R4-2514631, “WF on [116bis][112] R19 AI for air interface”, Qualcomm, 3GPP TSG RAN WG4 Meeting #116bis, October 2025

- R4-2514404, “RRM performance requirements for Beam Management and Positioning accuracy”, Nokia, 3GPP TSG RAN WG4 Meeting #116bis, October 2025