3GPP TSG RAN WG4 Meeting #117	R4-2521443

Dallas, USA, Nov. 17-21, 2025

Agenda item:			7.12.2

Source:	vivo

Title:	General aspects on AI/ML for mobility

Document for:	Discussion

- Introduction

In RAN#109 meeting, the latest WID [1] was approved. The description of objectives is copied as:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

The RAN4 task specifies the highlighted sections above, primarily covering both RRM measurement prediction and measurement event prediction for the UE-sided model.

In the last meeting, RAN4 conducted a preliminary discussion on issues related to scenarios and sub-use cases, as well as simulation scenarios and assumptions. The outcome of the discussion were captured in WF [2]. In this contribution, we further provide our views on issues regarding General aspects for AI mobility based on the outcome reached in the last meeting[2].

- Discussion

  - Scenarios and sub-use cases

In the last meeting, RAN4 discussed the scenarios and sub-use cases that need to be considered and evaluated, and the conclusions of the discussion are as follows.

Issue 1-2-2: Scenarios and sub-use cases Online agreements on Tuesday: Agreement: RAN4 to discuss the potential RRM requirements for cell-level based prediction for the following scenarios: Evaluation scenario; Methodology. FR1 to FR1 intra-frequency temporal domain case A; intra-cell. FR1 to FR1 intra-frequency temporal domain case B; Intra-cell. FR1 to FR1 inter-frequency (frequency domain) (co-located case); Inter-cell. FR2 to FR2 intra-frequency temporal domain case A; Intra-cell. FR2 to FR2 intra-frequency temporal domain case B; Intra-cell. FFS whether to discuss the requirements for the following scenario: Evaluation scenario; Methodology. FR2 to FR2 inter-frequency (frequency domain) (co-located case); Inter-cell.

Regarding the FFS part on the scenario of FR2 to FR2 inter-frequency (frequency domain), we tend to evaluate it as a low-priority scenario. Firstly, if the frequency of the current measurement cell and the frequency of the predicted cell are in the same FR2 band, the predicted cell can directly use the measurement values of the measurement cell without measurement. Secondly, for inter-frequency prediction in FR2 deployment, it may be more difficult compared to FR1, and more factors need to be considered. Considering the workload, it is recommended to deprioritize this scenario.

Proposal 1: From the perspective of evaluation and requirement definition, RAN4 to deprioritize FR2 to FR2 inter-frequency (frequency domain) (co-located case)

Way forward: - Consider the following frequency pairs as baseline for frequency domain prediction evaluation. - Candidate option 1: UE use the measurement on 2GHz to predict 4GHz. - Candidate option 2: UE use the measurement on 1.8GHz to predict 3.5GHz. - Candidate option 3: UE use the measurement on 1.8GHz to predict 2.1GHz. - Other options are not precluded. - Further discuss how to define requirements based on the available simulation results.

Regarding the frequency pairs to be evaluated for frequency domain prediction evaluation, for Option 3, the two frequency points are very close to each other, which may not be representative from the perspective of frequency prediction (for the higher-to-lower frequency case and vice versa for the lower-to-higher frequency case). As for Option 1 and Option 2, we can consider selecting only one as the baseline for the evaluation phase, and we tend to use Option 1, which was previously evaluated by RAN2. For other frequency pairs, just like the approach to treating other evaluation factors (e.g., sub-use case), interested companies can also submit results of other frequency pairs to provide evidence for the impact of frequency pairs on prediction performance.

Proposal 2: As starting point, evaluate frequency pairs 2GHz-4GHz as baseline for frequency domain prediction.

- Interested companies can also provide results for other frequency pairs.

  - Simulation assumption for AI mobility

Issue 1-3-3: Simulation aspects for frequency domain prediction Further discuss based on the open issues in [1] and the following way forward: - Discuss the proposals considering: - How to model the frequency-correlation for frequency domain prediction. - Option 1: RAN4 to consider the RAN2 definition of correlation coefficient as a starting point. - Option 2: Define correlation coefficient for inter-frequency predictions as the correlation between PDPs of different frequency layers channel profiles. - Whether and how to set varied BS configurations for different frequencies.

Regarding how to model the frequency-correlation for frequency domain prediction, for Option 1, the Pearson correlation coefficient is used for correlation coefficient calculation. The problem is that each company calculates the corresponding correlation coefficient based on its own evaluation data, but there is no further calibration of the simulation among companies for this coefficient. For Option 2, it is not yet clear how to define the correlation coefficient through the PDPs of different frequency layers channel profiles, and how to calibrate the data provided by different companies through PDPs. Therefore, in order to better align the results, we suggest establishing a reference mixed dataset for evaluation to avoid inconsistent correlation coefficients obtained from different data used in different companies' evaluation. The generation of this dataset involves mixing the datasets provided by companies to serve as the reference dataset.

Proposal 3: RAN4 to establish a reference dataset formed by mixing datasets provided by companies for evaluation to model the frequency-correlation for frequency domain prediction

- Summary

Proposal 1: From the perspective of evaluation and requirement definition, RAN4 to deprioritize FR2 to FR2 inter-frequency (frequency domain) (co-located case)

Proposal 2: As starting point, evaluate frequency pairs 2GHz-4GHz as baseline for frequency domain prediction.

- Interested companies can also provide results for other frequency pairs.

Proposal 3: RAN4 to establish a reference dataset formed by mixing datasets provided by companies for evaluation to model the frequency-correlation for frequency domain prediction

- References

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, OPPO, Interdigital

- R4-2514827, WF on NR_AIML_Mob_RRM, OPPO