3GPP TSG-RAN WG4 Meeting #117		  R4-2521449
Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on 6G modulation

Source:	OPPO

Agenda item:	8.2.2

Document for:	Approval

Introduction

In RAN4 #116bis meeting, about modulation, it was agreed that [1]

- RAN4 evaluation work focus on the feasibility study and RF requirements impact

- To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1

  - Identify the main affected requirements for modulation evaluations

    - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions.

  - Align on the evaluation assumptions.

  - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation.

  - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed.

  - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1.

- Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations

  - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies.

  - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

In this contribution, we further discuss 6G modulation based on the progress and open issues in last meeting.

Discussion

RF requirements impacted by modulation

Based on existing 5G NR requirements, we can see the modulation will impact MPR, AMPR, EVM and IBE requirements. The impact of modulation on MPR, AMPR and IBE is mainly achieved through the EVM requirements. Therefore, RAN4 should evaluate EVM requirements for different modulation firstly.

The modulation evaluations need to consider EVM, MPR, AMPR and IBE requirement.

MPR, AMPR and IBE impacted by modulation are through the EVM requirements.

5G FR1 Tx EVM requirements for different modulation are defined as following Table 1:

Table 1 5G FR1 Tx EVM requirements

| Modulation scheme for PDSCH | Required EVM | Application |
| --- | --- | --- |
| QPSK | 17.5 % | For both of UE Tx and BS Tx |
| 16QAM | 12.5 % |  |
| 64QAM | 8 % |  |
| 256QAM | 3.5 % |  |
| 1024QAM | 2.5 %1 2.8 %2 | Only for BS Tx |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |  |

There are three options for 6G Tx EVM:

- Keep the same Tx EVM values with 5G

- Relax the Tx EVM requirements based on 5G values

- Tighten the Tx EVM requirements based on 5G values

It’s unnecessary to tighten the Tx EVM in 6G since current system can work well. Whether to and how to relax UE Tx EVM requirements depends on whether the BS Rx SNR can be relaxed in 6G.

For 5G existing UE Tx modulation, RAN4 should evaluate whether the BS Rx SNR can be relaxed before evaluate UE Tx EVM, MPR and AMPR.

For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

RAN4 can focus on new higher-order modulation evaluation based on uniform constellation firstly, i.e., UL 1024 QAM.

EVM budget

Considering the feasibility of higher-order modulation from UE implementation, the EVM budget need discuss and evaluate. In the transmitter chain, the Tx EVM contributors mainly include PA, transmitter, I/Q Imbalance and LO phase noise. In LTE/NR, the EVM budget also was evaluated in previous modulation discussion, especially for UL 256QAM, Table 2 given the 256QAM EVM budget reference previous discussion.

Table 2 EVM budget for 256QAM

| Non-linearity source | 256QAM |  | 1024QAM |  |
| --- | --- | --- | --- | --- |
|  | % | C/N | % | C/N |
| PA | 2.0% | 34.0 | 1.8% | 34.9 |
| Transmitter nonlinearity | 1.03% | 39.7 | 1.02% | 39.9 |
| LO Phase noise | 1.78% | 35.0 | 1.02% | 39.9 |
| IQ Imbalance | 2.0% | 34.0 | 1.02% | 39.9 |
| Transmitter total | 3.5% | 29.1 | 2.5% | 32.0 |

And for UL 1024QAM, we need evaluate what EVM the real PA can achieve firstly to consider the feasibility. The PA may need contribute at least 1.8% EVM for UL 1024QAM provided by Some PA vendor, it needs further confirm by more PA vendors.

In NR, the EVM is 2.5% for DL 1024QAM. If UL 1024 QAM still adopts 2.5% EVM, and assumed PA contributes 1.8% EVM, then other non-linearity sources only contribute 1.75% EVM totally.

If the contribution of each other non-linearity source to EVM is equal and equals to 1.02%, we can see it is hard to achieve 1.02% EVM for LO phase noise and IQ imbalance compared to 256QAM.

It is hard to achieve lower EVM for LO phase noise and IQ imbalance than the values for 256QAM.

Therefore, RAN4 need evaluate EVM budget for higher-order modulation from the perspective of UE implementation.

EVM budget for higher-order modulation need consider the UE implementation based on the non-linearity sources of PA, transmitter, I/Q imbalance and LO phase noise.

Align the PA model with other evaluation in 6G.

SNR target evaluation (Link-level simulation)

Link level simulation is targeted as mainstream way to evaluate the required SNR value to achieve a certain throughput under different EVM requirement for a given modulation and evaluate whether higher-order modulation can achieve benefit by comparing to lower-order modulation. Therefore, link level simulation is needed to evaluate higher-order modulations, i.e., 1024QAM for UE UL. RAN4 should discuss the link-level simulation assumption firstly to facilitate the modulation evaluation.

To determine a proper EVM for higher-order modulation, RAN4 need evaluate the required SNR of the BS Rx by link-level simulation.

Due to phase noise is frequency dependency, we only need consider the impact of phase noise as AWGN in FR1.

The phase noise model doesn’t need to introduce in 6G FR1 link-level simulation.

RAN1 #122bis meeting agreed that the following configurations for system-level simulations could be used for 6GR evaluation:

|  | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated BW | Follow system bandwidth per carrier frequency in TR 38.914 as 1) Around 700 MHz: Up to 60 MHz 2) Around 2GHz: Up to 200 MHz 3) Around 4GHz: Up to 300 MHz 4) Around 7GHz: Up to 400MHz 5) Around 15GHz: Up to 400MHz 6) Around 30GHz: Up to 1GHz |  |  |  |  |
| Simulation BW | Around 700 MHz: 20MHz, 60MHz |  |  |  |  |
|  | Around 2 GHz: 20MHz, 100MHz, 200MHz |  |  |  |  |
|  | Around 4 GHz: 20MHz, 100MHz, 200MHz, 300MHz |  |  |  |  |
|  | Around 7 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 15 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 30GHz: 100MHz, 400MHz, 800MHz |  |  |  |  |
|  | Note: other simulation BW could be considered. |  |  |  |  |
| Note: The layout for each scenario will be separately discussed, including the carrier frequency combination for single layer and/or two layers. |  |  |  |  |  |

RAN1 has given some configurations and scenarios for system-level simulations.

Considering the applicable scenarios of high-order modulation and the time limitation, RAN4 can consider the scenarios of Urban Macro and Indoor.

For higher-order modulation, RAN4 can focus on the scenarios of Urban Macro and Indoor using the parameters in Table 3 as starting point:

- Table 3 The frequency parameters and scenarios for modulation evaluation

|  | Urban Macro | Indoor Hotspot |
| --- | --- | --- |
| Carrier frequency | Around 700 MHz Around 2 GHz Around 7 GHz | Around 2 GHz Around 7 GHz |
| Simulation BW | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |  |

RAN4 can discuss the link-level simulation assumption firstly, based on the format in Table 4

Table 4 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700 MHz, 2 GHz,7GHz |
| CBW/SCS | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, 5Hz Doppler frequency TDL-D 10ns delay spread, 5Hz Doppler frequency Static (AWGN) |
| MCS | Based on 5G MCS for 1024QAM PDSCH in Table 5.1.3.1-4 of TS 38.214: CP-OFDM: MCS 23, 24 other MCSs are not precluded |
| Symbol type | CP-OFDM |
| HARQ | 4, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| txEVM + rxEVM | txEVM = rxEVM: 2%+2%, 2.5%+2.5%, 3%+3%, 3.5%+3.5% txEVM > rxEVM: 3%+2.5%, 3.5%+2.5% |

In last meeting, it was agreed for high order QAM with uniform constellation, e.g., 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies.

RAN4 can first evaluate the benefit of UL 1024QAM through link-level simulation, once link-level simulation assumptions are approved.

Benefit of higher-order modulation in NW (System-level simulation)

The system level simulation as supplementary for link level simulation is to check whether the UE working on the given modulation can achieve target SNR at BS side and to further confirm the modulation is feasible.

In RAN1 #122bis meeting, the BS antenna modelling were agreed as following:

- For around 700MHz,

  - For around 700MHz, 32 for total number of antenna element at base station, 4 for total number of TXRU at base station, (8, 2, 2, 1, 1; 1, 2) for (M,N,P,Mg,Ng; Mp, Np), and (0.5, 0.5)λ for (dH,dV) are assumed as the baseline combination.

  - For around 700MHz, 64 for total number of antenna element at base station, 8 for total number of TXRU at base station, (8, 4, 2, 1, 1; x, y) for (M,N,P,Mg,Ng; Mp, Np), and (0.5, 0.5)λ for (dH,dV) are assumed as the optional combination.

- For around 2GHz carrier frequency, for BS antenna modelling

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg , Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1(Optional) | 8 | 4 | (2, 2, 2, 1, 1; 1, 2) | (0.5, 0.5)λ |
| Combination 2 (Baseline) | 32 | 8 | (4, 4, 2, 1, 1; 1, 4) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1(Optional) | 32 | 4 |  | (0.5, 0.8)λ |
| Combination 2 (Baseline) | 192 | 64 | (12, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

- For around 7GHz carrier frequency:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 64 | 32 | (4, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 768 | 128 | TBD | (0.5, 0.8)λ |
| Combination 2 | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ |
| Combination 3 | 1536 | 256 | TBD | (0.5, 0.8)λ |
| Combination 4 | 2048 | 256 | (32, 32, 2, 1, 1, 8, 16) | (0.5, 0.5)λ |
| Combination 5 | 2048 | 512 | (64, 16, 2, 1, 1; 16, 16) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

RAN1 has agreed BS modelling for 700MHz, 2GHz and 7GHz.

RAN4 could first evaluate higher-order modulation for 700MHz, 2GHz, and 7GHz using RAN1 agreed BS modelling.

RAN4 could discuss the system-level simulation assumption firstly based on the format in Table 5

- Table 5 System level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 500m | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | 3D-Uma LOS and NLOS in Table 7.2-1 of 36.873 | 3D-InH LOS and NLOS in Table 7.2-1 of 36.873 |
| Carrier frequency |  | 700MHz, 2GHz, 7GHz |  |
| BS antenna configuration |  | 700MHz: (M,N,P,Mg,Ng; Mp, Np)= (8, 2, 2, 1, 1; 1, 2) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi 2GHz (M,N,P,Mg,Ng; Mp, Np)= (12, 8, 2, 1, 1; 4, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi 7GHz (M,N,P,Mg,Ng; Mp, Np)= (64, 16, 2, 1, 1; 16, 16) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi | 2GHz (M,N,P,Mg,Ng; Mp, Np)= (4, 4, 2, 1, 1; 1, 4) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi (M,N,P,Mg,Ng; Mp, Np)= (16, 16, 2, 1, 1; 8, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | Omnidirectional, 0dBi |  |
| System bandwidth |  | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |  |
| Target SNR at BS side |  | Get from link-level simulation |  |
| UE max output power |  | Around 700 MHz: PC3 23dBm Around 2 GHz: FDD 23dBm, TDD 26dBm Around 7 GHz: PC2: 26 dBm |  |
| Power control |  | Power control parameters set to achieve the target SNR at BS side. |  |

Conclusion

This paper discussed modulation and proposed:

RF requirements impacted by modulation

Proposal 1: 	The modulation evaluations need to consider EVM, MPR, AMPR and IBE requirement.

Observation 1: 	MPR, AMPR and IBE impacted by modulation are through the EVM requirements.

Proposal 2: 	For 5G existing UE Tx modulation, RAN4 should evaluate whether the BS Rx SNR can be relaxed before evaluate UE Tx EVM, MPR and AMPR.

Proposal 3: 	RAN4 can focus on new higher-order modulation evaluation based on uniform constellation firstly, i.e., UL 1024 QAM.

EVM budget

Observation 2: 	It is hard to achieve lower EVM for LO phase noise and IQ imbalance than the values for 256QAM.

Proposal 4: 	EVM budget for higher-order modulation need consider the UE implementation based on the non-linearity sources of PA, transmitter, I/Q imbalance and LO phase noise.

Proposal 5: 	Align the PA model with other evaluation in 6G.

SNR target evaluation

Proposal 6: 	To determine a proper EVM for higher-order modulation, RAN4 need evaluate the required SNR of the BS Rx by link-level simulation.

Proposal 7: 	The phase noise model doesn’t need to introduce in 6G FR1 link-level simulation.

Observation 3: 	RAN1 has given some configurations and scenarios for system-level simulations.

Proposal 8: 	For higher-order modulation, RAN4 can focus on the scenarios of Urban Macro and Indoor using the parameters in Table 3 as starting point:

- Table 3 The frequency parameters and scenarios for modulation evaluation

|  | Urban Macro | Indoor Hotspot |
| --- | --- | --- |
| Carrier frequency | Around 700 MHz Around 2 GHz Around 7 GHz | Around 2 GHz Around 7 GHz |
| Simulation BW | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |  |

Proposal 9: 	RAN4 can discuss the link-level simulation assumption firstly, based on the format in Table 4

Table 4 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700 MHz, 2 GHz,7GHz |
| CBW/SCS | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, 5Hz Doppler frequency TDL-D 10ns delay spread, 5Hz Doppler frequency Static (AWGN) |
| MCS | Based on 5G MCS for 1024QAM PDSCH in Table 5.1.3.1-4 of TS 38.214: CP-OFDM: MCS 23, 24 other MCSs are not precluded |
| Symbol type | CP-OFDM |
| HARQ | 4, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| txEVM + rxEVM | txEVM = rxEVM: 2%+2%, 2.5%+2.5%, 3%+3%, 3.5%+3.5% txEVM > rxEVM: 3%+2.5%, 3.5%+2.5% |

Proposal 10: 	RAN4 can first evaluate the benefit of UL 1024QAM through link-level simulation, once link-level simulation assumptions are approved.

System-level simulations

Observation 4: 	RAN1 has agreed BS modelling for 700MHz, 2GHz and 7GHz.

Proposal 11: 	RAN4 could first evaluate higher-order modulation for 700MHz, 2GHz, and 7GHz using RAN1 agreed BS modelling.

Proposal 12: 	RAN4 could discuss the system-level simulation assumption firstly based on the format in Table 5

- Table 5 System level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 500m | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | 3D-Uma LOS and NLOS in Table 7.2-1 of 36.873 | 3D-InH LOS and NLOS in Table 7.2-1 of 36.873 |
| Carrier frequency |  | 700MHz, 2GHz, 7GHz |  |
| BS antenna configuration |  | 700MHz: (M,N,P,Mg,Ng; Mp, Np)= (8, 2, 2, 1, 1; 1, 2) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi 2GHz (M,N,P,Mg,Ng; Mp, Np)= (12, 8, 2, 1, 1; 4, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi 7GHz (M,N,P,Mg,Ng; Mp, Np)= (64, 16, 2, 1, 1; 16, 16) (dH, dv) = (0.5λ, 0.5λ) GE,max = 8 dBi | 2GHz (M,N,P,Mg,Ng; Mp, Np)= (4, 4, 2, 1, 1; 1, 4) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi (M,N,P,Mg,Ng; Mp, Np)= (16, 16, 2, 1, 1; 8, 8) (dH, dv) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | Omnidirectional, 0dBi |  |
| System bandwidth |  | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |  |
| Target SNR at BS side |  | Get from link-level simulation |  |
| UE max output power |  | Around 700 MHz: PC3 23dBm Around 2 GHz: FDD 23dBm, TDD 26dBm Around 7 GHz: PC2: 26 dBm |  |
| Power control |  | Power control parameters set to achieve the target SNR at BS side. |  |

Reference

R4-2514650 WF on [116bis][101] 6G system parameter, Huawei, RAN4 #116bis 2025-10