3GPP TSG-RAN WG4 Meeting #117		    R4-2521451
Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on 6GR spectrum

Source:	OPPO

Agenda item:	8.5

Document for:	Approval

- Introduction

In RAN4 last meeting, the WF agreed this thread main discuss frequency range, bands and regulatory survey [1].

In this contribution, we further discuss the 6GR frequency range and related bands issues.

- Discussion

  - Frequency ranges

2.1.1 Candidate criteria for frequency ranges

In last meeting, it was agreed further discuss 6G frequency range based on following candidate criteria as the starting point [1]:

- RAN4 acknowledge that a definition of frequency ranges depends on multiple criteria’s which may have different importance dependent on individual company views. As a starting point the following can be discussed, not precluding companies to also consider others:

  - Regulatory framework

  - UE RF front-end architecture (e.g. whether requirements can be verified conducted or not)

  - Commonality of UE RF/RRM requirements

  - Numerology dependency

  - Unification of TN and NTN operation

  - BS types and requirements

- Companies are encouraged to share information on the criteria they are considering for their proposal on frequency range definition(s)

- RAN4 will initially focus the discussion on frequency ranges based on TN deployment

- RAN4 encourage companies to be precise when referring to proposed frequency ranges by only using defined NR ranges as FR1, FR2-1 or FR2-2 or write the definition with frequency point(s) when referring to a proposal.

  - Note this is not related to the discussion of a band around 7 and 15 GHz

- Postpone the discussion on naming for frequency ranges in 6G to after an agreement on whether and how they are defined.

To further discuss 6G frequency range based on above candidate criteria, we first analysis the relationship of 5G frequency range and these candidate criteria, Table 1 listed the key impact factors in NR for the candidate criteria:

Table 1: The key impact factors in NR for the candidate criteria

| Candidate criteria for FR definition | Key impact factors in NR |
| --- | --- |
| Regulatory framework | SAR/MPE |
| UE RF front-end architecture | Whether requirements can be verified conducted or not |
| Commonality of UE RF requirements | Antenna connector/Conducted requirements OTA/Radiated requirements |
| BS types and requirements | BS type 1-C/antenna connector/Conducted requirements BS type 1-H/TAB/Conducted requirements BS type 1-H/RIB/OTA requirements BS type 1-O/RIB/OTA requirements BS type 2-O/RIB/OTA requirements |
| Commonality of UE RRM requirements | Beam management/Rx antenna and Rx chain support/timing requirements/measurement gap/delay/interruption |
| Numerology dependency | Different SCSs with different max CBW in different FR |
| Unification of TN and NTN operation | Different FR for TN and NTN in NR |

The SAR or MPE limitation from regulatory is due to radiofrequency electromagnetic fields (RF EMFs) have the ability to penetrate the human body, with the main effect of this being a rise in temperature in the exposed tissue. Another general characteristic of RF EMFs is that the higher the frequency, the lower the depth of penetration of the EMFs into the body. Hot spots effect is more pronounced as frequency increases due to the smaller penetration depth. Therefore, adopting SAR or MPE limitation depends on the depth of penetration of the EMFs.

- Adopting SAR or MPE limitation for the given frequency depends on the depth of penetration of the EMFs.

In the research of ICNIRP, from 400 MHz to 6 GHz, ICNIRP specifies the restriction in terms of specific energy absorption (SA) of any 10-g cubic mass; Above 6 GHz, ICNIRP specifies the exposure level for both Head and Torso, and Limbs, in terms of absorbed energy density (Uab) over any square averaging area of 4 cm2; An additional exposure level for square 1-cm2 averaging areas is applicable for EMFs with frequencies of >30 to 300 GHz to account for focused beam exposure. Excerpt the basic restriction from ICNIRP website as following table:

- ICNIRP uses 6GHz as the boundary of SAR or MPE limitation, and account for focused beam exposure above 30GHz.

In addition, SAR and MPE is tested through OTA, it’s no relationship with the format of RF requirements definition, i.e., based on conducted or radiated.

- Both of SAR and MPE are tested through OTA, SAR/MPE shouldn’t be the limit factors of frequency range definition.

In 5G NR, the reason why the RF requirements for FR2-1 are defined based on radiated characteristics rather than the same as FR1 which is based on conducted characteristics is mainly because of UE/BS RF front-end architecture:

- FR2-1 belongs to millimeter waves, the wavelength is relatively short, correspondingly, the size of RF proponents is relatively small.

- Moreover, the spatial transmission loss of millimeter waves is significant. The beamforming gain of the array antenna needs to be utilized to enhance the transmission distance in UE implementation.

- Further, each antenna element of the array antenna needs one Tx and Rx RF chain, the array antenna and the RF front-end are highly integrated, resulting in the RF front-end lacking a conductive test port. In additional, conductive test can’t reflect the array antenna’s beamforming gain.

Further from BS FR1 RF requirements, we can see:

- According to different requirements boundary, defined three BS types, i.e., type 1-C, type 1-H, type 1-O.

- But only two sets of RF requirements were defined, one is Conducted requirements and another is OTA requirements.

- And type 1-H can adopt conducted requirements and OTA requirements separately based on different boundaries, e.g., BS type 1-H with transceiver array boundary (TAB) adopting conducted requirements, BS type 1-H with radiated interface boundary (RIB) adopting conducted requirements, as shown in Figure 1:

Figure 1 BS type 1-H with different test boundary

- BS type 1-H can adopt conducted requirements and OTA requirements separately based on different boundaries.

- For UE and BS, the key difference between adopting conducted requirements and radiated requirements lies in whether conducted connectors can be obtained.

For 6G, the potential new spectrum are 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz according to WRC-27 discussion. Actually, in Rel-19, RAN4 has studied the identification of frequency band(s) for the terrestrial component of IMT, e.g., 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz from the RF requirements and BS/UE implementations. The study results have been captured into TR 38.922.

For 4400-4800 MHz and 7125-8400 MHz, there is no concern that the UE can have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

- For 4400-4800 MHz and 7125-8400 MHz, there is no concern that the UE can have a conducted isotropic radiation pattern antenna interface.

For 14.8-15.35GHz, one of the main challenges is the size of potential panel. As the researching in TR 38.922, the array antenna for different frequencies were compared as shown in Figure 5 from TR 38.922:

Figure 5 Patch array dimensions vs. frequency

We can observe that

- The size of single element for 15GHz is 10mm and the array length of 15GHz band is 40mm

- The antenna size for 15GHz is almost 1.5 times that of 24GHz.

- The array length for 15GHz is almost twice as long as that of a single element of 7GHz.

In addition, the SMA connector can cover up to 18GHz.The larger array antenna will increase the difficulty of UE implementation. In addition, the number of antennas and RF chains required for 6G UE will be larger than those used in 5G NR, e.g., new 6G bands of 8GHz and 15GHz need separate antennas from 3.5GHz bands. Therefore, FR1-like antenna is more feasible for 14.8-15.35 GHz, from UE design flexibility perspective.

- The size of single element for 15GHz is 10mm and the array length of 15GHz band is 40mm.

In TR 38.922, it also studied that FR1 like antenna is feasible for around 15GHz. In addition, the SMA connector to be used test can cover up to 18GHz.

- It is possible for 15GHz to obtain conducted connector and adopt conducted test.

In RAN4, numerology and FFT size will decide the max CBW together, in 6G SI, the guidance is one numerology for the same Band / sub-range. And in the spec, the numerology is defined per band. Therefore, numerology will not impact the frequency range definition.

- Numerology will not impact the frequency range definition.

By reason for the foregoing, the key criteria for frequency range definition are whether the conducted connector can be obtained, that is related to the UE/BS RF front-end architecture.

- The key criteria for frequency range definition are whether the conducted connector can be obtained.

2.1.2 6G frequency range

In last meeting, it was agreed RAN4 will initially focus the discussion on frequency ranges based on TN deployment. Currently, there is no potential spectrum between 15GHz to 24GHz, RAN4 shouldn’t consider this spectrum range in 6G day one.

Figure 2: 6G frequency range

- 15GHz to 24GHz shouldn’t be considered in TN frequency range definition in 6G day one.

- Extended FR1 up to 15300MHz and FR2-1 keeps 24250-52600 MHz as NR FR2-1, as shown in Figure 2.

  - Band definition and simplification

2.2.1 New spectrum for 6G

WRC-23 had allocated 6425-7125 MHz and 10-10.5 GHz to international mobile communications (IMT) spectrums.

For 6425-7125 MHz is totally overlapping with NR band n104.

For 10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

- 6425-7125 MHz is totally overlapping with NR band n104.

- 10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

The WRC-27 issue 1.7 aims to find new frequency allocations for future mobile communications (IMT) in the following new frequency bands: 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz.  Based on results of studies, the identification of frequency band(s), to consider for the terrestrial component of IMT:

- 4400-4800 MHz, or parts thereof, in Region 1 and Region 3;

- 7125-8400 MHz, or parts thereof, in Region 2 and Region 3;

- 7125-7250 MHz and 7 750-8 400 MHz, or parts thereof, in Region 1;

- 14.8-15.35 GHz;

Currently, ITU only identified potential new spectrum ranges of 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz.  Therefore, it’s unnecessary to research the spectrum ranges of 15-20GHz in 6G early phase.

- Candidate 6G Spectrum in WRC-27 includes 6 425-7 125 MHz, 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz which also need to be studied in RAN4.

New frequency 4400-4800 MHz has been covered by NR band n79, as shown in below Figure 2, therefore, the general parameters can be extracted from requirements defined for NR operating band n79.

Because of n79 will also be considered for re-farming into 6G, RAN4 should first investigate whether the n79 can be re-employed for 6G to cover the new 4400-4800MHz spectrum range of 6G, rather than directly introducing a new 4400-4800MHz frequency band.

Figure 7 candidate new bands in 6GR

- New spectrum range of 4400-4800 MHz has been covered by NR band n79, furthermore, band n79 will also be considered for re-farming into 6G.

- RAN4 should first study whether the n79 can be re-employed for 6G to cover the new 6G spectrum 4400-4800MHz.

ITU may assign different parts of 7125-8400 MHz to different regions in 6G, to facilitate adopt a unified design especially UE RF front-end, RAN4 should first investigate whether 7125-8400 MHz can be introduced as a global band for 6G.

- For the candidate spectrum 7125-8400 MHz, NS value could be used to solve specific requirements.

- RAN4 should strive to define global bands for 7125-8400 MHz and 14.8-15.35 GHz for 6G.

2.2.2 Re-farming bands

Although 5G NR has defined 70+ FR1 bands and 6 FR2-1 bands, many frequency bands even for FR1 have not been actually deployed due to various reasons. The similar problems will still exist, and not all 5GR bands can be re-farmed into 6G quickly in 6G day one, operators need to coordinate the allocation of 6G and 5G frequencies for re-farming bands.

- 6G will coexist with 5G NR for a long time, and operators also need time to re-farm the 5G spectrum.

- FR1 re-farming bands should be introduced step by step based on Operators’ request.

In the beginning of 6G, a variety of aspects will be brought up and required to be enhanced here, however, with limited TU, it is important for RAN4 to identify the most important aspects that need to be focus on.

In additional, so far there is only a few countries deployed 5G in FR2 across the globe due to the limitation of the application scenarios.

- With 6G limited TU, it is important for RAN4 to identify the most important aspects to focus on.

- The re-farming of FR2 bands should depend on the level of interest from industry.

2.2.3 Re-farming band simplification

As description in the 6G study item, from the technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). Therefore, the 6G bands mainly come from Re-farming bands from 5G NR and 6G new bands.

In current 5G NR spec, it has defined 70+ FR1 bands and 6 FR2-1 bands, some bands are partially overlapping as shown in below Figure 8:

Figure 8 Partially overlapping bands in 5G NR

These overlapping bands defined through separate band number is due to they are deployed in different countries or regions. For example, for overlapping bands of band n85 and band n12, n85 was defined for 700 MHz A Block spectrum of America, and n12 is only be deployed in Japan; for band n105 and n71, n105 is almost deployed in APT and n71 is deployed mainly in north America; for band n77 and n78, n77 is deployed in those countries like North America and Japan that require ultra-wideband, but n78 is one 5G frequency band globally, with extremely strong universality in Europe, China, and the Asia-Pacific region, and so on.

Although these overlapping frequency bands were defined as separate band in 5G, with the evolution of technology, the UE implementations have begun to adopt a unified design especially RF front-end to support dual overlapping frequency bands. For example, n2 and n25, n77 and n88, n38 and n41, these overlapping bands have been unified design using one set of RF front-end in UE implementation.

- In 5G NR, many overlapping bands have adopted same RF front-end in real UE implementation which makes defining separate bands is not meaningful.

In additional, 5G NR has defined tens of thousands of band combs, more band numbers will cause more band combination numbers, this trend is unsustainable in the future. RAN4 should avoid to define separate band numbers for overlapping frequency bands. For example, RAN4 should consider to introduce band n25 into 6G when operators’ request re-farm band n2 into 6G, since the spectrum range of n25 is totally overlapping with band n2 and its spectrum range is larger than n2.

- Consider to merge overlapping bands as much as possible when re-farming FR1 bands.

    - Naming 6G bands

In last meeting, it was agreed RAN4 shall re-use the prefix concept from NR for 6G

- Note: this agreement has no implication or limitation on the RAN1 and potential RAN4 discussion of UL and DL decoupling.

- Postpone the discussion on naming for the prefix of the 6G bands to next meeting to allow companies to consider their view.

In 5G, 5G Radio Access Technology is called “NR”, the prefix of band number is “n”. In the 6G SI [2], 6G Radio Access Technology is abbreviated with “6GR”, so the prefix of band number could be “r”.

- The prefix of the 6G bands could be “r”.

- Conclusion

This paper discussed the related topics of spectrum and proposed:

Frequency ranges

Observation 1: Adopting SAR or MPE limitation for the given frequency depends on the depth of penetration of the EMFs.

Observation 2: ICNIRP uses 6GHz as the boundary of SAR or MPE limitation, and account for focused beam exposure above 30GHz.

Proposal 1:    Both of SAR and MPE are tested through OTA, SAR/MPE shouldn’t be the limit factors of frequency range definition.

Observation 3: BS type 1-H can adopt conducted requirements and OTA requirements separately based on different boundaries.

Proposal 2:   For UE and BS, the key difference between adopting conducted requirements and radiated requirements lies in whether conducted connectors can be obtained.

Observation 4: For 4400-4800 MHz and 7125-8400 MHz, there is no concern that the UE can have a conducted isotropic radiation pattern antenna interface.

Observation 5: The size of single element for 15GHz is 10mm and the array length of 15GHz band is 40mm.

Proposal 3:   It is possible for 15GHz to obtain conducted connector and adopt conducted test.

Proposal 4:   Numerology will not impact the frequency range definition.

Proposal 5:   The key criteria for frequency range definition are whether the conducted connector can be obtained.

Proposal 6:   15GHz to 24GHz shouldn’t be considered in TN frequency range definition in 6G day one.

Proposal 7:   Extended FR1 up to 15300MHz and FR2-1 keeps 24250-52600 MHz as NR FR2-1, as shown in Figure 2.

New spectrum for 6G

Observation 6: 6425-7125 MHz is totally overlapping with NR band n104.

Observation 7: 10-10.5 GHz with very restrict limitation, no operator will support it in 6G clearly.

Observation 8: Candidate 6G Spectrum in WRC-27 includes 6 425-7 125 MHz, 4400-4800 MHz, 7125-8400 MHz, and 14.8-15.35 GHz which also need to be studied in RAN4.

Observation 9: New spectrum range of 4400-4800 MHz has been covered by NR band n79, furthermore, band n79 will also be considered for re-farming into 6G.

Proposal 8:   RAN4 should first study whether the n79 can be re-employed for 6G to cover the new 6G spectrum 4400-4800MHz.

Observation 10: For the candidate spectrum 7125-8400 MHz, NS value could be used to solve specific requirements.

Proposal 9:   RAN4 should strive to define global bands for 7125-8400 MHz and 14.8-15.35 GHz for 6G.

Re-farming bands

Observation 11: 6G will coexist with 5G NR for a long time, and operators also need time to re-farm the 5G spectrum.

Proposal 10:   FR1 re-farming bands should be introduced step by step based on Operators’ request.

Observation 12: With 6G limited TU, it is important for RAN4 to identify the most important aspects to focus on.

Proposal 11:   The re-farming of FR2 bands should depend on the level of interest from industry.

Re-farming band simplification

Observation 13: In 5G NR, many overlapping bands have adopted same RF front-end in real UE implementation which makes defining separate bands is not meaningful.

Proposal 12:   Consider to merge overlapping bands as much as possible when re-farming FR1 bands.

Naming 6G bands

Proposal 13:   The prefix of the 6G bands could be “r”.

- Reference

R4-2514626 WF on 6G Spectrum, Nokia, RAN4 #116bis

RP-251881 New SID: Study on 6G Radio, NTT DOCOMO	RAN#108