3GPP TSG-RAN WG4 Meeting #117	R4-2521468

Dallas, USA, Nov 17th – 21st, 2025

Source:	Nokia

Title:	RRM performance requirements for Beam Management

Agenda item:	6.11.6

Document for:	Discussion

Introduction

The discussion on AI/ML enabled BM use case was continued in RAN4#116bis in the scope of Rel-19 WI NR_AIML_air [1] based on the objectives agreed in RAN#107. Agreements achieved during RAN4#116bis meeting are summarized in the WF [2].

In this paper, we address RRM performance requirements for AI/ML enabled beam management use case. More precisely, we discuss beam id prediction accuracy and absolute and relative RSRP accuracy requirements for predicted L1-RSRP.

Discussion

Performance requirements for AI/ML Beam Management

Beam id prediction accuracy

The following agreement on beam Id prediction was reached in RAN4#116bis:

Agreement: - Regarding the metric for beam ID only prediction, where top-K predicted beam(s) are reported, RAN4 only specifies the requirements for the following scenario(s): - K=1 - In this case, the ground truth RSRP of the predicted beam is larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB

Since the performance metric definition was finalized in RAN4#116bis meeting, we have provided the relevant specifications change in our draft CR [3] on RRM performance requirements for beam management use case with NR AI/ML Air Interface.

RAN4 to consider specification impact for beam id prediction accuracy metric captured in draft CR [R4-2521470]

However, the value of x is still to be further discussed. RAN4 should consider following aspects in order to define the value of x for beam id prediction accuracy:

- Relaxation has already been provided by giving the possibility to not predict exactly the same beam id as the strongest genie added beam id

- Another relaxation has already been provided by using erroneous UE reported measurements as ground truth for the verification of this metric

- Now, in case of wrong beam id prediction, a successful occurrence’s ground truth RSRP value should be within x dB of the ground truth RSRP value of the strongest genie added beam. Therefore, by considering a high value of x, RAN4 won’t be able to ensure comparable performance to legacy. It should be noted that legacy mechanism is based on physically performed measurements.

A relatively higher value of x would risk a much worse AI/ML BM performance in comparison to legacy.

Another main issue observed is the misalignment of simulation results among different companies. The performance delta for same value of K and x is significantly higher among different companies. This makes it very difficult to select a value of x based on available simulation results.

The simulation results are not aligned among different companies for same value of K and x.

Therefore, RAN4 needs to strive for further alignment of simulation results since the averaging of such diverse results doesn’t look like a reasonable solution. At this stage, it won’t be judicious to select a relatively higher value of x based on unaligned simulation results.

RAN4 to limit the value of x to a relatively lower value and the value of x should be based on aligned simulation results among different companies.

Furthermore, RAN4 should also consider values of K>1. However, in this case, the performance metric needs to be discussed again and only verifying the accuracy of one predicted beam is not sufficient.

RAN4 to reconsider defining beam id prediction accuracy for cases when UE reports more than one beam (i.e. K>1). However, this performance metric should verify the prediction accuracy for more than one predicted beam.

Absolute and relative RSRP accuracy

The following agreement was reached in RAN4#116 meeting for absolute and relative RSRP accuracy metric:

Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

Since in legacy, all reported beams are supposed to fulfill the accuracy requirements, the same principle should be followed in AI/ML beam management and absolute and relative accuracy requirements should be applicable to all top-K reported beams. The value of K should be the same as considered in RAN1.

Similar to legacy, in case of AI/ML BM, absolute and relative L1-RSRP accuracy requirements for predicted L1-RSRP should be applicable to all reported K beams. The value of K can be considered as 4.

Conclusion

In this paper we share our views on RAN4 impacts from issues related to RRM performance requirements for AI/ML enabled beam management use case. Specifically, we cover following aspects:

- Beam id prediction accuracy

- Absolute/relative RSRP accuracy for predicted L1-RSRP

In the paper, the following Observations and Proposals were made:

Proposal 1: RAN4 to consider specification impact for beam id prediction accuracy metric captured in draft CR [R4-2521470]

Observation 1: A relatively higher value of x would risk a much worse AI/ML BM performance in comparison to legacy.

Observation 2: The simulation results are not aligned among different companies for same value of K and x.

Proposal 2:  RAN4 to limit the value of x to a relatively lower value and the value of x should be based on aligned simulation results among different companies.

Proposal 3: RAN4 to reconsider defining beam id prediction accuracy for cases when UE reports more than one beam (i.e. K>1). However, this performance metric should verify the prediction accuracy for more than one predicted beam.

Proposal 4: Similar to legacy, in case of AI/ML BM, absolute and relative L1-RSRP accuracy requirements for predicted L1-RSRP should be applicable to all reported K beams. The value of K can be considered as 4.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2514631, “WF on requirements for AIML air interface part 1”, Qualcomm Incorporated, RAN4#116bis, Prague, Czech Republic, 13th ‒ 17th October, 2025.

- R4-2521470, “Draft CR on RRM performance requirements for beam management use case with NR AI/ML Air Interface”, Nokia, RAN4#117, Dallas, USA, November 17th-21st, 2025.