3GPP TSG-RAN WG4 Meeting #117           	R4-2521472

Dallas, USA, 17th-21th, Nov, 2025

Agenda item:	6.20.4

Source:	Samsung

Title:	Discussion and simulation results on demodulation requirement for IoT NTN

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope for IoT NTN phase 2 was discussed, the related agreement was captured into the WF in [1] as following

- OCC length - RAN4 shall configure the length of OCC operation, and add the note following when specifying the NPUSCH 1 requirement into specification: - Note: The total number of slots in NPUSCH transmission after transmission after OCC is applied Repetition number* OCC length *NslotsULNRU, where NslotsUL is number of UL slot and NRU is the number of RU. - How to model two UEs - RAN4 shall specify the SNR to achieve 70% of the maximum throughput of NPUSCH format 1 for OCC sequence indexes 0 and 1, which corresponds to one UE and another UE, respectively. - SNR with two UEs - RAN4 to set the same SNR for NPUSCH format 1 with OCC2 from two UEs.

In this contribution, the view on  remainig issue of test setup on  NPUSCH1 demodualtion requirement is provided. Meanwhile, the initial simulation results are provided for alignment purpose.

# 2	Discussion

In this section, the view on remaining issue of test setup for NPUSCH format 1 demodulation requirement

Timing offset

Besides frequency offset, during Rel-18 IoT NTN discussion, another factor related with timing offset was under discussed considering the satellite moving. RAN4 has agreed that not consider timing offset drift model in TS 36.10 but consider it in TS 36.181 as following agreement made in R4-2305965.

Agreement: Whether to consider timing offset drift model - The total timing offset doesn’t exceed the CP. - Not consider timing offset drift model in TS36.108 but consider it in TS36.181 Agreement: Timing offset drift model - Apply the additional transmitting timing drift to PUSCH, PUCCH, NPUSCH format 1 and NPUSCH format 2. - Not consider additional transmitting timing drift for PRACH and NPRACH performance requirements - Apply the gradual transmission timing offset model with step size Δt. See the table below for the step size per channel. Channel; Repetition; Tx duration in one segment; Step size Δt. eMTC PUSCH CE mode A; 8; 8ms; [0.01] µs per subframe. eMTC PUSCH CE mode B; 256; 256ms; [0.01] µs per subframe. eMTC PUCCH format 1a; 8; 8ms; [0.01] µs per subframe. NPUSCH format 1, 3.75KHz, 1 tone; 4; 256ms; [0.32] µs per RU. NPUSCH format 1, 15KHz, 12 tones; 16; 16ms; [0.01] µs per RU. NPUSCH format 2, 3.75KHz, 1 tone; 16; 128ms; [0.08] µs per RU. NPUSCH format 2, 15KHz, 1 tone; 16; 32ms; [0.02] µs per RU.

Similar as IoT NTN in FDD scenario, we prefer not to consider the timing offset drift model for IoT NTN with OCC in TS 36.108. And adding the gradual transmission timing offset for NPUSCH format1 with 15khZ and 3.75khz in TS 36.181.

Proposal 1: Apply the additional transmitting timing drift to NPUSCH format 1 with OCC feature. The values were not considered in TS 36.108, and consider it in TS 36.181 with following value

| Channel | Step size Δt |
| --- | --- |
| NPUSCH format 1, 3.75KHz, 1 tone | [0.32] µs per RU |
| NPUSCH format 1, 15KHz, 1 tone | [0.08] µs per RU |

# 3	Simulation Results

In this section, based on agreed test setup as following table 1 and table 2 for 3.75KHz SCS and 15KHz SCS, the initial simulation results are provided for alignment purpose.

Table 1: FRC parameters for NB-IoT NPUSCH format 1, 3.75KHz SCS

| Reference channel | A7-1 |
| --- | --- |
| Subcarrier spacing (kHz) | 3.75 |
| Number of allocated subcarriers | 1 |
| Diversity | No |
| Modulation | BPSK |
| ITBS / IRU | 0 / 1 |
| Payload size (bits) | 32 |
| Allocated resource unit | 2 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.29 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 96 |
| Total symbols per resource unit | 96 |
| Channel estimation length (ms) Note 1 | 16 |

Table 2: FRC parameters for NB-IoT NPUSCH format 1, 15KHz SCS

| Reference channel | A16-2 |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Number of allocated subcarriers | 1 |
| Diversity | No |
| Modulation | BPSK |
| ITBS / IRU | 0 / 1 |
| Payload size (bits) | 32 |
| Allocated resource unit | 2 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.29 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 96 |
| Total symbols per resource unit | 96 |
| Channel estimation length (ms) Note 1 | 4 |
| Note 1: Channel estimation lengths are included in the table for information only |  |

The initial simulation results are provided for alignment purpose in table 3 and table 4 for 3.75KHz and 15KHz SCS, respectively.

Table 3:  Initial simulation results for 3.75KHz with supporting inter-symbol OCC, single tone

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -5.36 |
|  | 1 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -5.26 |
| 2 | 0 | 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -8.14 |
|  | 1 | 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -8.13 |

Table 4: Initial simulation results for 15KHz with supporting inter-slot OCC, single tone

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 128HZ | 256 | 8 before OCC | 70% | -10.29 |
|  | 1 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 128HZ | 256 | 8 before OCC | 70% | -10.31 |
| 2 | 0 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 128HZ | 256 | 8 before OCC | 70% | -13.2 |
|  | 1 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 128HZ | 256 | 8 before OCC | 70% | -13.24 |

# 4	Conclusion

In this contribution, the view on remaining issue of NPUSCH demodulation requirement with supporting inter-slot OCC. Meanwhile, the initial simulation results are provided for alignment purpose.

Proposal 1: Apply the additional transmitting timing drift to NPUSCH format 1. The values were not considered in TS 36.108, and consider it in TS 36.181 with following value

| Channel | Step size Δt |
| --- | --- |
| NPUSCH format 1, 3.75KHz, 1 tone | [0.32] µs per RU |
| NPUSCH format 1, 15KHz, 1 tone | [0.08] µs per RU |

# References

R4-2515160, Way Forward for [116bis][312] Rel-19 Demodulation_Part2

R4-2305965,  WF on IoT NTN SAN demodulation requirements