3GPP TSG-RAN WG4 Meeting #117		R4-2521502
Dallas, USA, Nov. 17-21, 2025

Title:	R16 on Modified MPR behavior

Source:	OPPO

Agenda item:	10.2.1

Document for:	Approval

Introduction

Modified MPR behavior has been discussed for a long time, and WF [1] was agreed to further discuss and see what can be optimize. This paper shares views on it.

Discussion

How much change can be made for modifiedMPR-Behaviour

Now, 5G has come to the end releases, and all the 5G UE/NW products has been on the market for a long time. If there is no critical error in the spec, then for the commercial releases, it should be kept unchanged as much as possible.

Therefore, for this modifiedMPR-Behaviour, if purely optimization we would prefer not to do it at least for the early releases.

If no error in the spec, keep modifiedMPR-Behaviour unchanged for the frozen releases. Optimization focus on open releases or 6G.

Applicable requirements when bit is not set to 1

Regarding the applicable requirements when bit is not set to 1, the definition is not clear in the spec sometimes as shown in below figure.

For n5 case, it defines the applicable requirements when this bit is set to 1, and no mention of which requirements will be applied if this bit is set to 0 or this IE is absent. One interpretation is that the requirements before the targeted version of specification is applied. If there is no other interpretation then it doesn’t matter, otherwise, explicitly specify which requirement is applied will be clearer.

For n30 case, it defines the applicable requirement when this IE is “not set”. As discussed above, the wording “this bit is not set” should be either set to “0” or this IE is absent.

The applicable requirement when the bit is “not set” or “set to 0” is unclear in current spec, and may have different interpretations. The applicable requirement should be explicitly stated.

The applicable requirements when the bit is “set to 0” shall be explicitly specified in the modifiedMPR-Behaviour table.

Change more than once

The MPR/AMPR requirements shall be kept as stable as possible and no frequent changes in the spec. From this perspective, more than two changes (tighten or relaxation) of same requirements are not expected and shouldn’t happen.

However, if this kind of change happens, then separate modifiedMPR-Behaviour bit shall be used.

MPR/AMPR requirements shall be kept as stable as possible and more than two changes (tighten or relaxation) of same requirements are not expected and shouldn’t happen.

If requirement change more than once happens, then separate modifiedMPR-Behaviour bit shall be used.

Version numbers

It is valid point that the MPR/AMPR may change (correction) in the later releases and fix the version number will make these changes bypassed. However, with the interpretation of the version number in the table is only to indicate the MPR is changed in which version of spec, then it becomes clear.

To keep the spec unchanged/consistent, keep the version number in current spec, and add one additional note to clarify the meaning of this version number is a good way to go. The NOTE can be defined as below:

- NOTE x: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P.

keep the version number in current spec, and add one additional note to clarify the meaning of this version number as below:

    - NOTE x: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P.

N41 related bits

Bit 0/1 contradict with bit 2

In n41 several modifiedMPR bits have been defined, and bit 0/1 are mandatory to set to 1 since Rel-16, meanwhile the bit 2 may be set to 1. If look at the bit definition, bit 0/1 are only for MPR requirements defined in 101-3 v15.5.0, and bit 2 is for both MPR and AMPR defined in 101-3 v16.4.0. Here, bit 0/1 and bit 2 are referring to different MPR tables. If bit 0/1 are mandatory to set to 1, then 101-3 v15.5.0 MPR is applied, how could UE further set bit 2 to 1 and refer to another MPR table?

For n41, bit 0/1 is contradicting with bit 2 in the applied MPR due to “shall set to 1” for bit 0/1 but still “may set to 1” for bit 2.

History of the bit changes in n41

To find out the difference of MPR/AMPR among different releases, below captured the changing history of n41.

- In and before 38.101-3 v15.4.0, the MPR of EN-DC referred to LTE and NR MPR spec respectively.

- Then in 38.101-3 v15.5.0, the MPR specifically for EN-DC was defined and modifiedMPR bit 0/1 for n41 were introduced.

- In 38.101-3 v16.3.0 (CR R4-2008904), the AMPR was changed together with PC1.5 DC_(n)41AA introduced. And the n41 modifiedMPR bit 2 was introduced. Up to now, the Rel-16 modifiedMPR still “may” instead of “shall”.

  - Note: MPR is not changed in this CR, but in another CR R4-2006650 there is indeed one typo correction

- Then, in RAN4#95e the modifiedMPR table was agreed to move from 101-3 to 101-1(R15 CR R4-2009177). And in RAN4#96e, the R16 CR R4-2010343 was agreed and changed n41 modifiedMPR bit 0/1 from “may” to “shall”, and add bit 2 to indicate the MPR/AMPR in 101-3 v16.4.0.

From above history, it is unclear why the bit 0/1 was changed from “may” to “shall”.

When the modifiedMPR table was moved from 101-3 to 101-1, the n41 bit 0/1 were changed from “may” to “shall” with unknown reason.

MPR change before and after 101-3 v15.5

Comparing the difference 101-3 v16.4 and v15.5, it can be found the difference is some additional clarification of the requirement application in v16.4 as highlighted below. And this change is made from v15.6 which means after v15.5, there is no technical changes.

- For contiguous CA

- For non-contiguous CA

The MPR for n41 CA in 101-3 didn’t change after v15.5, i.e., MPR is same between v15.6 and v16.4. This means the bit 2 indication of v16.4 is actually for AMPR changes instead of MPR.

Therefore, to handle the MPR contradiction between bit 0/1 and bit 2, one solution is to remove MPR from bit 2, and keep it purely for AMPR.

To solve the MPR contradiction between bit 0/1 and bit 2, it is proposed to remove the MPR from bit 2, and keep it purely for AMPR.

Conclusion

This paper discussed xxxx

How much change can be made for modifiedMPR-Behaviour

Proposal 1: 	If no error in the spec, keep modifiedMPR-Behaviour unchanged for the frozen releases. Optimization focus on open releases or 6G.

Applicable requirements when bit is not set to 1

Observation 1: 	The applicable requirement when the bit is “not set” or “set to 0” is unclear in current spec, and may have different interpretations. The applicable requirement should be explicitly stated.

Proposal 2: 	The applicable requirements when the bit is “set to 0” shall be explicitly specified in the modifiedMPR-Behaviour table.

Change more than once

Observation 2: 	MPR/AMPR requirements shall be kept as stable as possible and more than two changes (tighten or relaxation) of same requirements are not expected and shouldn’t happen.

Proposal 3: 	If requirement change more than once happens, then separate modifiedMPR-Behaviour bit shall be used.

Version numbers

Proposal 4: 	keep the version number in current spec, and add one additional note to clarify the meaning of this version number as below:

    - NOTE x: Specification version in the “Definition” column in Table L.1-1 refer to the version N in which the requirements were modified. If UE supports release P which is later than release N then UE supports requirements defined in that release P.

N41 related bits

Observation 3: 	For n41, bit 0/1 is contradicting with bit 2 in the applied MPR due to “shall set to 1” for bit 0/1 but still “may set to 1” for bit 2.

Observation 4: 	When the modifiedMPR table was moved from 101-3 to 101-1, the n41 bit 0/1 were changed from “may” to “shall” with unknown reason.

Observation 5: 	The MPR for n41 CA in 101-3 didn’t change after v15.5, i.e., MPR is same between v15.6 and v16.4. This means the bit 2 indication of v16.4 is actually for AMPR changes instead of MPR.

Proposal 5: 	To solve the MPR contradiction between bit 0/1 and bit 2, it is proposed to remove the MPR from bit 2, and keep it purely for AMPR.

Reference

R4-2511776 WF on Modified MPR Behavior, Xiaomi, MTK, OPPO, Nokia, #116 2025-08