3GPP TSG-RAN WG4 Meeting # 117	R4-2521517

# Dallas, USA, Nov. 17-21, 2025

Source:	LG Electronics

Title:	Discussion for Rel-20 HPUE RF requirement

Agenda item:	7.1.4

Document for:	Discussion

Introduction

In the RAN4#115, the WF on HPUE requirements for NR-NTN was approved. In this discussion, we share our point of view for the HPUE requirements for NR-NTN based on the approved WF [1].

Discussion

Table 1 Way forward for UE RF requirements for NTN_NR_HPUE [1]

REFSENS degradation Agreement: Encourage companies to provide the general methodology for deriving the REFSENS degradation (RSD) in next meeting.

Reference sensitivity degradation for 2Tx FDD

For PC1.5 operation in 2Tx FDD mode, the transmitter and receiver are active simultaneously. This concurrent operation can introduce additional uplink (UL) leakage, which may affect the receiver sensitivity (REFSENS). Therefore, it is necessary to define the Receiver Sensitivity Degradation (RSD).

To define RSD, prior agreement on the duplex isolation factor and antenna isolation factor is required:

Duplex Isolation Factor

- For handheld devices, typical duplex isolation is in the range of 45–50 dB.

- For non-handheld devices, the duplex isolation factor can be 5 dB or more higher compared to handheld devices.

- Additionally, duplex isolation may vary depending on Tx-Rx frequency separation for each band.

  - For example, in n1, the Tx-Rx separation is 190 MHz, so the same duplex isolation factor may apply for both narrow and wide channel bandwidths.

  - In contrast, for n25, the Tx-Rx separation is only 80 MHz, meaning the duplex isolation factor could differ depending on channel bandwidth.

Antenna Isolation Factor

- In a 1T-2R configuration, antenna isolation provides a benefit.

- However, in a 2T-2R configuration, the benefit from antenna isolation is negligible.

- Therefore, agreement on antenna isolation is not necessary for 2T-2R structures.

Proposal 1: Consider duplexer isolation factor for specifying the PC1.5 2Tx FDD RSD requirement as shown below table.

|  | CBW≤20 MHz | CBW> 20 MHz |
| --- | --- | --- |
| n1 | 50 dB | 50 dB |
| n25 | 50 dB | 40 or 35 dB |

Proposal 2: No need to consider antenna isolation factor for 2t-2r architecture assuming dominant interference from duplexer.

Conclusion

Proposal 1: Consider duplexer isolation factor for specifying the PC1.5 2Tx FDD RSD requirement as shown below table.

|  | CBW≤20 MHz | CBW> 20 MHz |
| --- | --- | --- |
| n1 | 50 dB | 50 dB |
| n25 | 50 dB | 40 or 35 dB |

Proposal 2: No need to consider antenna isolation factor for 2t-2r architecture assuming dominant interference from duplexer.

- References

# [1] R4-2515087, Way Forward for [116bis][316] NR_UE_RF_Ph5 (Nokia)