# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2521543

# Dallas, USA, 17th – 21th Nov, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements on Ku band NTN

Agenda item:	5.10.2

Document for:	Approval

Introduction

RRM core requirements for Rel-19 Ku band NTN are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our views on RRM performance requirements on Ku band NTN.

Discussion

In previous meetings, we have already defined several core requirements, we shall recognize defined requirements and discuss which test cases shall be confirmed.

In core part discussion, we reached several agreements as below:

Agreement: - For SCS, consider SCS=30kHz in FR1-NTN numerology and 120kHz in FR2-NTN numerology, following the discussion in BDaT session. - Mobile VSAT UE shall be considered from RAN4 RRM requirement perspective. There are four cases shall be considered for mobile VSAT UE: - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology Agreement: - Define RRM requirements for added 15kHz SCS in FR1-NTN numerology for GSO in Ku-band Agreement: - Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

It is evident that the requirements defined for FR2-NTN apply for VSAT UE working in FR1-NTN, except for timing requirements. Since the timing requirements are not related to UE antenna but different SCS. Based on this point, we would like to define measurement accuracy requirements and test procedures.

During the last meeting, RAN4 mainly discussed the performance requirements including measurement accuracy requirements, the following agreements could be seen as below:

Agreement - The legacy measurement accuracy requirements for Ka band NTN (FR2-NTN) shall be reused for Ku band with FR1 numerology and FR2 numerology. - Introduce FR1 numerology in FR2-NTN accuracy requirements table. - The following measurement accuracy requirements shall be considered: - Intra-frequency RSRP accuracy requirements - Inter-frequency RSRP accuracy requirements - Intra-frequency RSRQ accuracy requirements - Inter-frequency RSRQ accuracy requirements - Intra-frequency SINR accuracy requirements - Inter-frequency SINR accuracy requirements - L1-RSRP accuracy requirements - FFS the band group to include Ku band

It is observed that the legacy measurement accuracy requirements of Ka band NTN will be reused for Ku band with FR1 numerology and FR2 numerology. However, some companies have some concern whether the band group contains Ku band. We deem that only legacy NTN frequency range was extended in RF session, the band group shall maintain the legacy ones.

Proposal 1: RAN4 shall not add the new band group.

For test cases for Ku bands in FR1-NTN, RAN4 listed two options as below:

- Further discuss the following options - Option 1: reuse exiting test cases defined for SAN FR1 - Add a table clarifying test applicability for the corresponding test cases for VSAT UEs. - Add OTA related parameters - Option 2: reuse existing test cases defined for FR2-NTN, unless otherwise specified - Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

During previous meetings, RAN4 reached a conclusion as below:

Agreement: Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

That means the legacy FR2-NTN requirements shall apply for VSAT UE working on Ku band with FR1 numerology. It is believed that the existing test cases defined for FR2-NTN shall be reused naturally. However, a new configuration shall be clarified for FR1 numerology.

Proposal 2: Reuse existing test cases defined for FR2-NTN and specify the test configuration with FR1 numerology.

For test applicability and principle, only one remaining issue shall be discussed, as follows:

- RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists. - Option 1: it is at least to pass two test configurations: one with FR1 numerology and one with FR2 numerology. - Other options are not precluded.

To our understanding, option 1 shall be discussed case by case. Except UL timing requirements, other RRM requirements refer to FR2-NTN requirements. For UL timing requirements, VSAT UE with FR1 and FR2 numerology has different requirements, for this reason, it is at least to pass two test configurations: one with FR1 numerology and one with FR2 numerology. For other RRM requirements, we believe the existing test cases defined for FR2-NTN could be reused and VSAT UE works on Ku band could focus on FR2 numerology even if FR1 numerology has been clarified in test configuration.

Proposal 3: For test cases of UL timing requirements, VSAT UE shall be at least to pass two test configurations. Otherwise, only considering test configuration with FR2 numerology.

Conclusion

In this paper we provided our views on RRM performance requirements on Ku band NTN.

Proposal 1: RAN4 shall not add the new band group.

Proposal 2: Reuse existing test cases defined for FR2-NTN and specify the test configuration with FR1 numerology.

Proposal 3: For test cases of UL timing requirements, VSAT UE shall be at least to pass two test configurations. Otherwise, only considering test configuration with FR2 numerology.

- 4 References

- TS38.133