3GPP TSG-RAN WG4 Meeting #117		    R4-2521566
Dallas, USA, Nov. 17-21, 2025

Title:	On 6GR waveform

Source:	OPPO

Agenda item:	8.2.1

Document for:	Approval

Introduction

In the RAN plenary (RAN#108), a study item on ‘6G RAN’ was approved [1]. This contribution discusses the 6GR waveform aspect.

Discussion

Waveform

General

During the last RAN4#116bis meeting, a WF on system parameter has been agreed as [2]. The waveform part has been discussed and the agreement is captured as below. The further RAN4 evaluation work has been discussed and proposed.

- Agreement

  - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1

  - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects

    - Tx assumption including PA model

    - Related RF requirements which should be taken into consideration

    - Both UL and DL are considered

    - Take the candidate waveforms identified/discussed in RAN1 as the baseline

    - Identify the evaluation metric, e.g. net gain

    - Implementation constraints

  - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs

    - Evaluation cases could be selected step-by-step, based on RAN1 progress

    - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

From RAN4 perspective, the whole RAN4 framework needs to focus on three aspects at the stage first as:

- TX assumption including PA model

- Related RF requirements

- Evaluation metric

Below we give some discussion on evaluation metric and corresponding RF requirement, and the TX assumption as well as PA model will be discussed in separate sub-clause.

Waveform evaluation metric

As captured in the WF agreement [2], the net gain has been proposed in the last RAN4#116-bis meeting. During the last RAN1 meeting, the evaluation metric is also discussed and captured as below:

Agreement

- For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

    - A realistic PA model should be used

    - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account.

    - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged.

    - FFS: Other evaluation metrics

    - Note: Companies to report how to calculate the Tx power gain, modulation and coding

It can be observed that RAN1 has also discussed and agreed that to use Net Gain for low-PAPR proposal evaluation. During the last RAN4#116-bis meeting, companies has commented that for waveform evaluation metric, not only to consider the net gain but also the spectrum efficiency and throughput to evaluate both cell edge UE and cell center UE. However, if the main focus is for low-PAPR proposals, the Net Gain metric can be used. Also, to align the evaluation of both groups helps to simplify the procedure.

RAN1 use Net Gain as metric for waveform evaluation of low-PAPR proposals.

If the main focus is for low-PAPR proposals, the Net Gain metric can be used as evaluation metric.

Hence, we would like to also use the Net Gain definition as agreed in last RAN1 meeting as below to be the evaluation metric in RAN4 for low P-APR waveform proposals.

	Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

Here the “TX power gain relative to the reference” should be based on the evaluation considering the emission requirements as ACLR, SEM and TX EVM.

It is proposed to use Net Gain as metric for waveform evaluation of low-PAPR proposals.

Emission requirements as ACLR, SEM, TX EVM need to be evaluated in RAN4 to decide the Tx power gain.

Waveform evaluation RF requirement

In the last RAN4 WF agreement [2], RF requirements have been listed as an important part for waveform evaluation. The UE RF requirement is currently discussed in another specific agenda. However, as far as it is concerned, the formal 6GR UE RF requirement will be defined finally in work item phase. Hence, it is not realistic to wait for the UE RF requirements settled down. For evaluation propose, we should find some starting point of UE RF requirements and focus on the new around 7GHz bands which is also aligned with RAN1 LS.

The 6GR UE RF requirements will be settled down in work item phase.

The evaluation focus on the newly potential around 7GHz bands which is aligned with RAN1 LS.

For waveform evaluation, the MPR will be evaluated. Hence the SEM, ACLR, TX EVM are needed. Although currently the co-existence simulation is prepared and the ACLR will be considered base on the co-existence study, it is still reasonable to reuse the legacy NR UE FR1 ACLR at this stage for waveform evaluation. The SEM and TX EVM can be reused, too. The channel bandwidth might be different as larger CBW might be introduced, i.e., 200MHz for UE and the spectrum utilization can reuse the SU for 100MHz.

For around 7GHz bands, reuse the 5G FR1 UE RF requirement (e.g., ACLR, SEM, TX EVM, etc.) for waveform evaluation.

RAN4 can focus on 100MHz in the beginning of 6G waveform evaluation, and other Larger CBW introduced in 7GHz band can be discussed further considering the SU status .

Waveform evaluation Simulation assumption

For the TX gain evaluation, below table 1 is proposed as the starting point for simulation assumption. The

Table 1 Simulation assumption for TX gain evaluation

| Parameter | Assumption |
| --- | --- |
| Center frequency | 700MHz, 2GHz, 7GHz |
| Maximum output power | 26 dBm |
| Channel bandwidth | Around 700MHz: 20MHz Around 2GHz: 20MHz/100MHz Around 7GHz: 20MMHz/200MHz |
| Numerology | Around 700MHz: 15kHz Around 2GHz: 15/30kHz Around 7GHz: 30kHz |
| Modulation | High Priority: QPSK Further study: 16QAM/64QAM/256QAM |
| Waveform | Low P-APR candidate waveform |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |
| PA model | As proposed below in clause 2.2 |

It is proposed table 1 as simulation assumption for TX gain evaluation.

For the SNR link level simulation, below simulation assumption is proposed.

It is proposed table 2 as simulation assumption for SNR evaluation.

Table 2 Simulation assumption for SNR evaluation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700 MHz, 2 GHz,7GHz |
| CBW/SCS | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, 5Hz Doppler frequency TDL-D 10ns delay spread, 5Hz Doppler frequency Static (AWGN) |
| MCS | 7, QPSK, other MCSs are not precluded |
| Symbol type | CP-OFDM |
| HARQ | 4, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |

Waveform evaluation for DL DTF-s-OFDM

As the DFT-s-OFDM enjoys better PAPR compared to CP-OFDM, it has been proposed to study the DFT-s-OFDM waveform also for DL in 6GR. It can provide NES gain in initial access and coverage gain in PDCCH/PDSCH channels.

The DFT-s-OFDM for DL can provide NES gain and coverage gain in different channels.

Although we have to agree on the better PAPR performance of DFT-s-OFDM but for DL with two different waveforms, UE is required to differentiate these two different waveforms which will increase UE complexity. Furthermore, when UE is in blind detection for initial access, it requires UE to differentiate two different waveforms.

Two DL waveforms (DFT-s-OFDM and CP-OFDM) will increase UE implementation complexity.

However, as it is SI for 6GR, it is acceptable at this stage to study the DL DFT-s-OFDM waveform. Similar evaluation as UL waveform should be done. The DL PA model should be agreed first and it is encouraged venders to provide such PA model. Unlike the UE TX, for BS as the power consumption and PCB area are not quite limited, the PA usually works with quite large voltage and current and at a very high non-linear point but use DPD to help the BS PA output to fulfil the BS RF requirements. In such case the PA calibration point for BS and the DPD should be considered together.

No BS PA model has been agreed in RAN4.

BS PA works at high non-linearity point with DPD.

It is encouraged vendors to provide DL PA model together with DPD capability.

PA model

Background and R1 LS

PA model is one of the most important topics in RAN4 6G study which will impact the RF requirement definitions and also the candidate techniques in RAN1 evaluations.

RAN1 has sent an LS [1] to RAN4 to ask the PA models for UL/DL around 7GHz. However, from the LS, one important information missing is the targeted power class. In RAN4, different power class has different PA calibration points and in the end lead to different PA models. For around 7GHz, PC2 might be needed to achieve larger coverage, meanwhile PC3 usually is considered as the baseline. Therefore, both power classes seem need to be considered.

1. Overall Description: For waveform evaluations, e.g. evaluations of PAPR reduction/MPR, PA models are needed. RAN1 respectfully asks RAN4 to provide appropriate PA model(s) and other relevant RF parameters for both downlink and uplink evaluations, at least for around 7GHz. Any timely feedback is appreciated, before which RAN1 continues waveform evaluations with PA model assumptions that might be based on companies’ inputs. 2. Actions: To RAN4 ACTION: RAN1 would like to respectfully ask RAN4 to provide feedback in a timely manner.

PA models need to be provided for UL/DL around 7GHz. However, unclear what is the targeted power class.

Both PC2 and PC3 PA models need to be considered, if time is not enough, PC2 PA at 7GHz can be prioritized.

In 5G study phase, RAN4 has received similar request from RAN1, and RAN4 provided one UE PA model based on a 3.5GHz LTE PC3 PA [2]. The AM/AM and AM/PM figures are as below figure 1.

Figure 1 LTE PC3 PA@3.5GHz provided to RAN1 in 5G SI [2]

RAN4 has provided one UE PA model based on a 3.5GHz LTE PC3 PA in 5G SI phase.

N79 PC2 PA

In RAN1 LS, they are asking for around 7GHz, but unfortunately there is no commercialized 7GHz IMT PA that can be used to provide feed back to RAN1. The most adjacent available commercial IMT PA is at n79. Therefore, in this paper we have conducted some measurements based on commercialized n79 PC2 PA to derive the PA models.

We measured the PA input and output and got the AM/AM and AM/PM relations. Then, the polynomial is derived to best fit the PA input and output as it can be seen in figure 2 below.

Figure 2 n79 PC2 PA measurements and polynomial PA model compare

Then, the PA model is calibrated according to RAN4 MPR calibration point (DFT/QPSK/20MHz/100RB/1dB MPR/4dB RFFE IL to meet PC2 ACLR). And the calibrated PA input and output relation can be seen in figure 3.

The calibrated PA output PSD can be seen in figure 4.

Figure 3 n79 PC2 PA model after calibration

Figure 4 PC2 PA output PSD for 20MHz CBW with 100RB

The PA model after calibration is as below, where PA output y(t) to be computed from input x(t) using the formula

y(t) = p8∙x(t)8 + p7∙x(t)7 + … + p0

The coefficients are organized as follows: [p8  p7  …  p0].

- AM-AM with mW-mW units:

p_am = [-0.000274170142316334,	0.0146367116261329,	-0.307832240000256,	3.17339840884096,	-15.8958108449148,	32.7851558535623,	-79.9467234141316,	 883.173260757077,	 -0.390835520966759]

- AM-PM with mW-deg units:

p_pm = [-2.00011308984179e-05,	0.00111169057793715,	-0.0249345108649042,	0.287198594259000,	-1.77714955149587,	5.54327160174535,	-6.99314777127620,	4.54094996068356,	101.270585064848]

Consider the below PC2 n79 PA model to be used as reference for 7GHz PC2.

- AM-AM with mW-mW units:

p_am = [-0.000274170142316334,	0.0146367116261329,	-0.307832240000256,	3.17339840884096,	-15.8958108449148,	32.7851558535623,	-79.9467234141316,	 883.173260757077,	 -0.390835520966759]

- AM-PM with mW-deg units:

p_pm = [-2.00011308984179e-05,	0.00111169057793715,	-0.0249345108649042,	0.287198594259000,	-1.77714955149587,	5.54327160174535,	-6.99314777127620,	4.54094996068356,	101.270585064848]

N79 PC3 PA

Similar approaches can be done for PC3 PA, however, in real products the same PC2 PA is also applied to PC3, therefore, here we use same PA to derive the polynomial of PA model, and then calibrated to PC3 (DFT/QPSK/20MHz/100RB/1dB MPR/4dB RFFE IL to meet PC3 ACLR).

After calibration, the PC3 PA AM/AM and AM/PM can be found in figure 5. And the PA output PSD can be found in figure 6.

Figure 5 n79 PC3 PA model after calibration

Figure 6 PC3 PA output PSD for 20MHz CBW with 100RB

Similar as PC2 PA model, the PC3 PA model after calibration is as below, where PA output y(t) to be computed from input x(t) using the formula

y(t) = p8∙x(t)8 + p7∙x(t)7 + … + p0

The coefficients are organized as follows: [p8  p7  …  p0].

- AM-AM with mW-mW units:

p_am = [-0.000212823782438733,	0.0106033602392184,	-0.208120136740953,	2.00227903301997,	-9.36014736276067,	18.0167831973250,	-41.0015872157464,	422.713299333046,	-0.174579814445625]

- AM-PM with mW-deg units:

p_pm = [-3.47579818511953e-05,	0.00180295100443726,	-0.0377399093938306,	0.405678797153715,	-2.34273937168193,	6.81971393975844,	-8.02920793925361,	4.86571554699038,	101.270585064848]

Consider the below PC3 n79 PA model to be used as reference for 7GHz PC3.

- AM-AM with mW-mW units:

p_am = [-0.000212823782438733,	0.0106033602392184,	-0.208120136740953,	2.00227903301997,	-9.36014736276067,	18.0167831973250,	-41.0015872157464,	422.713299333046,	-0.174579814445625]

- AM-PM with mW-deg units:

p_pm = [-3.47579818511953e-05,	0.00180295100443726,	-0.0377399093938306,	0.405678797153715,	-2.34273937168193,	6.81971393975844,	-8.02920793925361,	4.86571554699038,	101.270585064848]

Comparison of PC3 N79 PA and LTE 3.5GHz PA

In order to better understand the difference between the n79 PA and the legacy 3.5GHz LTE PA, below figure 7 shows the AM/AM and AM/PM of these two PAs, and figure 8 shows the corresponding PSD.

From these figures, it seems the n79 PA in this paper is more linear than the legacy PA in 5G SI that RAN4 provided to RAN1.

Figure 7 AM/AM and AM/PM comparison of PC3 N79 PA and LTE 3.5GHz PA

Figure 8 PSD comparison of PC3 N79 PA and LTE 3.5GHz PA

The n79 PC3 PA in this paper seems more linear than the legacy LTE 3.5GHz PA.

BS PA model

RAN1 also asked about the PA model for DL evaluation, however, based on the discussion history in NR SI, it seems the BS PA model is impacted by factors like CFR/DPD and is implementation dependent which makes it difficult to agree on a common model [2] at that time as shown in below table.

It is also discussed what constitutes a realistic PA, especially from BS’s point of view. In general, since realistic BS PA models often use algorithms such as Crest Factor Reduction (CFR) and/or Digital Pre-Distortion (DPD) to improve PA performance and such algorithms are implementation-dependent, it is difficult for RAN4 to agree on a common model. As such, the info provided below is mainly for UE PA models.

Now RAN1 is evaluating some DL waveforms and some reference PA model seems necessary. The PA model inputs from BS side is needed.

RAN4 didn’t provide BS PA model due to implementation dependent factors like Crest Factor Reduction (CFR) and/or Digital Pre-Distortion (DPD).

RAN4 can focus on UE PA model first and then discuss possible BS PA model for the DL waveform evaluation.

Other relevant RF parameters

RAN1 also asked the relevant RF parameters for waveform evaluations, e.g., evaluations of PAPR reduction/MPR. However, apparently, the MPR evaluation is RAN4 centric works and correspondingly the PAPR impacts to MPR should be evaluated in RAN4.

For the relevant RF requirements, the Tx power and in-band/out of band emissions are all relevant like ACLR, SEM, SE, EVM, IBE, MPR, etc. RAN1 may lack of the expertise in evaluating them.

RAN1 lacks of the expertise in evaluating RF requirement impacts caused by the new waveforms.

Inform RAN1 about the RF related requirements in waveform evaluation, meanwhile, emphasize that these evaluations are RAN4 centric work and RAN1 may lack of expertise in fully evaluating them.

Draft LS reply to RAN1

Based on above discussion, the LS reply to RAN1 can consider as below.

1. Overall Description: RAN4 has discussed the RAN1 request of PA model for around 7GHz in UL and DL. However, currently there is no commercial IMT PA specifically for around 7GHz. RAN4 has discussed alternative PAs in other frequency bands like n79 and 3.5GHz and it is RAN4 understanding that these PAs can be used for the waveform evaluation in around 7GHz. The PA models discussed in RAN4 were based on commercial PAs measured by several companies, and then AM/AM and AM/PM polynomial models were derived. It is recoginized that these PA models are valid, and can be used in the waveform evaluations. It depends on RAN1 to select the proper PA models for their evaluation. The candidate PA models are as below: - UE PA model 1: n79 PC2 PA output y(t) to be computed from input x(t) using the formula y(t) = p8∙x(t)8 + p7∙x(t)7 + … + p0 The coefficients are organized as follows: [p8 p7 … p0]. - AM-AM with mW-mW units: p_am = [-0.000274170142316334, 0.0146367116261329, -0.307832240000256, 3.17339840884096, -15.8958108449148, 32.7851558535623, -79.9467234141316, 883.173260757077, -0.390835520966759] - AM-PM with mW-deg units: p_pm = [-2.00011308984179e-05, 0.00111169057793715, -0.0249345108649042, 0.287198594259000, -1.77714955149587, 5.54327160174535, -6.99314777127620, 4.54094996068356, 101.270585064848] - UE PA model 2: n79 PC3 PA output y(t) to be computed from input x(t) using the formula y(t) = p8∙x(t)8 + p7∙x(t)7 + … + p0 The coefficients are organized as follows: [p8 p7 … p0]. - AM-AM with mW-mW units: p_am = [-0.000212823782438733, 0.0106033602392184, -0.208120136740953, 2.00227903301997, -9.36014736276067, 18.0167831973250, -41.0015872157464, 422.713299333046, -0.174579814445625] - AM-PM with mW-deg units: p_pm = [-3.47579818511953e-05, 0.00180295100443726, -0.0377399093938306, 0.405678797153715, -2.34273937168193, 6.81971393975844, -8.02920793925361, 4.86571554699038, 101.270585064848] - UE PA model 3: 3.5GHz PC3 Refer to [1]. Note that the validity of these polynomial model deteriorates below input power -35 to -30 dBm, and it is suggested that fully linear model is applied below such input levels. Also, extremely deep in compression, e.g. at input power above 9 dBm, the model does not provide realistic results. This should be fine since the signal peaks would anyway be limited to a reasonable value before entering the PA. For the relevant RAN4 centric RF requirements, the Tx power and in-band/out of band emissions are all relevant which may need to be considered in PAPR/MPR evaluation like ACLR, SEM, SE, EVM, IBE, MPR, etc. References [1] R4-164542 Response LS on realistic power amplifier model for NR waveform evaluation, R4->R1, R4#79, 2016-05 2. Actions: To RAN1: RAN4 asks RAN1 to take above into account in NR waveform evaluations

Reply RAN1 on PA model as above table.

Conclusion

This paper discussed the 6GR waveform, and got below observations and proposals.

Waveform

Observation 1: 	RAN1 use Net Gain as metric for waveform evaluation of low-PAPR proposals.

Observation 2: 	If the main focus is for low-PAPR proposals, the Net Gain metric can be used as evaluation metric.

Proposal 1: 	It is proposed to use Net Gain as metric for waveform evaluation of low-PAPR proposals.

Proposal 2: 	Emission requirements as ACLR, SEM, TX EVM need to be evaluated in RAN4 to decide the Tx power gain.

Proposal 3: 	For around 7GHz bands, reuse the 5G FR1 UE RF requirement (e.g., ACLR, SEM, TX EVM, etc.) for waveform evaluation.

Proposal 4: 	RAN4 can focus on 100MHz in the beginning of 6G waveform evaluation, and other Larger CBW introduced in 7GHz band can be discussed further considering the SU status.

Proposal 5: 	It is proposed table 1 as simulation assumption for TX gain evaluation.

Table 1 Simulation assumption for TX gain evaluation

| Parameter | Assumption |
| --- | --- |
| Center frequency | 700MHz, 2GHz, 7GHz |
| Maximum output power | 26 dBm |
| Channel bandwidth | Around 700MHz: 20MHz Around 2GHz: 20MHz/100MHz Around 7GHz: 20MMHz/200MHz |
| Numerology | Around 700MHz: 15kHz Around 2GHz: 15/30kHz Around 7GHz: 30kHz |
| Modulation | High Priority: QPSK Further study: 16QAM/64QAM/256QAM |
| Waveform | Low P-APR candidate waveform |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |
| PA model | As proposed below in clause 2.2 |

Proposal 6: 	It is proposed table 2 as simulation assumption for SNR evaluation.

Table 2 Simulation assumption for SNR evaluation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 700 MHz, 2 GHz,7GHz |
| CBW/SCS | Around 700 MHz: 20MHz with 15kHz SCS Around 2 GHz: 100MHz with 30kHz SCS Around 7 GHz: 200MHz with 30kHz SCS |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, 5Hz Doppler frequency TDL-D 10ns delay spread, 5Hz Doppler frequency Static (AWGN) |
| MCS | 7, QPSK, other MCSs are not precluded |
| Symbol type | CP-OFDM |
| HARQ | 4, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |

Observation 5: 	The DFT-s-OFDM for DL can provide NES gain and coverage gain in different channels.

Observation 6: 	Two DL waveforms (DFT-s-OFDM and CP-OFDM) will increase UE implementation complexity.

Observation 7: 	No BS PA model has been agreed in RAN4.

Observation 8: 	BS PA works at high non-linearity point with DPD.

Proposal 7: 	It is encouraged vendors to provide DL PA model together with DPD capability.

PA model

Observation 9: 	PA models need to be provided for UL/DL around 7GHz. However, unclear what is the targeted power class.

Proposal 8: 	Both PC2 and PC3 PA models need to be considered, if time is not enough, PC2 PA at 7GHz can be prioritized.

Observation 10: 	RAN4 has provided one UE PA model based on a 3.5GHz LTE PC3 PA in 5G SI phase.

Proposal 9: 	Consider the below PC2 n79 PA model to be used as reference for 7GHz PC2.

	AM-AM with mW-mW units:

p_am = [-0.000274170142316334,	0.0146367116261329,	-0.307832240000256,	3.17339840884096,	-15.8958108449148,	32.7851558535623,	-79.9467234141316,	 883.173260757077,	 -0.390835520966759]

	AM-PM with mW-deg units:

p_pm = [-2.00011308984179e-05,	0.00111169057793715,	-0.0249345108649042,	0.287198594259000,	-1.77714955149587,	5.54327160174535,	-6.99314777127620,	4.54094996068356,	101.270585064848]

Proposal 10: 	Consider the below PC3 n79 PA model to be used as reference for 7GHz PC3.

	AM-AM with mW-mW units:

p_am = [-0.000212823782438733,	0.0106033602392184,	-0.208120136740953,	2.00227903301997,	-9.36014736276067,	18.0167831973250,	-41.0015872157464,	422.713299333046,	-0.174579814445625]

	AM-PM with mW-deg units:

p_pm = [-3.47579818511953e-05,	0.00180295100443726,	-0.0377399093938306,	0.405678797153715,	-2.34273937168193,	6.81971393975844,	-8.02920793925361,	4.86571554699038,	101.270585064848]

Observation 11: 	The n79 PC3 PA in this paper seems more linear than the legacy LTE 3.5GHz PA.

Observation 12: 	RAN4 didn’t provide BS PA model due to implementation dependent factors like Crest Factor Reduction (CFR) and/or Digital Pre-Distortion (DPD).

Proposal 11: 	RAN4 can focus on UE PA model first and then discuss possible BS PA model for the DL waveform evaluation.

Observation 13: 	RAN1 lacks of the expertise in evaluating RF requirement impacts caused by the new waveforms.

Proposal 12: 	Inform RAN1 about the RF related requirements in waveform evaluation, meanwhile, emphasize that these evaluations are RAN4 centric work and RAN1 may lack of expertise in fully evaluating them.

Proposal 13: 	Reply RAN1 on PA model as below.

1. Overall Description: RAN4 has discussed the RAN1 request of PA model for around 7GHz in UL and DL. However, currently there is no commercial IMT PA specifically for around 7GHz. RAN4 has discussed alternative PAs in other frequency bands like n79 and 3.5GHz and it is RAN4 understanding that these PAs can be used for the waveform evaluation in around 7GHz. The PA models discussed in RAN4 were based on commercial PAs measured by several companies, and then AM/AM and AM/PM polynomial models were derived. It is recoginized that these PA models are valid, and can be used in the waveform evaluations. It depends on RAN1 to select the proper PA models for their evaluation. The candidate PA models are as below: - UE PA model 1: n79 PC2 PA output y(t) to be computed from input x(t) using the formula y(t) = p8∙x(t)8 + p7∙x(t)7 + … + p0 The coefficients are organized as follows: [p8 p7 … p0]. - AM-AM with mW-mW units: p_am = [-0.000274170142316334, 0.0146367116261329, -0.307832240000256, 3.17339840884096, -15.8958108449148, 32.7851558535623, -79.9467234141316, 883.173260757077, -0.390835520966759] - AM-PM with mW-deg units: p_pm = [-2.00011308984179e-05, 0.00111169057793715, -0.0249345108649042, 0.287198594259000, -1.77714955149587, 5.54327160174535, -6.99314777127620, 4.54094996068356, 101.270585064848] - UE PA model 2: n79 PC3 PA output y(t) to be computed from input x(t) using the formula y(t) = p8∙x(t)8 + p7∙x(t)7 + … + p0 The coefficients are organized as follows: [p8 p7 … p0]. - AM-AM with mW-mW units: p_am = [-0.000212823782438733, 0.0106033602392184, -0.208120136740953, 2.00227903301997, -9.36014736276067, 18.0167831973250, -41.0015872157464, 422.713299333046, -0.174579814445625] - AM-PM with mW-deg units: p_pm = [-3.47579818511953e-05, 0.00180295100443726, -0.0377399093938306, 0.405678797153715, -2.34273937168193, 6.81971393975844, -8.02920793925361, 4.86571554699038, 101.270585064848] - UE PA model 3: 3.5GHz PC3 Refer to [1]. Note that the validity of these polynomial model deteriorates below input power -35 to -30 dBm, and it is suggested that fully linear model is applied below such input levels. Also, extremely deep in compression, e.g. at input power above 9 dBm, the model does not provide realistic results. This should be fine since the signal peaks would anyway be limited to a reasonable value before entering the PA. For the relevant RAN4 centric RF requirements, the Tx power and in-band/out of band emissions are all relevant which may need to be considered in PAPR/MPR evaluation like ACLR, SEM, SE, EVM, IBE, MPR, etc. References [1] R4-164542 Response LS on realistic power amplifier model for NR waveform evaluation, R4->R1, R4#79, 2016-05 2. Actions: To RAN1: RAN4 asks RAN1 to take above into account in NR waveform evaluations

Reference

RP-251881	New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

R4-2514641	WF on [116bis][101] 6G system parameter, Huawei