3GPP TSG-RAN WG4 Meeting #117	R4-2521574

Dallas, USA, Nov. 17th – 21st 2025

Agenda Item:	8.9

Source:	Ericsson

Title:	Considerations on spectrum sharing for 6G

Document for:	Discussion

# Introduction

At RAN4#116bis meeting, spectrum sharing was discussed under the 6G SI framework. Some initial high-level agreements have been reached. To facilitate the discussion, the agreements are excerpted as follows [1]:

## Sub-topic 1-0: Agenda handling

Agreements

MRSS should be considered in the ultimate RAN4 decision, including RAN4’s input to interim milestone.

Regarding some of the system parameters (e.g. SCS, sync raster, channel raster etc.), separated agenda will be designated for the scenarios of with (in spectrum sharing) and without (in system parameter) MRSS considered.

- If feasible, RAN4 strives to define unified system parameters for the scenarios with and without MRSS.

- Revisit this agenda arrangement after Feb. meeting in 2026 for potentially merging the discussion under the single agenda.

Before RAN4 receives concrete design from RAN1/2, RAN4 discussion will be based on the agreed hypothesis.

## Sub-topic 1-1: General aspects

#### Issue 1-1-1: Migration scheme

Agreements

- RAN4 focus on MRSS between 6GR and NR until further RANP guidance

- RAN4 focus on inter-RAT mobility between 6GR and NR

#### Issue 1-1-2: Scenarios for MRSS between 6GR and NR

Agreements

- For operating frequency range or band for MRSS between 6GR and NR

- Start RAN4 6G-5G MRSS study in FR1

- Whether to consider 6G-5G MRSS in FR2-1 is further discussed

Sub-topic 1-2: Aspects to facilitate MRSS

Issue 1-2-0: General consideration

Agreement

- To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study

- System parameters

- RF requirements

- RRM requirements

- Interference handling

- Taking following table as starting point to discuss which system parameters are impacted by MRSS.

| System parameters | Whether impacted by MRSS |
| --- | --- |
| Numerology | Maybe impacted |
| Channel raster | Maybe impacted |
| Sync raster | Maybe impacted |
| Waveform | FFS |
| Modulation | FFS |
| Channel bandwidth | FFS |

This contribution addresses several open issues identified in the context of spectrum sharing.

# Discussion

## 6G-5G MRSS in FR2-1

MRSS in FR2 is a critical enable for 5G to 6G transition allowing dynamic resource sharing between 5G NR and 6G systems in high-frequency spectrum. While early 5G mmWave (FR2) rollouts were indeed limited, by mid-2025, deployments have accelerated globally, creating a strong case for spectrum sharing to support 6G integration. According to GSMA, as of July 2025, 203 operators across 56 countries and territories are actively investing in 5G mmWave networks, with a steady increase in commercial launches and user adoption.

FR2-1 (24.25 – 52.6 GHz) is considered a mainstream 5G mmWave band, and it is increasingly used for high-capacity applications in urban areas, FWA, and enterprise scenarios. With 6G targeting even higher data rates and low-latency use cases (e.g., XR and AI-driven services), MRSS in FR2, especially in FR2-1, ensures seamless reuse of these expanding deployments, avoiding disruptions and maximizing spectrum efficiency during the 2030 transition. From a technical point of view, FR2-1 MRSS is not expected to be significantly more complex than FR1 MRSS.

- It is proposed to study 6G-5G MRSS in FR2-1 in RAN4.

## MRSS with NTN

NTN operates in dedicated bands, and it has been evolving since Rel-17 for 5G, with enhancements in Rel-18-19 paving the way for deeper 6G integration.

The different characteristics between TN and NTN could complicate synchronization and interference management. E.g., NTN's Doppler shifts and longer propagation delays (e.g., 10-20 ms for LEO vs. sub-ms in TN) might disrupt the tight timing required for RB scheduling. A single NTN beam typically spans vast geographic areas, overlapping numerous TB BSs. This extensive beam coverage creates high complexity in scheduling synchronization between NTN and TN systems.

Furthermore, the co-channel interference between TN and NTN within the same coverage (presumably) remains a major unresolved issue, e.g. TN signals can severely degrade NTN performance and vice versa. Given the “infeasible” coexistence scenarios without significant performance trade-offs, it is proposed not to consider MRSS between TN and NTN in the 6G study at this stage.

- It is proposed not to consider MRSS with NTN and TN in the 6G study at this stage.

## Numerology and raster for MRSS

For NR, up to three SCS configurations/numerologies are supported for data per operating band, while one or two numerologies for synchronisation. The specification for nominal CA spacing and the guard bands accommodate multiple numerologies, but this specification turned out to be complex and is now subject to different interpretations; thus far the use of multiple numerologies in a band is very limited.

- A single sub-carrier spacing configuration per operating band or frequency range would simplify specification of the channel arrangement, channel/carrier spacing, spectrum utilization and guard bands for 6GR and specification of MRSS.

For support of NR-6GR sharing by MRSS, we expect that the NR and 6GR carriers will be sub-carrier aligned. Furthermore, carriers that share spectrum must also meet coexistence requirements with adjacent blocks. This affects both the design of GBs for channel bandwidths and the raster for 6GR carriers.

From a UE perspective, the NR and 6GR carriers would be transparent, at least without DC. Carriers must nevertheless be located such that the NW and the UEs can meet unwanted emissions limits within a spectrum block.

NR and 6GR may also be frame/subframe aligned from a timing reference standpoint and for maintaining orthogonality between collocated RATs.

Whether a 7.5 kHz shift in the UL is needed depends on RAN decisions on support for eMTC and NB-IoT.

NR was designed with an ingenious synchronisation raster of a significantly reduced granularity as compared LTE to reduce cell search complexity but that yet allowed location of the smallest bandwidth (5 MHz for Rel-15) at any 100 kHz channel raster entry for migrated LTE bands for compatibility with legacy systems.

### Channel raster

The 6GR channel raster should be specified so to allow compatibility of 6GR channels with legacy channels (up to an including NR) located on the 100 kHz channel raster.

- Given a synchronisation raster, consider a channel raster that allows 6GR compatibility with adjacent legacy channels on the 100 kHz raster, while taking into account the MRSS considerations.

It is tempting to adopt the enhanced channel raster with its 10 kHz raster straight away, but it is worth observing that

- The existing NR synchronisation raster does not allow location of an NR carrier at every possible 10 kHz raster entry of the enhanced channel raster.

The enhanced raster was designed primarily for location of a UE-specific channel bandwidth within a wider BS bandwidth without restrictions.

We assume the NR-ARFCN raster will be reused for 6GR.

Support of MRSS will add constraints on the channel raster: the 6GR carrier must at least be subcarrier aligned with the NR carrier. We propose that

- For spectrum sharing (MRSS), the channel raster for 6GR is designed to enable subcarrier alignment between a 6GR channel and an NR channel.

This does not necessarily mean that the 6GR raster must be aligned with the 100 kHz raster.

## BS RF

The BS will need to be able to transmit 6GR and 5G within the same carrier. This needs consistency between the numerologies and slot formats. The BS requirements should also be consistent and facilitate a multi-standard 6G/5G BS. The requirements may not be exactly the same, since 6G requirements should facilitate improved performance in the same RAT, but they should be compatible.

When designing BS RF requirements, take into account that compatibility between 6GR and legacy requirements is needed to facilitate multi-standard BS that can handle both 6GR and legacy RATs.

## Demodulation

For 5G, DSS demodulation requirements took into account puncturing of PDSCH due to LTE CRS. There may potentially be a need for 6G MRSS related demodulation requirements, however more details on the design of SSB, RS and MRSS are required in order to decide on the need for demodulation requirements for MRSS.

Consider demodulation requirements for MRSS once the RAN1/2 design is clearer.

## RRM

To achieve a smooth migration from 5G NR to 6GR, a multi-RAT spectrum sharing (MRSS) should be supported from 6G Day-1. The 6GR can be employed in 5G spectrum dynamically. The dynamic MRSS scheme, allowing the scheduler to assign resources to 5G or 6G users based on the traffic load dynamically.

In legacy NR, dynamic sharing with 4G technologies is in general more cumbersome given the relatively large amount of always-on signals in 4G. Especially, in that time, RAN4 only focused on the traditional inter-RAT handover requirement. In legacy NR, when UE is staying in a poor NR coverage, NW requires UE to perform inter-RAT EURAN handover to a EUTRAN coverage cell. The inter-RAT measurement is a gap-based measurement, and the interruption requirement of the total handover is too long in Rel-15. In a very late Rel-18, RAN4 first introduced a gapless measurement for DSS scenario.

To allow a dynamic sharing of resources between 5G and 6G in MRSS, a similar signal and procedure design should be introduced. For example, hiding a 6G-SSB can use the reserved resource mechanism defined in 5G as part of the forward-compatibility design. In other words, UE can seamless complete the 5G and 6GR sync and mobility procedures within the same time period.

In last meeting, RAN4 made the following way forward. Different with legacy inter-RAT handover, a possible new type of MRSS-based handover or carrier switching and measurement procedure should be investigated in RAN4. For example, the 5G or 6G resources in a cell may be dynamically assigned to the UE according to different traffic demands. The related NR measurement in a MRSS cell may be a gapless measurement and the MRSS-based handover interruption should be also minimized compared with a traditional inter-RAT handover. To avoid introducing the MRSS related mobility and measurement requirement in a late stage, RAN4 shall study the MRSS-based handover and measurement requirement in Day-1.

#### Issue 1-3-1: Inter-RAT handover interruption - Proposal from companies - Option 1 (CMCC): it is proposed to minimize handover interruption time for inter-RAT handover between NR and 6GR. - Way forward - Discuss whether and how to minimize handover interruption time for inter-RAT handover between NR and 6GR #### Issue 1-3-2: Inter-RAT measurement - Proposal from companies - Option 1 (CMCC): it is proposed to support inter-RAT measurements without gaps, including inter-RAT NR measurement without gap and inter-RAT 6GR measurement without gap, from 6G day-1. - Option 2 (Xiaomi): RAN4 shall study potential inter-RAT RRM measurement impact including w/o and with gap under MRSS scenario. - Way forward - Further discuss

- RAN4 should study inter-RAT handover scenarios with MRSS deployment.

- RAN4 should study inter-RAT measurements without gaps in different scenarios with MRSS deployment.

# Conclusion

Based on the discussion in the previous sections, we made the following observations:

Observation 1	A single sub-carrier spacing configuration per operating band or frequency range would simplify specification of the channel arrangement, channel/carrier spacing, spectrum utilization and guard bands for 6GR and specification of MRSS.

Observation 2	The existing NR synchronisation raster does not allow location of an NR carrier at every possible 10 kHz raster entry of the enhanced channel raster.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It is proposed to study 6G-5G MRSS in FR2-1 in RAN4.

Proposal 2	It is proposed not to consider MRSS with NTN and TN in the 6G study at this stage.

Proposal 3	Given a synchronisation raster, consider a channel raster that allows 6GR compatibility with adjacent legacy channels on the 100 kHz raster, while taking into account the MRSS considerations.

Proposal 4	For spectrum sharing (MRSS), the channel raster for 6GR is designed to enable subcarrier alignment between a 6GR channel and an NR channel.

Proposal 1	When designing BS RF requirements, take into account that compatibility between 6GR and legacy requirements is needed to facilitate multi-standard BS that can handle both 6GR and legacy RATs.

Proposal 2	Consider demodulation requirements for MRSS once the RAN1/2 design is clearer.

Proposal 5	RAN4 should study inter-RAT handover scenarios with MRSS deployment.

Proposal 6	RAN4 should study inter-RAT measurements without gaps in different scenarios with MRSS deployment.

# References

- R4-2514646, WF for [116bis][108] 6G spectrum sharing, Moderator (CMCC).