3GPP TSG RAN WG4 Meeting #117		R4-2521608

Dallas, US, 17th – 21st November 2025

Agenda item:	10.7

Source:	Vodafone

Title:	Enabling ETSI requirements for NTN in S-band

WI/SI:	NR_NTN_solutions-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The NR access technology for non-terrestrial networks (NTN) was introduced in Release-17 [1]. Correspondingly, frequency bands n255 (L-band) and n256 (S-band) were defined and added to the core specifications. Following Release-17, additional NTN L-bands were introduced (e.g. n250, n251, n253). However, the ETSI in-band and out-of-band emission requirements for both the S-band and L-bands were not captured in the RAN4 specifications, even though such limits are already defined in relevant ETSI standards. In the ETSI region, it is essential that RAN4 specifications reflect these ETSI requirements to enable operators to use the allocated satellite spectrum under harmonised technical conditions. During the RAN4#114 meeting, it was agreed to start capturing the existing ETSI requirements for the L-bands [2].

In this contribution, we propose to initiate corresponding work for the S-band to capture the existing ETSI in-band and out-of-band emission requirements and ensure alignment between RAN4 and ETSI deliverables.

# 2 Discussion

The introduction of NR access technology for NTN in Release 17 enabled operation over satellite frequency allocations such as the S-band, which corresponds to the Mobile-Satellite Service (MSS) S-band allocation used in the ETSI region and many CEPT countries. For a band to be fully deployable in practice, the 3GPP core specifications must include a complete set of emission requirements that are consistent with the regulatory framework of the regions where the band is authorised.

In the case of the S-band, such technical limits are already defined in the Harmonised European standards ETSI EN 301 442 and in ETSI EN 302 574-3. These standards describe both in-band emission masks and out-of-band emission limits. At present, these ETSI-defined S-band emission requirements are not reflected in the 3GPP RAN4 specifications.

The in-band emission masks can be found in sub-clause 4.2.2 of ETSI EN 301442 and sub-clause 4.2.3 of ETSI EN 302574-3, as shown in Figure 2.1 and 2.2 respectively.

Figure 2.1: In-band emission masks defined in ETSI EN 301442 for a device operating in 1980.1-2009.9 MHz, 1978.1-1980.1 MHz and 2009.9-2011.9 MHz

Figure 2.2: In-band emission masks defined in ETSI EN 302574-3 for S-band.

The out-of-band requirements can be found in sub-clause 4.2.1.2 of ETSI EN 301442, sub-clause 4.2.2.2 of ETSI EN 302574-3 and sub-clause 8.2.3.1.2 of 302574-2 as shown in Figures 2.3, 2.4 and 2.5 respectively.

Figure 2.3: Out-of-band emission requirements defined in ETSI EN 301442 for a device operating in 1980.1- 2009.9 MHz

Figure 2.4: Out-of-band emission requirements defined in ETSI EN 302574-3 for a device operating in 1980-2010MHz (left: for frequencies which are 2,5 MHz or greater away from the UE centre carrier frequency; right: for frequencies which are less than 2,5 MHz away from the UE centre carrier frequency).

Figure 2.5: Out-of-band emission requirements defined in ETSI EN 302574-2 for a device operating in ≤ 3GHz.

To provide context, there are five sets for in-band requirements and four sets for out-of-band requirements. These requirements are associated with the carrier (TDMA, CDMA, carrier-on/off state), and result in emission masks with slightly different shapes. However, these masks differ from the general emission mask currently used in RAN4 for NTN S-band.

Inclusion of emission requirements for S-band (in- and out-of-band) would provide the necessary alignment between 3GPP and ETSI deliverables. Such alignment is essential for operators and UE vendors to deploy NTN services using the S-band under harmonised technical conditions accepted by regional regulators.

Proposal 1:	Capture the existing ETSI's in-band and out-of-band emission requirements for the S-band.

Proposal 2:	Capture the existing in-band and out-of-band emission requirements for the S-band as defined in ETSI EN 301442 and check if the requirements in ETSI EN 302574-2/3 should be included as well.

# 3	Conclusions

In this contribution, we have presented our technical input on the existing ETSI regulations for a device in S-band.

Proposal 1:	Capture the existing ETSI's in-band and out-of-band emission requirements for the S-band.

Proposal 2:	Capture the existing in-band and out-of-band emission requirements for the S-band as defined in ETSI EN 301442 and check if the requirements in ETSI EN 302574-2/3 should be included as well.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- R4-2502850, " WF on NR and IoT NTN bands", Aalyria