3GPP TSG-RAN WG4 Meeting #117	R4-2521635

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on performance requirements for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	6.13.3

Document for:	Discussion

Introduction

RRM performance requirements for SBFD are discussed in RAN4#116-bis, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Accuracy requirements

- Test cases

In this paper, we will provide our views on remaining issues in RRM performance requirements in SBFD.

Discussion

Accuracy requirements

L1-SRS-RSRP

Issue 1-1-1: Time offset for L1-SRS-RSRP measurements - Agreement - Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for [15kHz SCS and] 30kHz SCS and without cell phase error for other SCS

In RAN4#116-bis, some companies mentioned that all existing commercial TDD NWs in FR1 are using 30kHz SCS, so there is no need to define accuracy requirements for 15kHz SCS. We have a different view.

- The requirements should be generic because we cannot preclude the possibility that some TDD NWs may use 15kHz SCS in future. This is also the reason why in existing accuracy requirements, all SCS-es are considered without considering whether there is commercial deployment or not.

- 15kHz SCS for TDD is one of the test configurations used in most existing RRM test cases.

- There is not much effort to define requirements for 15kHz SCS as the simulation results are ready.

Proposal 1: Define L1-SRS-RSRP accuracy requirements for all SCS-es.

L1-RSRP and L1-SINR

Issue 2-1-1: Measurement accuracy for CSI-RS based L1 measurements - Way forward - Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB) - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), - for a UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB apply. - Option 2: accuracy requirements based on 24 RB apply. - for UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB apply

In [2] we provide our simulation results for 24 RB, and we observe that the performance difference between 24 RB and 48 RB is around 0.5dB, and for some configuration of L1-SINR there is even no degradation. Therefore, we suggest to define the accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB).

Proposal 2: Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB).

On the applicability of the existing and relaxed accuracy requirements for case 3, for the case of UE not indicating CSI processing time scaled by 2, the relaxed accuracy based on 24 RB should apply as UE may only measure 1 DL subband. For the case of UE indicating CSI processing time scaled by 2, we understand it depends on UE implementation, e.g. whether L1 is measured in the same way as CQI. As SBFD is a new feature, we prefer option 1 for better performance.

Proposal 3: For DUD case 3, when UE indicates CSI processing time scaled by 2, the existing accuracy requirements based on 48 RB apply.

In RAN4#116-bis, the CBW and subband configuration for FR2 test was discussed. One issue identified is that with 100MHz CBW, the min BW for UL subband is 50MHz, so the max BW for DL subband is 50MHz. This means even for DU case, there is only 32 RB in DL subband which is less than 48 RB, and the existing accuracy cannot apply. So far, the relaxed accuracy based on 24 RB is only considered for DUD case 3. Considering above, we suggest to extend the relaxed accuracy to DU case.

Proposal 4: The relaxed accuracy requirements based on 24 RB also apply for DU when the CSI-RS BW in the DL subband is less than 48 RB.

Test cases

CBW and subband configuration for FR2

Issue 3-1-2: Test configurations - Agreements - FR1: NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex - FR2: - Option 1: NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode - Option 2: NR 120 kHz SCS, 400 MHz bandwidth, TDD duplex mode - TDD UL/DL configuration : DDDSU - Time domain configuration of SBFD slots/symbols: DXXXU - Frequency domain configuration of SBFD subbands for FR1 - DU: 80 MHz – 20 MHz - DUD: 40 MHz – 20 MHz – 40 MHz - FFS: Frequency domain configuration of SBFD subbands for FR2

In RAN4#116-bis, the CBW and subband configuration for FR2 test was discussed. On one hand, come companies mentioned that most commercial NWs for FR2 are using 100MHz CBW; on the other hand, it is very difficult to support DUD with 100MHz CBW – considering the min BW for UL subband is 50MHz, it will leave 16 RB for each DL subband which is not a very meaningful configuration and it is below the side condition for most RRM requirements.

To move forward, we suggest to define the following 2 test configurations for FR2:

- DU: 100MHz with 50MHz – 50MHz split

- DUD: 200MHz with 75MHz – 50MHz – 75MHz split

In our view, it is too restrictive to preclude DUD in FR2, and based on our understanding 200MHz CBW is also widely supported in FR2 BS and UE products.

Proposal 5: Adopt the following test configuration for FR2.

- DU: 100MHz with 50MHz – 50MHz split

- DUD: 200MHz with 75MHz – 50MHz – 75MHz split

Open test cases

9-10; FFS:Radio Link Monitoring Out-of-sync Test for FR1/FR2 PCell configured with CSI-RS-based RLM in SBFD; A.6.5.1.x A.7.5.1.x; CSI-RS based RLM, collision with UL; FFS: DU/DUD Non-DRX All CSI-RS on SBFD symbols UL scheduling hit some CSI-RS occasions. 11-12; Radio Link Monitoring In-sync Test for FR1/FR2 PCell configured with CSI-RS-based RLM in SBFD; A.6.5.1.y A.7.5.1.y; CSI-RS based RLM, collision with UL; DU DRX All CSI-RS on SBFD symbols UL scheduling hit some CSI-RS occasions. 13-14; Beam Failure Detection and Link Recovery Test for FR1/FR2 PCell configured with CSI-RS-based BFD and LR in SBFD; A.6.5.5.x A.7.5.5.x; CSI-RS based BFR, puncturing in frequency domain; DUD Non-DRX All CSI-RS on SBFD symbols, CSI-RS BW is same as CBW.

Based on the test case list in [1], TC9-10 is FFS and some companies suggested to merge the test case with TC13-14. The test purpose for TC9-10 is to check whether UE can correctly handle the collision with UL. In our view, it can be indeed combined with TC13-14, if in TC13-14 we have UL scheduling hit some CSI-RS occasions. The RLM OOS procedure is similar to BFD, and the impact of SBFD is same to the 2 procedures, so there is little loss on the test coverage.

Proposal 6: Remove TC9-10 and verify “collision with UL” in TC13-14.

- Further discuss whether to define the following test-cases: - CSI-RS L1-SINR measurement (delay and accuracy)

There was another discussion on whether to define test cases for CSI-RS based L1-SINR measurement, including both delay and accuracy. We support to define test cases for delay part for SSB based CMR and CSI-RS based IMR.

- As commented by some companies, for L1-SINR UE needs to handle CMR and IMR separately, and this is not verified in L1-RSRP test cases.

- We do not have test case to check whether UE can correctly handle the collision with invalid symbol type with DRX. The non-DRX case is verified in L1-RSRP delay test cases, and we can use L1-SINR test cases to verify DRX case.

Proposal 7: Define TC19-20 for L1-SINR delay as follows.

| 19-20 | CSI-RS based L1-RSRP measurement in SBFD FR1/FR2 | A.6.6.8.x A.7.6.6.x | L1-SINR delay, SSB as CMR and CSI-RS as IMR, distribution in two symbol types | DU DRX CSI-RS has occasions on both SBFD and non-SBFD symbols with different RSRP level |
| --- | --- | --- | --- | --- |

There was another discussion on whether to define test cases for CSI-RS based L1-SINR measurement, including both delay and accuracy. We support to define test cases for delay part for SSB based CMR and CSI-RS based IMR.

UL scheduling pattern

One issue raised up during CR drafting is the UL scheduling pattern, i.e. which CSI-RS resource occasions are hit by the UL scheduling during the test. It will impact TC11-12 for RLM IS and TC13-14 for BFD.

Taking RLM IS as example, the IS evaluation period is 10*N DRX cycles without SBFD, and in TC10-11, a number of DRX cycles need to be “hit”, i.e. UE is scheduled to transmit UL in the same OFDM symbols as CSI-RS resource occasion in on-duration. RAN4 needs to discuss which DRX cycles are hit.

In our view, there are 2 options

- Option 1: pre-defined UL scheduling pattern

- Option 2: random UL scheduling pattern

In option 1, RAN4 needs to define a scheduling pattern, e.g. with 10-bit bitmap of 1010101010, every other DRX cycle is hit. It is simple for TE implementation but the pattern is known to UE.

In option 2, RAN4 needs to define a probability of hitting and a maximum number of hitting, e.g. each DRX cycle is hit with 30% probability and at maximum 5 DRX cycles are hit. The maximum number is needed because the test time (T4 in TC11-12) needs to be fixed. This option is more complicated for spec and TE implementation, but the pattern is not known to UE.

We do not have strong view but slightly prefer option 1 which simple. Also, in real world UE will gets the UL scheduling information before the measurement occasion, so we do not see additional test coverage from option 2 because it is only verifying whether UE can correctly decode UL grant or not.

Proposal 8: RAN4 to define a pre-defined UL scheduling pattern for TC11-12 for RLM IS and TC13-14 for BFD.

Conclusions

In this paper we provided our views on remaining issues in RRM performance requirements in SBFD.

Proposal 1: Define L1-SRS-RSRP accuracy requirements for all SCS-es.

Proposal 2: Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding 0.5dB relaxation on top of existing accuracy (based on 48 RB).

Proposal 3: For DUD case 3, when UE indicates CSI processing time scaled by 2, the existing accuracy requirements based on 48 RB apply.

Proposal 4: The relaxed accuracy requirements based on 24 RB also apply for DU when the CSI-RS BW in the DL subband is less than 48 RB.

Proposal 5: Adopt the following test configuration for FR2.

- DU: 100MHz with 50MHz – 50MHz split

- DUD: 200MHz with 75MHz – 50MHz – 75MHz split

Proposal 6: Remove TC9-10 and verify “collision with UL” in TC13-14.

Proposal 7: Define TC19-20 for L1-SINR delay as follows.

| 19-20 | CSI-RS based L1-RSRP measurement in SBFD FR1/FR2 | A.6.6.8.x A.7.6.6.x | L1-SINR delay, SSB as CMR and CSI-RS as IMR, distribution in two symbol types | DU DRX CSI-RS has occasions on both SBFD and non-SBFD symbols with different RSRP level |
| --- | --- | --- | --- | --- |

Proposal 8: RAN4 to define a pre-defined UL scheduling pattern for TC11-12 for RLM IS and TC13-14 for BFD.

Reference

- R4-2515166, WF for [116bis][309] Rel-19 RRM performance_Part1, Moderator (Qualcomm)

- R4-251xxxx, Simulation results for L1-RSRP and L1-SINR accuracy, Huawei, HiSilicon