3GPP TSG-RAN WG4 Meeting #117	R4-2521640

Dallas, USA, 17 – 21 November, 2025

Title: 	Discussion on test case for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	6.16.2

Document for:	Discussion

Introduction

RRM test cases for LP-WUR are discussed in RAN4#116-bis, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Test cases for entry condition

- Test cases for higher priority carrier

- Test cases for LP-WUS monitoring

In this paper, we will provide our views on remaining issues in RRM test cases for LP-WUR.

Discussion

Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3 Ad hoc meeting agreement: RAN4 to define at least test cases for exit conditions. FFS on whether to define test cases for evaluation of entry conditions. Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3 Continue discussion on whether to introduce a test case to verify when conditions for a particular state is not met, the UE should not enter into this particular state. Note: Candidate states includes LP-WUR monitoring, serving cell measurement offloading and RRM relaxation.

In RAN4#116-bis it was agreed to define test cases to verify whether UE correctly evaluates exit conditions, and it was open about entry condition. Our view is that RAN4 should define 1 test case for entry condition.

It is not meaningful to verify UE correctly enters LR mode when the entry condition is met because using LR is not mandatory. However, in real world, if UE mistakenly enters LR mode when the entry condition is not met, it may take time for UE to exit, and during this time UE may miss paging. Therefore, it is meaningful to verify that UE does not enter LR mode when the entry condition is not met.

Among the candidate LR modes, we suggest to use serving cell measurement offloading for the test. Using LP-WUS monitoring has some testability issue as we will discuss below, and using serving cell measurement offloading or RRM relaxation is same, and considering we already have 2 test cases for Case 3, it is better to define the new one for Case 1.

In the test, the signal levels should be set such that UE shall not enter serving cell measurement offloading mode, even considering the measurement accuracy. On the other hand, the signal levels should be set such that cell reselection shall be triggered also considering the measurement accuracy. A correct UE shall trigger cell reselection following legacy DRX based measurement delay.

Proposal 1: Define one test case to verify whether UE correctly evaluates entry condition for serving cell measurement offloading mode.

Issue 3-2-7: Test case design for higher priority frequency layer search Continue discussion

In RAN4#116-bis there were different views on whether to define test cases for higher priority frequency layer search. Technically, it is meaningful to verify whether UE correctly performs higher priority frequency layer search when in offloading mode, but we can see the concern on test time, which needs to be 1 hour. As the requirement is very relaxed and test time is very long, we prefer to not have this test case.

Proposal 2: Do not define test case for higher priority frequency layer search.

Issue 3-2-6: Test case design for LP-WUS monitoring Continue discussion

In RAN4#116-bis there were different views on whether to define test cases for LP-WUS monitoring. We do not support to define this test case.

- The test case, if defined, is to verify whether UE correctly 1) monitors PO after receiving LP-WUS and 2) wakes up within the wake-up delay reported via capability. 1) can be verified via demod test or RAN5 signaling test, and 2) can be verified via TC1.

- There is testability issue – although TE can send paging at the PO associated with the LP-WUS occasion, it is difficult to verify whether UE is correctly monitoring PO or not because there is no requirement on when UE should start RA after receiving paging.

Proposal 3: Do not define test case for LP-WUS monitoring.

Conclusions

In this paper we provided our views on remaining issues in RRM test cases for LP-WUR.

Proposal 1: Define one test case to verify whether UE correctly evaluates entry condition for serving cell measurement offloading mode.

Proposal 2: Do not define test case for higher priority frequency layer search.

Proposal 3: Do not define test case for LP-WUS monitoring.

Reference

- R4-2514811, WF on NR_LPWUS_RRM, Moderator (vivo)