3GPP TSG-RAN4 Meeting #117	R4-2521654

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | xxxx | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on measurement test cases for IoT NTN TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD-Perf |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on WF R4-2515055, RRM test cases for measurement of neighbor cells need to be defined for IoT NTN TDD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define RRM test cases for measurement of neighbor cells for IoT NTN TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed test cases are missing, and UE performance for measurement of neighbor cells for IoT NTN TDD is not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.13.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## A.13.5	UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access

### A.13.5.1	HD-FDD and IoT NTN TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

#### A.13.5.1.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT intra-frequency neighbour cell measurement requirement in clause 8.14A.6.3 and 8.14B.6.3 is met.

The test parameters are given in table A.13.5.1.1-1, table A.13.5.1.1-2 and table A.13.5.1.1-3 below. nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the same frequency carrier. The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively.

Table A.13.5.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| 3 | NGSO, TDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.5.1.1-2: General test parameters for HD-FDD and TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  |  | standalone |  |
| Initial condition | Active cell |  |  | nCell1 |  |
|  | Neighbour cells |  |  | nCell2 |  |
| Final condition | Active cell |  |  | nCell2 |  |
| Satellite information | Config 1 |  |  | SSC.1 |  |
|  | Config 2, 3 |  |  | SSC.2 |  |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| NPRACH Configuration |  | Config 1, 2 |  | NPRACH.R-1 | Refer to A.3.18 |
|  |  | Config 3 |  | NPRACH.R-X |  |
| NPDCCH repetition level |  |  |  | 8 | NPDCCH Rmax |
| N310 |  |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311-v13xy |  |  | ms | 15000 | RRC re-establishment timer |
| DRX |  |  |  | OFF |  |
| T1 |  |  | ms | 5000 |  |
| T2 |  |  | ms | 900 |  |
| T3 |  | Config 1, 2 | ms | 3100 |  |
|  |  | Config 3 |  | 2800 |  |
| T4 |  |  | ms | 500 |  |
| T5 |  | Config 1, 2 | ms | 8520 |  |
|  |  | Config 3 |  | TBD |  |
| s-MeasureIntra |  |  | dBm | -95 |  |
| s-MeasureDeltaP |  |  | dB | 6 |  |
| t-MeasureDeltaP |  |  | s | 60 |  |

Table A.13.5.1.1-3: General test parameters for HD-FDD and TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage

| Parameter |  | Unit | nCell 1 |  |  |  |  | nCell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| BWchannel |  | kHz | 200 |  |  |  |  | 200 |  |  |  |  |
| NPDSCH parameters | Config 1, 2 |  | R.18 HD-FDD |  |  |  |  | R.18 HD-FDD |  |  |  |  |
|  | Config 3 |  | R.X TDD |  |  |  |  | R.X TDD |  |  |  |  |
| NPDCCH parameters |  |  | R.30 HD-FDD |  |  |  |  | R.30 HD-FDD |  |  |  |  |
| NOCNG Patterns | Config 1, 2 |  | NOP.3 FDD |  |  |  |  | NOP.3 FDD |  |  |  |  |
|  | Config 3 |  | NOP.X TDD |  |  |  |  | NOP.X TDD |  |  |  |  |
| NPBCH_RA |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| NPBCH_RB |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPSS_RA |  | dB |  |  |  |  |  |  |  |  |  |  |
| NSSS_RA |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB |  | dB |  |  |  |  |  |  |  |  |  |  |
| NOCNG_RANote 1 |  | dB |  |  |  |  |  |  |  |  |  |  |
| NOCNG_RBNote 1 |  | dB |  |  |  |  |  |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  |  | dB | 9 | -3 | -8.5 | -Infinity | -Infinity | -Infinity | -Infinity | 2.2 | 4 | 4 |
| Note2 |  | dB | 9 | -3 | -8.5 | -Infinity | -Infinity | -Infinity | -Infinity | 2.2 | 4 | 4 |
| NRSRP Note2 |  | dBm/15 kHz | -89 | -101 | -101 | -Infinity | -Infinity | -Infinity | -Infinity | -94 | -94 | -94 |
| Propagation Condition |  |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Antenna Configuration |  |  | 1x1 |  |  |  |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 |  | ms | - |  |  |  |  | 3 |  |  |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.13.5.1.2	Test Requirements

UE shall trigger RLF during T4 and complete neighbour cell measurement before end of T4. UE shall start to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2 before the end of T5 to fulfil the RRC re-establishment delay to a known NB-IoT FDD intra frequency cell.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 0 ms

-	TSI_NB-IoT = 8320 and TBD ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD and TDD cell, respectively.

-	TPRACH_NB-IoT = 80 and TBD ms; it is the additional delay caused by the random access procedure for the target NB-IoT FDD and TDD cell, respectively.

### A.13.5.2	HD-FDD and IoT NTN TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

#### A.13.5.2.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT inter-frequency neighbour cell measurement requirement in clause 8.14A.6.3 and 8.14B.6.3 is met.

The test parameters are given in table A.13.5.2.1-1, table A.13.5.2.1-2, table A.13.5.2.1-3 and table A.13.5.2.1-4 below. nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the different frequency carriers. The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively.

Table A.13.5.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| 3 | NGSO, TDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.5.2.1-2: General test parameters for HD-FDD and TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  |  | standalone |  |
| Initial condition | Active cell |  |  | nCell1 |  |
|  | Neighbour cells |  |  | nCell2 |  |
| Final condition | Active cell |  |  | nCell2 |  |
| Satellite information | Config 1 |  |  | SSC.1 |  |
|  | Config 2 |  |  | SSC.2 |  |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| NPRACH Configuration |  | Config 1, 2 |  | NPRACH.R-1 | Refer to A.3.18 |
|  |  | Config 3 |  | NPRACH.R-X |  |
| NPDCCH repetition level |  |  |  | 8 | NPDCCH Rmax |
| N310 |  |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311-v13xy |  |  | ms | 15000 | RRC re-establishment timer |
| DRX |  |  | ms | OFF |  |
| T1 |  |  | ms | 5000 |  |
| T2 |  |  | ms | 1300 |  |
| T3 |  | Config 1, 2 | ms | 8500 |  |
|  |  | Config 3 |  | 8200 |  |
| T4 |  |  | ms | 5200 |  |
| T5 |  | Config 1, 2 | ms | 8520 |  |
|  |  | Config 3 |  | TBD |  |
| s-MeasureInter |  |  | dBm | -95 |  |
| s-MeasureDeltaP |  |  | dB | 6 |  |
| t-MeasureDeltaP |  |  | s | 60 |  |

Table A.13.5.2.1-3: General test parameters for HD-FDD and TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Parameter |  | Unit | nCell 1 |  |  |  |  | nCell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| BWchannel |  | kHz | 200 |  |  |  |  | 200 |  |  |  |  |
| NPDSCH parameters | Config 1, 2 |  | R.18 HD-FDD |  |  |  |  | R.18 HD-FDD |  |  |  |  |
|  | Config 3 |  | R.X TDD |  |  |  |  | R.X TDD |  |  |  |  |
| NPDCCH parameters |  |  | R.30 HD-FDD |  |  |  |  | R.30 HD-FDD |  |  |  |  |
| NOCNG Patterns | Config 1, 2 |  | NOP.3 FDD |  |  |  |  | NOP.3 FDD |  |  |  |  |
|  | Config 3 |  | NOP.X TDD |  |  |  |  | NOP.X TDD |  |  |  |  |
| NPBCH_RA |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| NPBCH_RB |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPSS_RA |  | dB |  |  |  |  |  |  |  |  |  |  |
| NSSS_RA |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA |  | dB |  |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB |  | dB |  |  |  |  |  |  |  |  |  |  |
| NOCNG_RANote 1 |  | dB |  |  |  |  |  |  |  |  |  |  |
| NOCNG_RBNote 1 |  | dB |  |  |  |  |  |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  |  | dB | 9 | -3 | -3 | -Infinity | -Infinity | -Infinity | -Infinity | 4 | 4 | 4 |
| Note2 |  | dB | 9 | -3 | -3 | -Infinity | -Infinity | -Infinity | -Infinity | 4 | 4 | 4 |
| NRSRP Note2 |  | dBm/15 kHz | -89 | -101 | -101 | -Infinity | -Infinity | -Infinity | -Infinity | -94 | -94 | -94 |
| Propagation Condition |  |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Antenna Configuration |  |  | 1x1 |  |  |  |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 |  | ms | - |  |  |  |  | 3 |  |  |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.13.5.2.1-4: DRX-Configuration for HD-FDD and TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

#### A.13.5.2.2	 Test Requirements

UE shall trigger RLF during T4 and complete neighbour cell measurement before end of T4. UE shall start to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2 before the end of T5 to fulfil the RRC re-establishment delay to a known NB-IoT FDD inter frequency cell.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 0 ms

-	TSI_NB-IoT = 8320 and TBD ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD and TDD cell, respectively.

-	TPRACH_NB-IoT = 80 and TBD ms; it is the additional delay caused by the random access procedure for the target NB-IoT FDD and TDD cell, respectively.

### A.13.5.3	HD-FDD and IoT NTN TDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

#### A.13.5.3.1	Test Purpose and Environment

The purpose is to verify that the NB-IoT intra-frequency neighbour cell measurement requirement in clause 8.14A.6.3 and 8.14B.6.3 is met.

The test parameters are given in table A.13.5.3.1-1, table A.13.5.3.1-2 and table A.13.5.3.1-3 below. nCell1 and nCell2 are NB-IoT cells with different physical cell ID on the same frequency carrier. The test consists of 4 successive time periods, with time duration of T1, T2, T3, T4 respectively.

At the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SystemInformationBlockType31-NB of nCell 1 is exceeded by the configured value in distanceThresh plus 50m.

Table A.13.5.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| 3 | NGSO, TDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.13.5.3.1-2: General test parameters for HD-FDD and TDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| NB-IOT operational mode |  |  |  | standalone |  |
| Initial condition | Active cell |  |  | nCell1 |  |
|  | Neighbour cells |  |  | nCell2 |  |
| Final condition | Active cell |  |  | nCell2 |  |
| Satellite information | Config 1 |  |  | SSC.1 |  |
|  | Config 2 |  |  | SSC.2 |  |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| NPRACH Configuration |  | Config 1, 2 |  | NPRACH.R-1 | Refer to A.3.18 |
|  |  | Config 3 |  | NPRACH.R-X |  |
| NPDCCH repetition level |  |  |  | 8 | NPDCCH Rmax |
| N310 |  |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311-v13xy |  |  | ms | 15000 | RRC re-establishment timer |
| DRX |  |  |  | OFF |  |
| T1 |  |  | ms | 5000 |  |
| T2 |  | Config 1, 2 | ms | 3100 |  |
|  |  | Config 3 |  | 2800 |  |
| T3 |  |  | ms | 500 |  |
| T4 |  | Config 1, 2 | ms | 8520 |  |
|  |  | Config 3 |  | TBD |  |
| s-MeasureIntra |  |  | dBm | -95 |  |
| s-MeasureDeltaP |  |  | dB | 6 |  |
| t-MeasureDeltaP |  |  | s | 60 |  |

Table A.13.5.3.1-3: General test parameters for HD-FDD and TDD location-based Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage

| Parameter |  | Unit | nCell 1 |  |  |  | nCell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| BWchannel |  | kHz | 200 |  |  |  | 200 |  |  |  |
| NPDSCH parameters | Config 1, 2 |  | R.18 HD-FDD |  |  |  | R.18 HD-FDD |  |  |  |
|  | Config 3 |  | R.X TDD |  |  |  | R.X TDD |  |  |  |
| NPDCCH parameters |  |  | R.30 HD-FDD |  |  |  | R.30 HD-FDD |  |  |  |
| NOCNG Patterns | Config 1, 2 |  | NOP.3 FDD |  |  |  | NOP.3 FDD |  |  |  |
|  | Config 3 |  | NOP.X TDD |  |  |  | NOP.X TDD |  |  |  |
| NPBCH_RA |  | dB | 0 |  |  |  | 0 |  |  |  |
| NPBCH_RB |  | dB |  |  |  |  |  |  |  |  |
| NPSS_RA |  | dB |  |  |  |  |  |  |  |  |
| NSSS_RA |  | dB |  |  |  |  |  |  |  |  |
| NPDCCH_RA |  | dB |  |  |  |  |  |  |  |  |
| NPDCCH_RB |  | dB |  |  |  |  |  |  |  |  |
| NPDSCH_RA |  | dB |  |  |  |  |  |  |  |  |
| NPDSCH_RB |  | dB |  |  |  |  |  |  |  |  |
| NOCNG_RANote 1 |  | dB |  |  |  |  |  |  |  |  |
| NOCNG_RBNote 1 |  | dB |  |  |  |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |
|  |  | dB | 9 | 3.5 | -Infinity | -Infinity | -Infinity | -5.5 | 4 | 4 |
| Note2 |  | dB | 9 | 3.5 | -Infinity | -Infinity | -Infinity | -5.5 | 4 | 4 |
| NRSRP Note2 |  | dBm/15 kHz | -89 | -89 | -Infinity | -Infinity | -Infinity | -94 | -94 | -94 |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  |  | 1x1 |  |  |  | 1x1 |  |  |  |
| Timing offset to nCell 1 |  | ms | - |  |  |  | 3 |  |  |  |
| Note 1: NOCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Es/Iot and NRSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.13.5.3.2	Test Requirements

UE shall trigger RLF during T3 and complete neighbour cell measurement before end of T3. UE shall start to send NPRACH preambles to cell 2 for sending the RRCConnectionReestablishmentRequest message to cell 2 before the end of T4 to fulfil the RRC re-establishment delay to a known NB-IoT FDD intra frequency cell.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE-re-establish_delay_NB-IoT.

Where:

-	TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The NPRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

-	TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB-IoT + TSI_NB-IoT + TPRACH_NB-IoT

-	NNB-Iot-freq = 1

-	Tsearch_NB-IoT = 0 ms

-	TSI_NB-IoT = 8320 and TBD ms; it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target NB-IoT FDD and TDD cell, respectively.

-	TPRACH_NB-IoT = 80 and TBD ms; it is the additional delay caused by the random access procedure for the target NB-IoT FDD and TDD cell, respectively.

<End of Change 1>