3GPP TSG-RAN WG4 Meeting #117                                        R4-2521658 Dallas, USA, November 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On IoT-NTN PC1.5 single Tx

Agenda Item:	7.6.3

Document for:	Approval

Introduction

According to the Rel-20 WID on enhanced requirements for NR NTN and IoT NTN Phase 2 [1], the objective related to NTN HPUE is reproduced here.

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

A fruitful discussion with outcomes as captured in [2] happened in RAN4#116bis meeting. Some agreements are reproduced here.

- Issue: UE RF front end assumptions and related accommodations WayFoward: - FFS on power handling challenges when NB-IoT NTN UE is scheduled for long duration e.g. 256ms in UL with high power e.g. +29dBm. (e.g. potential accommodations to UE maximum output power requirements) - The timeline of the PC1.5 IoT NTN work shall not be impacted by this issue, even if no solution is agreed, no discussion after RAN4#118. - Issue: MPR requirements for PC1.5 NB-IoT NTN UE Agreement: - Take the following tables as starting point for the MPR for PC1.5 NB-IoT NTN UE (Based on MPR = maximum {0, PC1 MPR - 2dB}) Table 2-2-6-1 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone Modulation; π/2 BPSK, π/4 QPSK; π/2 BPSK, π/4 QPSK. SCS; 15 kHz; 3.75 kHz. Tone positions for 1 Tone allocation; 0-1; 2-9; 10-11; 0-5; 6-41; 42-47. MPR; ≤ [0] dB; [0] dB; ≤ [0] dB; ≤ [1.5] dB; [0] dB; ≤ [1.5] dB. Note 1: For single-tone allocation, the listed modulations and SCS are supported.. Table 2-2-6-2 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone Modulation; QPSK. SCS; 15 kHz. Tone positions for 3 Tones allocation; 0-2; 3-5 and 6-8; 9-11. MPR; ≤ [0] dB; ≤ [0] dB; ≤ [0] dB. Tone positions for 6 Tones allocation; 0-5 and 6-11. MPR; ≤ [0.5]dB; ≤ [0.5] dB. Tone positions for 12 Tones allocation; 0-11. MPR; ≤ [2] dB. Note 1: For multi-tone allocation, the listed modulation and SCS are supported.. - Interested companies can provide further evaluation in the next RAN4 meeting based on following assumptions to check whether modifications on the above two tables are needed - Option 1 - Calibration points for PC1.5 (based on NR PC2 PA): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss /or [5] dB - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 dBc / or [31] dBc - CIM3: 60 dBc - NR ACLR: 31dBc - Option 2 - DFT-s-OFDM 1RE (Single Tone) - Centre Single Tone [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK] - [3] or [4] dB post PA loss - 0dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value - Issue: A-MPR requirement for band 256 PC1.5 NB-IoT NTN UE Agreement: - Take the following as the starting point for the A-MPR for band 256 PC1.5 NB-IoT NTN UE with NS_24, depending on the final MPR value Table 2-2-7.1 Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5 Network Signalling value; Requirements (subclause); E-UTRA Band; A-MPR (dB). NS_24; 6.5B.4.4.3; 256; ≤ [8~8.5]. - Interested companies can provide further evaluation in the next RAN4 meeting based on the assumptions in issue 2-2-6, including whether the above A-MPR is applicable for all frequencies of band 256 or a subset of frequencies of band 256. - The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value

In this contribution, we would like to provide our further thinking regarding this topic.

Discussion

- 2.1 Further considerations on IoT-NTN PC1.5

One issue left for further study is the potential power handling challenge for a NB-IoT NTN UE under long period UL transmission approaching the rated power. To our understanding, the most concerning case might not be NB-IoT NTN. Firstly, unlike FDD mode, NB-IoT only requires half duplex operation. Secondly, the issue itself could be more serious as the output power ramps up, but there seems no dedicated scheme has been specified for PC1. Last but not the least, as it was also agreed in last meeting, P-MPR will be reused as a “back-up plan” for UE autonomous power handling. In conclusion, we think the current core requirements structure on output power for NB-IoT NTN UE needs no change for this issue.

Proposal 1: For the potential power handling challenge for a NB-IoT NTN UE under long period UL transmission approaching the rated power, the current core requirements structure on output power for NB-IoT NTN UE needs no change. E.g. the existing P-MPR mechanism can still be used.

Regarding the MPR/A-MPR evaluation, there are some discussions on using different assumption for post PA loss for re-evaluation. We understand the intention from the proponents can be leaving enough margin for existing RF implementation. Another approach is to do scaling based on existing requirements for PC1 and PC2, which is also a reasonable way since it can keep the consistency with the assumptions used in legacy measurements. Therefore we have the below proposal.

Proposal 2: Take the following tables into consideration for the MPR for PC1.5 NB-IoT NTN UE (scaling on existing requirements).

Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0] dB | [0] dB | ≤ [0] dB | ≤ [1.5] dB | [0] dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5]dB |  | ≤ [0.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Following the same logic, we support similar handling for A-MPR requirement for band 256.

Proposal 3: Take the following tables into consideration for the A-MPR for PC1.5 NB-IoT NTN UE for band 256 with NS_24 depending on the adopted MPR requirements.

Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ [8~8.5] |

Conclusion

In this contribution we share our further considerations on Rel-20 HPUE for NTN. We have following proposals:

Proposal 1: For the potential power handling challenge for a NB-IoT NTN UE under long period UL transmission approaching the rated power, the current core requirements structure on output power for NB-IoT NTN UE needs no change. E.g. the existing P-MPR mechanism can still be used.

Proposal 2: Take the following tables into consideration for the MPR for PC1.5 NB-IoT NTN UE (scaling on existing requirements).

Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0] dB | [0] dB | ≤ [0] dB | ≤ [1.5] dB | [0] dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5]dB |  | ≤ [0.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal 3: Take the following tables into consideration for the A-MPR for PC1.5 NB-IoT NTN UE for band 256 with NS_24 depending on the adopted MPR requirements.

Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ [8~8.5] |

- References

- RP-252958, “New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2”, Moderator (RAN4 Vice Chair), 3GPP TSG-RAN Meeting #109, Beijing, China, September 15-18, 2025.

- R4-2515157, “WF for [116bis][318] NR_IoT_NTN_Ph2”, Moderator (CHTTL), 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025.

- R4-2510268, “Big CR for 36.102 for IoT-NTN HPUE”, vivo, Apple, CATT, China Telecom, Echostar, Ericsson, Globalstar, Hisilicon, Huawei, Inmarsat, LGE, Mediatek, Nokia, OPPO, Qualcomm, Samsung, Skyworks, Thales, viasat, Xiaomi, ZTE, 3GPP TSG-RAN4 Meeting #116, Bengaluru, India, August 25-29, 2025.