3GPP TSG-RAN WG4 Meeting #117	R4-2521661

# Dallas, USA, Nov 17 – 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.141-1 conducted REFSENS for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-7 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce the testing on conducted REFSENS for SBFD-capable BS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test procedure related to conducted reference sensitivity is covered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing spec on the conducted REFSENS for SBFD-capable BS is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

x.2	Reference sensitivity level

x.2.1	Definition and applicability

The reference sensitivity power level is the minimum mean power received at the TAB connector for BS type 1-H inside the received SBFD UL subband for SBFD operation at which a throughput requirement shall be met for a specified reference measurement channel.

x.2.2	Minimum requirement

The minimum requirement for BS type 1-H is in TS 38.104 [2], clause 12.3.2.1.

x.2.3	Test purpose

To verify that for each BS type 1-H TAB connector at the reference sensitivity level the throughput requirement shall be met for a specified reference measurement channel.

x.2.4	Method of test

x.2.4.1	Initial conditions

Test environment:

-	Normal; see annex B.2.

-	Extreme, see annexes B.3 and B.5.

RF channels to be tested for single carrier: B, M and T; see clause 4.x.x.

Under extreme test environment, the test shall be performed on each of B, M and T under extreme power supply conditions as defined in annex B.5.

NOTE:	Tests under extreme power supply conditions also test extreme temperatures.

X.2.4.2	Procedure

The minimum requirement is applied to all connectors under test.

For BS type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the connector under test to measurement equipment as shown in annex D.4.1 for BS type 1-H.

2)	Set the BS to transmit a signal using the applicable test configuration and corresponding power setting specified in clauses 4.x using the corresponding test models or set of physical channels in clause 4.x.

3)	Start the signal generator for the wanted signal to transmit the Fixed Reference Channels for reference sensitivity according to annex A.1.

4)	Set the signal generator for the wanted signal power as specified in clause x.2.5.

5)	Measure the throughput according to annex A.1.

X.2.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table X.2.5-1 for Wide Area BS, in table X.2.5-2 for Medium Range BS and in table X.2.5-3 for Local Area BS.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.5-1a for Wide Area BS, in table 7.2.5-2a for Medium Range BS and in table 7.2.5-3a for Local Area BS for band n104.

Table X.2.5-1: NR Wide Area SBFD-capable BS reference sensitivity levels

| BS channel |  | Declared UL subband | Sub-carrier |  | Reference |  | Reference sensitivity power level, PREFSENS (dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth (MHz) |  | bandwidth (MHz) | spacing (kHz) |  | measurement channel |  | f ≤ 3.0 GHz |  | 3.0 GHz < f ≤ 4.2 GHz |  | 4.2 GHz < f ≤ 6.0 GHz |  |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -101.1 | -100.8 | -100.6 |
|  | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | 30 | G-FR1-A1-5 (Note 1) | -94.9 | -94.6 | -94.4 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

Table X.2.5-1a: NR Wide Area SBFD-capable BS reference sensitivity levels for band n104

| BS channel bandwidth (MHz) | Declared UL subband bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) (Note 1) |
| --- | --- | --- | --- | --- |
| 20, 30, 40, 50, 60, 70, 80, 90, 100 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | 30 | G-FR1-A1-5 (Note 1) | -93.1 |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |

Table X.2.5-2: NR Medium Range SBFD-capable BS reference sensitivity levels

| BS channel |  | Declared UL subband | Sub-carrier |  | Reference |  | Reference sensitivity power level, PREFSENS (dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandwidth (MHz) |  | bandwidth (MHz) | spacing (kHz) |  | measurement channel |  | f ≤ 3.0 GHz |  | 3.0 GHz < f ≤ 4.2 GHz |  | 4.2 GHz < f ≤ 6.0 GHz |  |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -96.1 | -95.8 | -95.6 |
|  | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | 30 | G-FR1-A1-5 (Note 1) | -89.9 | -89.6 | -89.4 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

Table X.2.5-2a: NR Medium Range SBFD-capable BS reference sensitivity levels for band n104

| BS channel bandwidth (MHz) | Declared UL subband bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) (Note 1) |
| --- | --- | --- | --- | --- |
| 20, 30, 40, 50, 60, 70, 80, 90, 100 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | 30 | G-FR1-A1-5 (Note 1) | -88.1 |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |

Table 7.2.5-3: NR Local Area SBFD-capable BS reference sensitivity levels

| BS channel | Declared UL subband | Sub-carrier | Reference | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| bandwidth (MHz) | bandwidth (MHz) | spacing (kHz) | measurement channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 10, 15 | 30 | G-FR1-A1-2 (Note 1) | -93.1 | -92.8 | -92.6 |
|  | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | 30 | G-FR1-A1-5 (Note 1) | -86.9 | -86.6 | -86.4 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

Table 7.2.5-3a: NR Local Area SBFD-capable BS reference sensitivity levels for band n104

| BS channel bandwidth (MHz) | Declared UL subband bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) (Note 1) |
| --- | --- | --- | --- | --- |
| 20, 30, 40, 50, 60, 70, 80, 90, 100 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | 30 | G-FR1-A1-5 (Note 1) | -85.1 |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |

==============End of change==============