# 3GPP TSG-RAN WG4 Meeting #117	R4-2521701

# Dallas, USA, Nov 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	SBFD BS to SBFD BS adjacent channel coexistence

Agenda Item:	7.2.4

Document for:	Approval

1 Introduction

SBFD BS to SBFD BS adjacent channel co-existence

- SBFD BS to SBFD BS adjacent channel coexistence

  - Focus on FR1 WA.

  - Consider both scenarios including SBFD capable BS being co-located and non-collocated.

  - Focus on bands: n104, n79, n41.

  - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include

    - TX unwanted emission

    - RX in-band blocking

  - Note 1: whether to comply with requirements is declared by BS.

  - Note 2: The existing TDD requirements apply for non-SBFD BSs

During RAN#109, RAN4 Rel-20 5G-Adavanced package has been approved. New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2 (BS RF requirement evolution) was agreed [1]. One work area is SBFD BS to SBFD BS adjacent channel coexistence.

Some progress was made in last RAN4#116bis meeting [4]. In this contribution we provide an overview on the work objectives for this topic.

2 Discussion

2.1 Background

At RAN4#116bis meeting, the following agreements were reached for SBFD BS to SBFD BS adjacent channel coexistence [4].

Agreements:

- For next meeting further consider following options, with the intention to down-select:

Option 1: UD (Operator 1) and DU (Operator 2)

Option 2: DUD (Operator 1 and 2)

Option 3: UD and DUD

Provide more description on each specific case.

- As baseline use simulation parameters defined in TR 38.858. For 6 GHz, new parameters for BS antenna should be established for SBFD.

2.2 Co-existence scenarios

In the Rel-20 WI, the scope is Macro SBFD scenarios. {D, U} or {U, D}, and {D, U, D} are the possible SBFD configurations. As concluded in Rel-18 adjacent co-existence evaluation, RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable in terms of performance (based on RAN4 models and parameters) to the {D, U, D} SBFD configuration. It is proposed to consider Option 1 as high priority. It is the consideration of simulation, and in the implementation and requirement evaluation, DUD will still be considered.

Table 2.2-1: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Simulation frequency | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- | --- |
| 1 | SBFD DL | SBFD UL | 7 GHz | D U D U | High |
| 2 | SBFD DL | SBFD UL | 4 GHz | D U D U | High |

Proposal 1: RAN4 agree on the co-existence cases as shown in Table 2.2-1.

2.3 Simulation assumptions

Simulation assumptions related to deployment parameter is captured in following table.

Table 2.3-1: Deployment parameters

| Parameter | Urban Macro |
| --- | --- |
| Carrier frequency | 4 GHz, 7 GHz |
| Layout | Single layer with 19 hexagonal cells with wrap around. |
| Inter-BS distance | 500 m |
| Grid shift | [25%] |
| Path-loss model | BS-to-UE: UMa + penetration loss, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.828 |
| LOS probability | BS-to-UE: UMa, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.803. |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) |
| BS height | 25 m |
| UE height | 1.5 m |
| UE distribution | Uniformly UE distribution 20% indoor and 80% outdoor |
| O-to-I penetration loss model | 80% low-loss model and 20% high-loss model |
| Minimum BS-UE (2D) distance | 35 m |
| DL power control | No |
| UL power control | Yes |
| Handover margin | 3 dB |
| BS mechanical down-tilt angle | 6 degrees |

Proposal 2: it is proposed to adopt the deployment parameters as in Table 2.3-1 for coexistence study

In Rel-18 and Rel-19, as baseline the array antenna model, single element configuration is adopted, as described in TR 38.803, subclause 5.2.3. As an option the antenna model extension, required for modelling sub-array configurations, is described in TR 38.803, subclause 5.2.3.2.4.

For bands n41 and n79, the simulation parameters defined in TR 38.858 can be reused. However, for band n104, the extended AAS model and parameters newly introduced for 7 GHz is more representative and closer to the actual implementation. The AAS model and parameters are defined in TR 38.922, subclause 5.4.1

Proposal 3: for band n104, it is proposed to adopt the extended AAS model and parameters as defined in TR 38.922, subclause 5.4.1

Assumptions related to BS RF characteristics relevant for different deployment scenarios are captured for FR1 in Table 2.3-2.

Table 2.3-2: FR1 BS RF parameters

| Parameter | Urban Macro (Wide Area) | Urban Macro (Wide Area) |
| --- | --- | --- |
| Carrier frequency | 4 GHz | 7 GHz |
| Transmitter power (Total conducted power) | 50 dBm/100 MHz | 50 dBm/100 MHz |
| Antenna configuration | (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi | Extended AAS model and parameters in TR 38.922, subclause 5.4.1 |
| ACIR | 60-80 dBc | 60-80 dBc |

Proposal 4: it is proposed to adopt the BS RF parameters as in Table 2.3-2 for coexistence study

The grid shift assumption has been discussed extensively. It should be choosing carefully since hose requirements are actually the guarantee of SBFD network performance. We feel the need to collect some measurements based on real deployment in order to cross-check the reliability of RAN4 co-ex simulation. To be specific, we extract the site location from a region where typical co-existence among operators with active NR traffic can be identified. Note that the concerning deployment is dense urban macro in C band, co-location case is also included. According to the CDF of site distance, following characteristics for such real deployment can be observed:

- Site distance is smaller than 10m @11%-tile of the CDF (co-site)

- Site distance is 10m - 20m @9%-tile of the CDF

- Site distance is 20m- 50m @25%-tile of the CDF

- Site distance is larger than 50m @55%-tile of the CDF

Observation 1: more than half of the base stations are more than 50 meters away from the interfering base stations in a real C band deployment.

In ECC report 281[2], the regulatory technical conditions for 5G MFCN operation in the band for non-synchronised TDD is analysed. It is exactly the same scenario as SBFD-SBFD co-existence, which is the base station to base station interference with non-time-aligned UL and DL transmission. Two deployment geometries are simulated, where the grid shift (separation between a victim BS and the nearest interfering BS) is 70 and 288 metres, respectively. The former case (case 1) is aligned with the assumption of 70 metre interferer-victim separation used in ECC Report 203 to derive the baseline BEM of -34 dBm/(5 MHz) for non-synchronised TDD.

Figure 16 in ECC report 281: Interferer-victim separation of 70 metres (Case 1)

Observation2: Interferer-victim separation of 70 metres is adopted to derive ECC BEM requirement for non-synchronised TDD in the 3400-3800 MHz band.

Proposal 5: It is proposed to reuse ECC assumption for the interferer-victim separation for SBFD-SBFD co-existence, i.e. 25% grid shift.

2.4 Simulation method

For TX unwanted emission, we can run the co-ex simulation to get a desired BS-BS ACIR level for less than 5% UL throughput loss. And then the additional unwanted emission limit to guarantee less than 5% degradation can be derived.

Proposal 6: it is proposed to run ACIR simulation to derive additional unwanted emission limit.

For in-band blocking, in Rel-19, the power level of interference from the DL adjacent channel is simulated. And 99% percentile is adopted to derive the requirement. It is noted that the method only the interfering signal, and the possibility for the wanted signal is weak and strong interfering signal come from the same direction would be very low. So it need to discuss whether 99% percentile CDF is reused for Rel-20. On the other hand, if we adopt ACIR approach to check the throughput loss which can avoid such discussion.

Proposal 7: it is proposed to further discuss and decide on the simulation methed to derive the additional blocking limits.

- Option 1: the power level CDF of blocking from the DL adjacent channel.

- Option 2: adopt ACIR to derive the additional blocking limits

2.5 Co-located scenarios

Existing OTA co-location requirements are based on single worst-case scenarios, which is inherited from passive antenna system. Edge-to-edge separation d between the BS type 1-O and the co-location reference antenna is set to 0.1 m.

Figure 2.2-1: Illustration of BS type 1-O enclosure and co-location reference antenna

The BS-to-BS isolation between two co-located base stations have been evaluated from OTA measurement and OTA co-location simulation results in Rel-19 work item on NR base station RF requirement evolution. From the table below, it can be found that the coupling loss is band/frequency dependent, e.g. the difference of measurement results can be 15-20 dB for the comparison of 7 GHz and 3.5 GHz.

Table 6.4.4.2-1 in 37.941: Intra band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical Separation | SA-to-SA | M: 54 | M: 53 | M: 56 |  |
| Horizontal Separation | SA-to-SA | M: 31 | M: 34 | M: 46 |  |
|  | A-to-SA |  | M: 35 S: 55±15 | M: 51 M: 69 | M: 55 S: 70±15 |
|  | A-to-A |  |  |  | S: 77±15 |
| M: measurement results S: simulation results. |  |  |  |  |  |

Observation 3: the coupling loss is band/frequency dependent, the higher frequency larger coupling loss is observed.

The 0.1 m edge-to-edge separation is the worst assumption. It is not suitable for SBFD-SBFD co-existence since it is not a typical scenario and it will result over stringent requirements which cause design complexity and cost. Some measurement can be found in [3].

Observation 4: The 0.1 m edge-to-edge separation is the worst-case assumption and it is not suitable for SBFD-SBFD co-existence.

Proposal 8: It is proposed to further discuss on the typical co-location scenario and deployments for SBFD-SBFD co-existence.

Conclusion

In this contribution we provide consideration on SBFD-SBFD coexistence objective.

Proposal 1: RAN4 agree on the co-existence cases as shown in Table 2.2-1.

Table 2.2-1: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Simulation frequency | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- | --- |
| 1 | SBFD DL | SBFD UL | 7 GHz | D U D U | High |
| 2 | SBFD DL | SBFD UL | 4 GHz | D U D U | High |

Proposal 2: it is proposed to adopt the deployment parameters as in Table 2.3-1 for coexistence study

Table 2.3-1: Deployment parameters

| Parameter | Urban Macro |
| --- | --- |
| Carrier frequency | 4 GHz, 7 GHz |
| Layout | Single layer with 19 hexagonal cells with wrap around. |
| Inter-BS distance | 500 m |
| Grid shift | [25%] |
| Path-loss model | BS-to-UE: UMa + penetration loss, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.828 |
| LOS probability | BS-to-UE: UMa, see TR 38.803 BS-to-BS: UMa (h_UT = 25 m), see TR 38.803. |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) |
| BS height | 25 m |
| UE height | 1.5 m |
| UE distribution | Uniformly UE distribution 20% indoor and 80% outdoor |
| O-to-I penetration loss model | 80% low-loss model and 20% high-loss model |
| Minimum BS-UE (2D) distance | 35 m |
| DL power control | No |
| UL power control | Yes |
| Handover margin | 3 dB |
| BS mechanical down-tilt angle | 6 degrees |

Proposal 3: for band n104, it is proposed to adopt the extended AAS model and parameters as defined in TR 38.922, subclause 5.4.1

Proposal 4: it is proposed to adopt the BS RF parameters as in Table 2.3-2 for coexistence study

Table 2.3-2: FR1 BS RF parameters

| Parameter | Urban Macro (Wide Area) | Urban Macro (Wide Area) |
| --- | --- | --- |
| Carrier frequency | 4 GHz | 7 GHz |
| Transmitter power (Total conducted power) | 50 dBm/100 MHz | 50 dBm/100 MHz |
| Antenna configuration | (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi | Extended AAS model and parameters in TR 38.922, subclause 5.4.1 |
| ACIR | 60-80 dBc | 60-80 dBc |

Observation 1: more than half of the base stations are more than 50 meters away from the interfering base stations in a real C band deployment.

Observation 2: Interferer-victim separation of 70 metres is adopted to derive ECC BEM requirement for non-synchronised TDD in the 3400-3800 MHz band.

Proposal 5: It is proposed to reuse ECC assumption for the interferer-victim separation for SBFD-SBFD co-existence, i.e. 25% grid shift.

Proposal 6: it is proposed to run ACIR simulation to derive additional unwanted emission limit.

Proposal 7: it is proposed to further discuss and decide on the simulation methed to derive the additional blocking limits.

- Option 1: the power level CDF of blocking from the DL adjacent channel.

- Option 2: adopt ACIR to derive the additional blocking limits

Observation 3: the coupling loss is band/frequency dependent, the higher frequency larger coupling loss is observed.

Observation 4: The 0.1 m edge-to-edge separation is the worst-case assumption and it is not suitable for SBFD-SBFD co-existence.

Proposal 8: It is proposed to further discuss on the typical co-location scenario and deployments for SBFD-SBFD co-existence.

References

- RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.

- ECC Report 281, Analysis of the suitability of the regulatory technical conditions for 5G MFCN operation in the 3400-3800 MHz band

- R4-2513773, “SBFD BS to SBFD BS adjacent channel coexistence”, Huawei, HiSilicon

- R4-2515091, “Way-Forward for NR_BS_RF_Ph2”, Ericsson