3GPP TSG-RAN WG4#117	R4-2521714

Dallas, TX, USA, November 17th – 21st, 2025

Agenda item:	4.6.1.1

Source: 	Qualcomm

Title: 	Remaining aspects of HPUE Applicability and MSD rules

Document for:	Approval

# Introduction

Considerations on the remaining aspects of HPUE Applicability and MSD rules are provided in this contribution.

- Discussion

UL CA_n5A-n8A (2UL/2DL) version of it has been discussed across two releases, Rel-18 and Rel-19. In RAN4#116, the discussion was transferred into RRM session to discuss RRM impacts. Perhaps as expected, there was no conclusion in RRM session either.

WF from [1] RAN4#116 is as follows.

Proposal 1: Introduce an interim solution in RAN4 #117 meeting to enable HPUE support for band combinations without HPUE applicability notes in Clause 5 based on [3] and the coordinated changes in the HPUE MSD LUT core requirements of sub-clause 7.3A.2.3.1 and sub-clause 7.3A.2.3.2.

Proposal 2: Companies are encouraged to study the HPUE MSD LUT solution for MSD due to one-band UL with two CCs and two-band UL with 3 CCs and identify cases where PC3 MSD is too low to be specified while its PC2/PC1.5 MSD is substantial enough to be captured in the specifications.

Proposal 3: RAN4 to finalize the process in Proposal 2 and have a complete solution to replace the HPUE applicability notes in Clause 5 by end of RAN4 #118 meeting.

Regarding proposal 1, The interim solution should be such that all versions of HPUE which are applicable according to the current notes in Chapter 5 must remain applicable also with interim solution. Changes into sub-clauses 7.2A.2.3.1 and 7.2A.2.3.2 should be such that the HPUE applicability is per notes in clause 5 and per clause 6 so that only small further changes are needed in RAN4#118 when the HPUE applicability will only refer to clause 6 in a pretty similar way like in EN_DC LUT.

Proposal 1: Write interim changes into sub-clauses 7.2A.2.3.1 and 7.2A.2.3.2 in a way that further changes in RAN4#118 are minimal when the HPUE applicability will only refer to clause 6 in similar fashion as in EN-DC.

Regarding proposal 2, and in particular for the HPUE MSD LUT solution for MSD due to one-band UL with two CCs and two-band UL with 3 CCs our thinking is as follows.

One-band UL with two CCs for TDD bands:

IMD2 power increase is 4dB and 6dB is for PC2 and PC1.5 respectively

IMD4 power increase is 8dB and 12dB is for PC2 and PC1.5 respectively

IMD6 power increase is 12dB and 18dB is for PC2 and PC1.5 respectively

IMD3 power increase is 6dB and 9dB is for PC2 and PC1.5 respectively

IMD5 power increase is 10dB and 15dB is for PC2 and PC1.5 respectively

IMD7 power increase is 14dB and 21dB is for PC2 and PC1.5 respectively

IMD9 power increase is 18dB and 27dB is for PC2 and PC1.5 respectively

The rationale for these calculations is very much aligned with [2] but with the difference that the IMD increase also for odd order IMD’s as assumed to go hand-in-hand with the MPR and power increase. We note that these are actually pretty challenging to characterise in rigorous manner and thus there is quite a bit of uncertainty. For both even and odd IMD’s we assume the limiting factor to be meeting SEM so hence despite some power increase for PC2 and PC1.5 compared to PC3 the IMD’s don’t go up really a lot becasue SEM remains the limiting factor.

Current LUT is for IMD2/3/4/5 only which is very adequate for most MSD cases. In our view, the LUT sub-clauses should not be made more complicated due to this One-band UL with two CCs for TDD bands. We recognize that these MSD cases are really the corner cases and as such not super-critical to be specified in very rigorous manner. We propose companies to evaluate whether very simple solution would be acceptable:

Proposal 2: Evaluate by RAN4#118  if all even and odd order IMD’s on One-band UL with two CCs for TDD bands could be mapped to ΔMSDmax= 9dB and 15dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

One-band UL with two CCs for FDD bands:

Currently only n5B-n12A, n5B-n14A, n5B-n29 would fall under this category, so instead of tweaking LUT any further we propose to handle these case by case.

Proposal 3: One-band UL with two CCs for FDD bands are handled case-by-case basis

Two-band UL with 3 CCs:

We think that the logic used in [2] is again very reasonable. The proposed mapping in [2] would be exactly the same than we have for the One-band UL with two CCs for TDD bands, so if feasible we would be able to use same mapping for both of these cases, avoiding unnecessary complexity in LUT requirements.

Proposal 4: Evaluate by RAN4#118  if two-band UL with 3 CC’s could be mapped to ΔMSDmax= 9dB and 15dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

Regarding process finalization in RAN4 #118, the HPUE CA LUT applicability wording must refer to Sub-clauses of Single band UE power class and Inter-band CA power class including UL CA/UL MIMO so that all cases for HPUE which are currently indicdated by Notes in clause 5 are covered.

Proposal 5: HPUE CA LUT applicability wording must refer to Sub-clauses of Single band UE power class and Inter-band CA power class in a way that all cases for HPUE CA which are currently indicated by Notes in clause 5 are covered when respective UE power class is defiend for band/band combination in Chapter 6

- Conclusion

Considerations on the remaining aspects of HPUE Applicability and MSD rules are provided in this contribution with the following proposals.

Proposal 1: Write interim changes into sub-clauses 7.2A.2.3.1 and 7.2A.2.3.2 in a way that further changes in RAN4#118 are minimal when the HPUE applicability will only refer to clause 6 in similar fashion as in EN-DC.

Proposal 2: Evaluate by RAN4#118  if all even and odd order IMD’s on One-band UL with two CCs for TDD bands could be mapped to ΔMSDmax= 9dB and 15dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

Proposal 3: One-band UL with two CCs for FDD bands are handled case-by-case basis

Proposal 4: Evaluate by RAN4#118  if two-band UL with 3 CC’s could be mapped to ΔMSDmax= 9dB and 15dB 2UL ΔMSD LUT for PC2 and PC1.5 respectively

Proposal 5: HPUE CA LUT applicability wording must refer to Sub-clauses of Single band UE power class and Inter-band CA power class in a way that all cases for HPUE CA which are currently indicated by Notes in clause 5 are covered when respective UE power class is defiend for band/band combination in Chapter 6

- Reference

[1] R4-2514628, Way Forward on HPUE notes in 38.101-1 Clause 5 tables, Apple, Skyworks, Nokia, Samsung

[2] R4-2514408, Proposal on 1UL band 2CC and 2UL band 3CC HPUE MSD, Skyworks Solutions, Inc.