3GPP TSG RAN4 Meeting #116bis		R4-2521730
Dallas, USA, 17th – 21st November, 2025

Source:	Nokia

Title:	Impact of configuring 2 SMTC periodicities over multiple SMTC occasions in NTN

Agenda item:	4.25.3

Document for:	Approval

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The WID contain a handful of objectives and among them, there is one topic on Downlink Coverage Enhancements. One of the main features discussed to be implemented as part of this feature is an extended SSB periodicity for the main (SA) cell in NTN, to match the periodic “active” and “inactive” periods of the cell. The switching between “active” and “inactive” is created in NTN to handle the limited power availability in the satellite.

RAN2 has agreed that up to 7 SMTCs might be configured by the serving cell in one frequency layer with up to 2 different SMTC periodicities. RAN2 has reached further agreements in their design and had notified RAN4 in the LS R2-2506298.

Discussion

Two SMTC periodicities within one frequency layer.

In the previous RAN4 meeting a LS was received from RAN2. In their LS to RAN4, RAN2 indicates that:

- “RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only. “

RAN2 is thereby confirming the implementation of 2 SMTC periodicities configured in parallel. In RAN4 #116, RAN4 has addressed the issue and has reached the following conclusion which remained unaltered after RAN4#116 bis :

Agreement (ad-hoc): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - For Ttarget_cell_SMTC_period in paging interruption requirements: - Ttarget_cell_SMTC_period is the SMTC periodicity corresponding to the physical cell ID of the target cell - For Kmulti_SMTC, K_satellite, Kp, Klayer1_measurement, and Kgap in RRC Connected mode intra/inter-frequency measurement requirement: - To be confirmed in maintenance phase: - Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. - Klayer1_measurement: Calculate scaling factor Klayer1_measurement based on the union of the SMTCs

The proposal above, highlighted in green to facilitate our discussion, is simple and addresses the problems of collisions. However, it is “too relaxed” as it tends in several cases lead to a significant increase in the total requirement beyond the necessary. This is better explained by looking at Figure 1. In this figure, the UE is configured with two SMTCs. SMTC 1 is configured with 20 ms periodicity, while SMTC 2 is configured with 160 ms periodicity. All occasions of SMTC1 are overlapped by SMTC2 occasions. The assumption is that in certain scenarios SMTC1 and SMTC2 cannot be measured in the same instant, for example, due to association to different satellites.

Figure 1. Two SMTCs (SMTC1 and SMTC2) configured with different periodicities. SMTC 1 is configured with 20 ms periodicity, while SMTC 2 is configured with 160 ms periodicity. All occasions of SMTC2 are overlapped by SMTC1 occasions.

If the proposal is confirmed and the max value is accepted, the value of Kmulti_SMTC will be equal to 2 for both cases, which means that the time-to-detect, time-to-measure and time-to-evaluate a cell will be multiplied by 2 for both cases. Observe that, although 1/8 of the SMTC1 occasions are colliding with other SMTCs, which means that the other 7/8 occasions are collision-free.

In the example of Figure 1, the requirements related to SMTC1 are expanded by 2, even though only 1/8 of the SMTC occasions are colliding with other SMTCs.

Likewise, for the SMTC with 160 ms, the requirements for measurement procedures will be relaxed by a factor of 2 if the regular SMTC sharing previously adopted in NTN is used in this scenario. This would be considering that the “collision” occasions will be equally split between SMTC1 and SMTC2, meaning that half of the SMTC2 occasions will be deferred to measurements in SMTC1, even though, SMTC1 had 7 collision-free opportunities already in this period.

In the past RAN4 meeting, we have discussed an alternative proposal:

When SMTCs with different periodicities collide, for measurements purposes, the SMTC with longer periodicity shall always take precedence over the SMTC with shortest periodicity.

Using this new rule, the total time to have 5 SMTCs available for measurements will be maintained unmodified alternating in  85/105 ms, whereas the total time to have 5 SMTCs available for measurements will drop from 1285 ms to 645 ms, a significant gain of 640 ms.

In the example above, the total time for the UE to have 5 measurement opportunities in SMTC2 will be enhanced from 1285 ms to 645 ms.

The gains are even more pronounced if more collisions are considered. Consider, for example, the case of Figure 2. In this example, in the worst case scenario, there is a collision among 4 SMTCs. This would cause the requirements for measurements to be relaxaed by a factor of 4. However, as previously discussed, if the longer periodicity takes precedence over the short periodicity, SMTC2 will only have to share its opportunities in collisions with SMTC4, and that would cause a relaxation factor equal to 2. Now, let’s take a look at SMTC1 and SMTC3. In this new example, some (1/8) of their opportunities are colliding with SMTCs with larger periodicity (which take precedence), and the other 7/8 are to be split equally between the two.

Figure 2 – Four SMTCs with different periodicities and offsets.

In both cases, Figure 1 and Figure 2, if we consider that the SMTC with shorter period doesn’t take precedence over the SMTCs with longer periodicity, then we will consider the value of K_multi_SMTC only for the occasions shared between SMTCs with same periodicity. Therefore, it has been proposed that:

The value of K_multi_SMTC is to be calculated only considering SMTCs with same periodicity.

But by doing so, the amount of SMTC opportunities left for SMTC1 in Figure 1 and SMTCs 1 and 3 in Figure 2 will be smaller (by a factor of 7/8) then they would be if these SMTCs did not collide with SMTCs with larger periodicities. In some scenarios this can be even smaller fraction. In order to compensate for the lost measurement opportunities, we propose to add an additive factor in the measurement requirements for SMTCs with shorter periodicity. Finally, in RAN4 #116bis, it has been proposed that:

For SMTCs with shorter periodicity that overlap with SMTCs with larger periodicity the formula for the measurement requirements shall consider a factor L_multi_SMTC, which corresponds to the number of measurement opportunities lost due to collision with SMTCs with larger periodicity within 5 SMTC occasions.

Figure 3 – Example with four configured SMTCs with different periodicities and offsets. SMTC occasions which overlap with SMTCs with different periodicities will be used for SMTCs with the largest periodicity

One example of the requirements considering that some of the measurement occasions will be skipped in some SMTCs is given below.

Table 1 - Example of measurement period

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( (5+ Lmulti_SMTC) x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x (5+ Lmulti_SMTC) x Kmulti_SMTC x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil((5+ Lmulti_SMTC) x Kmulti_SMTC x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1:If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Using the example in Figure 3, Lmulti_SMTC would always be 0 for the measurements of SMTC2 and SMTC4, the SMTCs with largest periodicity. In SMTC1 and 3, this term would be equal to the number of measurement occasions within the measurement period that overlap with the SMTC2 and SMTC4.

These proposals have been discussed in RAN4 #116bis, but companies highlighted that if this case was used with other SMTC periodicities, for example 40 ms and 20 ms, the SMTC with shortest periodicity would not be measured at all, as in the figure below:

Figure 4 .- Example of three configured SMTCs with different periods and offsets. SMTC occasions which overlap with SMTCs with different periodicities are used for the SMTCs with largest periodicity, resulting in the SMTC with shortest periodicity not being measured at all.

We acknowledge that this is a possible scenario considering the SMTC periodicities that can be defined in 5G NR. However, we note that the default SSB periodicities agreed in RAN1, apart from the existing 20 ms value, is 160 ms, as in the agreement from RAN1 #120:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The additional default value assumed by UE during initial access (apart from the existing 20ms value) is 160 ms.

Considering the agreement above, we note that even though the configuration is possible, the system should not be optimized for all the combinations of SMTC periodicity, given that the SSB periodicity is expected to be either 20ms or 160 ms for DL-CE cells.

Therefore, our view is that proposals 4 to 6 could be adopted in cases the longest SMTC periodicity is equal to 160ms, and for the case in which the longest SMTC periodicity is less than 160ms, the proposal from RAN4 #116 is adopted:

Adopt proposals 3, 4 and 5 for all cases in which the longest SMTC periodicity equals 160 ms, and in the other cases, adopt the proposal from RAN4 #116: Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

Conclusion

In this paper we discussed the impact of the new measurement framework proposed by RAN2 and RAN1 to handle the extended SSB periodicity in NTN under the topic of DL coverage enhancements. Based on our discussion and analysis, we made the following list of proposals:

Observation 1: In the example of Figure 1, the requirements related to SMTC1 are expanded by 2, even though only 1/8 of the SMTC occasions are colliding with other SMTCs.

Proposal 1: When SMTCs with different periodicities collide, for measurements purposes, the SMTC with longer periodicity shall always take precedence over the SMTC with shortest periodicity.

Observation 2: In the example above, the total time for the UE to have 5 measurement opportunities in SMTC2 will be enhanced from 1285 ms to 645 ms.

Proposal 2: The value of K_multi_SMTC is to be calculated only considering SMTCs with same periodicity.

Proposal 3: For SMTCs with shorter periodicity that overlap with SMTCs with larger periodicity the formula for the measurement requirements shall consider a factor L_multi_SMTC, which corresponds to the number of measurement opportunities lost due to collision with SMTCs with larger periodicity within 5 SMTC occasions.

Proposal 4: Adopt proposals 3, 4 and 5 for all cases in which the longest SMTC periodicity equals 160 ms, and in the other cases, adopt the proposal from RAN4 #116: Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

- References

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT R2-2506298, LS on additional agreements on SMTC enhancements, RAN WG2 Meeting #131, Bengaluru, India, 25th - 29thAugust, 2025

- R2-2506298, LS on additional agreements on SMTC enhancements, RAN WG2 Meeting #131, Bengaluru, India, 25th - 29thAugust, 2025

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm (Moderator), Bengaluru, India, 25th - 29thAugust, 2025