3GPP TSG-RAN WG4 Meeting#117                                R4-2521732

Dallas, USA, 17th – 21st, Nov. 2025

Agenda item:	8.4.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on 6GR TN BS study

Document for:	Approval

Introduction

In the RAN#108 meeting, a new WID on Study on 6G Radio [1] was approved. In the last RAN4 meeting, there were some initial discussions on 6G NR BS RF requirement and agreements were captured in WF [8].In this contribution, we would like to share some further understandings on 6GR TN BS study.

- Objective: - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - RAN4#1116bis agreement: Issue 1-1-1: Starting point Agreement: 5G BS RF requirements should be considered as starting point when defining the 6G BS RF requirements. Issue 1-1-2: Requirements to be re-assessed Way Forward: For next meeting, companies are encouraged to evaluate how 5G BS RF requirements should be considered for 6G and provide their view using the table in Annex A as example. Note: The goal is to identify: - 5G requirements that might also be applicable to 6G. - 5G requirements which will need to be re-evaluated based on 6G system parameters but would not need a further study. - 5G requirements which should be further studied in the scope of the 6G SI, a prioritization might be made in next meeting. Issue 1-2-1: BS type 1-H enhancement Agreement: BS Hybrid beamforming type of architecture shall be studied. Requirements for cmWave bands Issue 1-3-3: Rx Requirements Agreement: For above 7 GHz, further study how BS Rx requirements should be specified, using FR2 approach or not, including if blocking should be specified based on FR2 methodology.

Discussion

- 2.1. Review of 5G BS evolution

In the following Figure 2.1-1, we provide the overview of 5G NR BS RF evolution and study on 7-24GHz for ITU-R WP5D reply LS which has already provided a very good foundation for 6G BS RF requirement.

Figure 2.1-1. The review of 5G NR BS evolution from Rel-15 to Rel-20

Proposal 1: for 6GR BS, propose to consider the following 5G BS types at least as starting point.

6GR TN

NR BS

Repeater, NCR for coverage extension with low cost

SBFD

- 2.2. New BS types in addition to 5G NR

- 2.2.1. FR1 Hybrid beamforming

Due to the increasing operating frequency for FR1 and FR3, the large array size (e.g. 2K, 4K antenna elements as agreed in 6G TRP) is expected as one direct and efficient solution to try to achieve the same coverage as FR1 middle bands as requested in 6G SID for site sharing between FR1 middle bands and around 7GHz. However due to the high cost, overheating problems and baseband implementation complexity if there are too many transceivers, therefore the hybrid beamforming similar as FR2 as shown in the following Figure 2.2.1-1 is more appropriate to FR1high bands and potential FR3 bands if WRC-27 can confirm it as IMT operation.

Figure 2.2.1-1. The illustration of RF architecture of FR1 and around 7GHz hybrid beamforming

Due to the potential active component within RDN for variable analog beamforming, the TAB connectors for the requirement definition and conformance testing could be shifted to TAB2 as shown in Figure2.2.1-2 instead of the legacy TAB connectors between RDN and transceiver array. However this is also highly dependent on the implementation for variable phase shifter within the RDN network. If the variable phase shifter is implemented in passive manner as shown in Figure 2.2.1-1, then the legacy TAB connectors in TS38.104 could be still used for the RF requirement definition and conformance testing. If the variable phase shifter is implemented with active RF component, then the new TAB2 connectors as shown in Figure 2.2.1-2 is more appropriate for the RF requirement definition and the corresponding conformance testing.

Figure 2.2.1-2. The illustration of RF framework for specification

In addition, in the current TR 38.803, extended AAS antenna modelling is applicable for AAS BS with digital beamforming plus fixed sub-array tilting without variable beam steering capability considered within RDN, therefore in the following section, we propose to study the enhanced AAS antenna modelling for FR1 hybrid beamforming as shown in Figure 2.2.1-1.

The enhanced AAS antenna model for FR1 hybrid beamforming is proposed as following.

Wm3=1Msub3expj∙2πm3−1dv,subλ∙sinθsubtilt3

Sub-array pattern #3 with fixed down-tilt could be defined as follows:

Asub3θ,φ=AEθ,φ+10log10m3=1Msub3Wm3∙Vm32

where,

Vm3=expj∙2πm3−1dv,subλ∙cosθ

Sub-array pattern #2 with variable electrical down-tilt could be defined as follows:

Asub2θ,φ=Asub3θ,φ+10log10m2=1Msub2Wm2∙Vm22

where,

Wm2=1Msub2expj∙2πm2−1dv,sub∙Msub3λ∙sinθsubtilt2

Vm2=expj∙2πm2−1dv,sub∙Msub3λ∙cosθ

Composite antenna for the hybrid beamforming with digital beamforming could be defined as follows:

Acompositeθ,φ=Asub2θ,φ+10log10m1=1Mn=1NWm1,n∙Vm1,n2

where,

Wm1,n=1M∙Nexpj∙2πm−1dvλ∙sinθtilt−n−1dhλcosθtilt∙sinφscan

dv=α∙Msub3∙dv,sub

Vm1,n=expj∙2πm−1dvλ∙cosθ+n−1dhλsinθsinφ1

Proposal 2: propose to consider the antenna modelling of hybrid beamforming in 6GR especially for around 7GHz 6GR BS.

- 2.2.2. FR1 BS type 1-H in FR1 low bands

For BS at FR1 low bands especially below 1GHz, usually BS type deployed are mainly focusing on BS type 1-C with traditional RRU and passive antenna, however for these important coverage bands for both 5G and 6G, to evolve from the legacy BS type 1-C to BS type 1-H with more digital transceivers chains could help boost the capacity especially from MU-MIMO perspective. Indeed this kind of BS type implementation at FR1 low bands was also discussed in 5G-A phase as NR BS spec maintenance/improvement during Q4,2024, however there were no conclusions draw for it.

Figure 2.2.2-1. The illustration of AAS evolution at FR1 low bands (e.g. low than 1GHz)

Proposal 3: propose to consider the BS type 1-H at FR1 low bands as the evolution of the legacy BS type 1-C in 6G day1.

- 2.2.3. AAS BS supporting in-band NB-IoT operation/LP-WUS

NB-IoT In-band operation, NB-IoT guard band operation, NB-IoT standalone operation and NB-IoT operation in NR in-band, NB-IoT operation in NR guard band have not been supported for AAS MSR BS or NR BS type 1-H/1-O. The primary reason for this type of restriction is that NB-IoT is expected to be deployed in operating bands below 2 GHz. However, for these specific frequency bands, BS type 1-C was the most typical BS implementation previously. Recently, for FR1 low bands (e.g. lower than 2GHz), massive MIMO or BS type 1-H is also supported to further improve the network capacity, therefore it is necessary to specify the in-band NB-IoT operation under the BS type 1-H/1-O architecture in 6G considering the expected long life cycle of NB-IoT system in filed.

Proposal 4: propose to support the in-band NB-IoT operation for BS type 1-H and BS type 1-O in 6G day1.

LP-WUS signal was introduced in Rel-19 for both FR1 and FR2-1 as one of important 5G-A feature to enable ultra-low power consumption. However during the Rel-19 discussion, the technical concerns on the potential EVM degradation on NR DL signal due to the simultaneous transmission of LP-WUS signal was raised especially for high modulation order (e.g. 256QAM) which is almost the mandatory feature of the commercial NR BS, therefore we propose to conduct more technical discussions in 6G day1 to figure out the problems instead of reusing the agreement/requirements from 5G-A in Rel-19 and leave up to the implementation.

Proposal 5: if LP-WUS signal is supported in 6G day1, propose to further discuss the impacts on potential EVM degradation of NR signal due to the simultaneous LP-WUS signal transmission.

- 2.2.4. Calibrated beamforming for multi-band operation BS

In the Rel-15, the fractional bandwidth as shown in the following was introduced for NR BS type 1-H/1-O/2-O considering the NR BS supporting the large frequency range within the band (e.g. band n77, n104, FR2 bands) or across different bands for MB-BS and the corresponding antenna performance across the large frequency range.

fractional bandwidth: fractional bandwidth FBW is defined as FBW=200∙FFBWhigh−FFBWlowFFBWhigh+FFBWlow% For operating bands where the supported fractional bandwidth (FBW) is larger than 6%, two rated carrier EIRP may be declared by manufacturer: - Prated,c,FBWlow for lower supported frequency range, and - Prated,c,FBWhigh for higher supported frequency range.

In the following Figure 2.2.4-1, we try to clarify the background reasoning to define this fractional bandwidth in 5G and the potential improvement in 6G phase.

First of all, if the supporting frequency range for BS or physical antenna array is very large, then in order to avoid the antenna gating lobes at the highest supported frequency range to waste the radiation power, the spacing of antenna element is usually decided by the upper frequency instead of low frequency or middle frequency as shown in Figure 2.2.4-1. For beamforming at carrier 1, 2, 3, due to the different wavelength at these frequency range which is not matching with the actual physical element spacing for carrier 4, beamforming gain cannot been steered directly or with some mismatching beam steering, therefore we propose to have some further discussion on the feasibility on the calibrated beamforming for multi-band operation BS or wide spectrum operation BS.

Figure 2.2.4-1.The illustration of fractional bandwidth and potential calibrated beamforming in 6G phase

Proposal 6: in order to achieve the more accurate beam steering direction in 6G day1. propose to have some study  the architecture and RF feasibility on the calibrated beamforming steering for multi-band operation BS or wide spectrum operation BS instead of defining the fractional bandwidth for different rated EIRP.

- 2.2.5. Dynamic power sharing for multi-band operation BS

In the commercial network, most of BS can support the large aggregated carrier bandwidth (e.g. intra-band contiguous CA or non-contiguous CA or NR+LTE in the adjacent channel case) or even multi-band operation if these bands are close to each and owned by the same/collaborative operator and plan to commercialize these bands in the filed as shown in Figure 2.2.5-1, the legacy static power setting among different carriers within the band or across different bands is not very efficient from both optimal power consumption and network capacity/coverage perspective, therefore the dynamic power sharing would be useful to improve the network performance in 6G phase (e.g. dynamically adjust the power according to the traffic load in the different carriers/bands).

Figure 2.2.5-1. The illustration of dynamic power sharing for multi-band operation

Proposal 7: for the multi-band operation, propose to consider the dynamic power sharing across different bands to improve the network capacity/coverage.

- 2.2.6. Reconfigurable Intelligent Surfaces (RIS)

To fulfill the requirements of 6G in the future, Reconfigurable Intelligent Surfaces (RIS) as shown in Figure 2.2.6-1/2 stands out as a pivotal potential key technology, offering new physical dimensions, intelligent programming, and the capability to manipulate electromagnetic waves abnormally to provide the service to the target UE. RIS has also gather substantial attention from both academia and industry owing to its cost-effectiveness, low power consumption, and straightforward deployment.

Figure 2.2.6-1. The illustration of network controlled RIS

Figure 2.2.6-2. The illustration of standalone RIS

Proposal 8: for the efficient coverage extension, propose to consider the RIS deployment in 6G day1.

- 2.2.7. Integrated Sensing and Communication (ISAC)

Integrated Sensing and Communication (ISAC) is an emerging technology that seamlessly combines wireless communication and sensing functionalities into a unified system. By leveraging shared hardware, spectrum, and signal processing techniques, ISAC BS shown in Figure 2.2.7-1 enables simultaneous data transmission/reception and environmental sensing (e.g., object detection, localization, and motion tracking). This integration is particularly pivotal for next-generation networks like 5G-Advanced and 6G.

Figure 2.2.7-1. The illustration of ISAC BS type

Proposal 9: for the sensing technology, propose to consider the 6G ISAC BS in 6G day1 at least.

- 2.3. Improvement for the RF requirement of 6GR

- 2.3.1. EVM requirement

As shown in the following agreement, during the last RAN1#122 meeting, RAN1 has already agreed to use the 5G modulation order as basis and further discuss other enhancement and modulation orders. In general, we could apply the EVM requirement of 5G BS as starting point and further discuss the potential EVM requirement within 7-24GHz.

Agreement: RAN1_122 meeting - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Proposal 10: for minimum EVM requirement of existing 5G modulation order of 6GR, propose to apply the 5G BS EVM requirement as starting point for 6GR BS.

In addition, for the higher modulation order as shown in Figure 2.3.1-1 (e.g. 2048QAM or 4096QAM as used by the legacy microwave backhual transmission), the expected minimum EVM requirement for single layer transmission is presented in the following Table 2.3.1-1 which seems very challenging to meet this kind of stringent requirement with the current art of technology. If considering the multi-layer transmission for 2048QAM and 4096QAM, then the expected minimum EVM requirement would become even more stringent than the values in the following Table 2.3.1-1. In addition, considering the achievable SNR in filed, it seems also quite difficult to meet suck kind of SNR for correct demodulation from both intra-cell inter-user interference and inter-cell inter-user interference. The details could be further discussed and confirmed by the system level simulation results.

Figure 2.3.1-1. the potential EVM requirement for higher modulation order than 5G NR

| Modulation order | EVM | SNR(dB) |
| --- | --- | --- |
| 4096QAM | 0.89 % | 41.0122 |

Figure 2.3.1-1. The illustration of 4096QAM constellation

Since the EVM requirement is extremely critical for the network performance/achievable peak data rate in field. In the past, BS EVM requirement is mainly evaluated under the assumption of SU-MIMO with the limited MIMO layers (up to 2/4 MIMO layers ), however this kind of evaluation assumption is not quite aligned for the scenarios with high traffic load in the commercial network. For the high traffic load case, usually MU-MIMO and CA must be considered to guarantee the network capacity. In short, in 6G phase, we propose to discuss the optimal EVM requirement in addition to the minimum EVM requirement at least to reflect the more realistic network operation including SU-MIMO and MU-MIMO case.

Proposal 11: for the optimal EVM requirement, propose to have some discussion on the necessity and evaluation method to figure out the optimal/enhanced EVM requirement to enable the achievable peak data rate if possible.

- 2.3.2. Relaxed EVM for DL digital Predistortion (DPoD)

In the last RAN#109 meeting, it is indicated in the following NOTE[3] that 6G will firstly start the technical discussions on non-linearity modelling and other common part between 5G and 6G for the potential MPR reduction under the 6G agenda, the normative 5GA work will be further discussed after the completion of SI phase. From our understanding, DL DPoD is also useful to increasing the transmission power from BS DL (e.g. reduce the power backoff similar as MPR improvement). In addition, the relaxed EVM requirement from BS side would also help alleviate the implementation complexity and cost for pursuing the optimal EVM requirement e.g. 2.5/2.8% EVM requirement for DL 1024QAM.

Power enhancement with relaxed UE Tx EVM and advanced BS receiver - Note: Discuss both 5G-Adv and 6G reduced UE MPR values with the relaxed Tx EVM requirement as part of 6G study. RAN4 will further discuss and finalize (if agreed) the scope and other necessary details (e.g. NW control of this feature) in 2025.Q4 under 6G agenda items. It will be further decided in RAN#113 (Sep, 2026) whether or not there will be a follow up Rel-20 WI under 5G-Adv. For 5G-Adv, solution should not be AI based and there should not be any impact to RAN1.

- Figure 2.3.2-1. The illustration of potential solution for DL digital predistortion (AI or non-AI based DPoD compensation at UE side)

Proposal 12: for the relaxed EVM requirement for 6GR BS, propose to consider the AI or non-AI based DPoD compensation at UE side to improve the DL coverage.

- 2.3.3. TAE requirement

In the following Table 2.3.3-1, we summarized the background information how the NR TAE requirement was defined in the past and how these requirement have the impacts on the following release discussions (e.g. NES, fragment DL carriers, FR2 CBM/IBM, spectrum aggregated positioning).

Table 2.3.3-1. the summary of NR TAE requirement for FR1and FR2-1

| Freq range | MIMO and CA use case | TAE | Remark |
| --- | --- | --- | --- |
| FR1 | MIMO transmission | 65ns | This requirement is inherited from UTRA as 1/4 chip duration of 3.84MHz UTRA FDD system; |
|  | intra-band contiguous carrier aggregation | 260ns | This is based on assumption of single RF reception and single FFT operation, therefore this 260ns requirement is sufficient to support the orthogonality between these carriers, however this is not sufficient for spectrum aggregated based positioning as discussed in the previous release. |
|  | intra-band non-contiguous carrier aggregation | 3us | This is originally based on the assumption of individual RF chain reception for inter-band CA case. In the Rel-19, fragmented DL carrier with the shared RF chain for FR1 FDD bands, 3us is still less than CP 4.71us of 15kHz SCS. For FR1 high TDD bands with SCS as 30kHz, then the CP of SCS of target Scell if NES with SSB less operation (this has not been supported yet in 5G-A is supported ) is expected to be 2.35us which is less than the existing 3us. |
|  | inter-band carrier aggregation | 3us | This is based on the assumption of individual RF chain reception for inter-band CA case. In Rel-18, NES with SSB less operation is based on the assumption that BS TAE requirement is less that corresponding CP of SCS of target Scell. |
| FR2-1 | MIMO transmission | 65ns | This requirement is inherited from UTRA as 1/4 chip duration of 3.84MHz UTRA FDD system; |
|  | intra-band contiguous carrier aggregation | 130ns | Similar as FR1 intra-band contiguous CA and higher subcarrier spacing is assumed for FR2-1, the requirement is reduced from 260ns to 130ns. |
|  | intra-band non-contiguous carrier aggregation | 260ns | This is based on the assumption of shared RF chain reception for intra-band non-contiguous CA different from FR1 UE assumption. At the beginning, the minimum TAE requirement 3us is applied for FR2 intra-band non-contiguous CA, however 3us is larger than CP of FR2 supported SCS, considering the frequent Rx sweeping, then there would be lots of data interruption on Scell(s) which will cause the significant performance degradation, then this requirement was improved from 3us to 260ns to minimize the Scell reception interruption. |
|  | inter-band carrier aggregation | 3us | This is based on the assumption of individual RF chain reception for inter-band CA case. 3us TAE requirement for inter-band CA also make the UE CBM operation impossible. So far from FR2 inter-band CA scenario, only IBM operation is supported. CBM operation was discussed in the Rel-17, however due to the difficulty to reach the consensus on the applicable frequency separation for CBM, then it is dropped at the end. |

It could be easily found that more appropriate TAE requirements should be considered for 6G at the beginning which is extremely important for forward compatibility design.

For the TAE requirement for 6G MIMO feature, we propose to conduct the corresponding physical layer evaluation similar as the study in UTRA phase instead of reusing the requirement directly without the justification.

Proposal 13: for the MIMO TAE requirement for 6GR BS, propose to conduct the relevant physical layer evaluation with the TAE and different MIMO layers taken into account to define the more appropriate TAE requirement;

For the TAE requirement for 6G beamforming feature, we propose to conduct the relevant study to figure out the appropriate phase error requirement for it. Meanwhile the feasibility and achievable performance of online timing/amplitude calibration within the AAU should be also considered.

Proposal 14: for TAE requirement among different transceivers for 6GR BS beamforming, propose to discuss the necessity and how to define the requirement for it in 6G day 1.

For the intra-band contiguous CA for 6G BS, we propose to consider the different requirements for different use case if necessary e.g. the orthogonality to reduce the inter-carrier interference for communications, SSB-less for fast scell activation and spectrum aggregated positioning;

Proposal 15: For the intra-band contiguous CA for 6G BS, propose to consider the different requirements for different use case if necessary firstly e.g. the orthogonality to reduce the inter-carrier interference for communications, SSB-less for fast scell activation and spectrum aggregated positioning, then further discuss how and whether to specify the unified TAE requirement.

For the intra-band non-contiguous CA for 6G BS, similar as the consideration in 5G-A phase, both NES with SSB-less operation in FR1 and the potential interruption due to Rx sweeping in FR2-1 should be considered.

Proposal 16: For the intra-band non-contiguous CA for 6G BS, propose to discuss the necessary requirement from both with the consideration of NES with SSB-less operation in FR1 and the potential interruption due to Rx sweeping in FR2-1.

For inter-band CA, the existing requirement was defined as 3us for NR based on the worst assumption that the transceivers in different bands are implemented with individual/separate sync source and both sync source has been lost (e.g. 2 downlink bands), then transceivers per band need to maintain its sync performance by its internal maintenance or backup timing module, therefore its achievable timing difference is defined as 3us.

However as shown in the following Figure 2.3.3-1 for typical inter-band CA BS architecture in the co-located scenario, then practical timing difference among different bands should be optimized down to 10ns-65ns which is much less than the normal CP of the supported subcarrier spacing in FR1.

In addition, if the “adjacent” bands which could be supported by single RF transceiver chain (e.g. FR1 multi-band operation), then TAE between different bands would be mainly dominated by the group delay due to the analog RF component without any residual transceiver calibration error, then the TAE performance is also quite limited. Indeed for single RF chain case to support the multi-bands which is quite similar as intra-band contiguous CA RF architecture to support the BW aggregated positioning, it was agreed that TAE could be ignored otherwise the positioning performance cannot been guaranteed at all.

- Figure 2.3.3-1. The link budget for inter-band CA TAE performance in co-located scenario

- Observation 1: the FR1 inter-band CA TAE requirement in co-located scenario as 260ns is feasible.

Proposal 17: for the inter-band co-located CA for 6G BS, propose to discuss the optimal TAE requirement considering the NES with SSB-less operation in FR1 and FR2 UE Common beam management (CBM) in FR2-1,etc.

Lastly, the 6G spectrum utilization (e.g. new carrier aggregation mechanism instead of SU with the certain carrier in RAN4) design is expected to start in 2026, Q1, there might be some potential innovative solutions agreed by RAN1. From the procedure wise, after the consensus reached in RAN1/2, RAN4 also need to discuss and define the corresponding TAE requirement to enable the potential RRM measurement across different carriers with this new spectrum utilization mechanism.

Proposal 18: for the potential new spectrum utilization mechanism, RAN4 need to discuss and define the corresponding TAE requirement to enable the potential RRM measurement across different carriers.

- 2.3.4. Cell phase requirement

For BS RF requirement perspective, usually we only define the RF requirement for single BS or transceivers with the single enclosure for the deployment. For the cell phase minimum requirements 3us (or +/-1.5us according to the ITU-T regulation) for different BSs/TRPs, this is captured in TS 38.133 as the basic assumption to derive lots of the timing requirement from UE side (e.g.MRTD/MTTD requirement, N_TA_offset). In addition, this cell phase error requirement is also only applicable for TDD BSs with the overlapping coverage areas to provide the basic guidance on how to configure the DL-UL transition period or DL-UL transition gap to avoid the BS2BS/UE2UE CLI interference.

In addition, it should be noted that the minimum requirement 3us inherited from 3G to 4G and 5G is the worst case assumption with the external timing reference signal lost at the extreme conditions, then gNB should use the backup internal oscillator to maintain the basic timing unit for the baseband and AAU operation. Usually this kind of worst case performance is not expected to occur under the normal operation mode. As shown in the normal operation with either GPS/IEEE 1588 as input timing signal, the cell phase error among different BS with different reference timing source could be maintained within 200ns instead of 3us. If the different BS could be implemented with same timing reference as shown in the following Figure 2.3.4-1 and and some further internal known measurement/calibration to achieve better alignment, cell phase error performance could be maintained much better than 200ns.

- Proposal 19: for cell phase error requirement in 6G, propose to consider both the worst error requirement 3us and achievable cell phase error requirement under the normal operation mode to guide the relevant RRM requirement definition.

Figure 2.3.4-1. The improved cell phase requirement for CJT or cell free concept in 6G day1

Another important deployment scenario called the coherent joint transmission (CJT) as shown in Figure 2.3.4-2 will also have the direct impacts on cell phase error requirement. The coherent joint transmission (CJT) offers downlink spectral efficiency and coverage gain. In Rel-18, Type-II CSI has been enhanced to accommodate CJT assuming ideal synchronization and backhaul. In Rel-19, UE reporting enhancement for CJT deployment under the non-ideal sync and backhaul case is also specified. In short, in order to achieve the coherent reception from UE side, the phase error between different TRPs should be maintained in certain degree level to ensure the positive summation from different received signals.

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

Figure 2.3.4-2. The improved cell phase requirement for CJT or cell free concept in 6G day1

- Proposal 20: for cell phase requirement for CJT transmission, propose to have further discussion on the potential required phase alignment requirement with the associated UE measurement reporting information for timing misalignment, freq/ phase offset measurement and reporting. The accuracy of UE measurement reporting will also have direct impacts on the achievable performance at BS side.

- 2.3.5. ACLR requirement

ACLR requirement is a critical RF requirement to guarantee the coexistence performance between adjacent carries of the same RAT or different 3GPP RAT. Meanwhile ACLR requirement is also one of the important parameters to dimension the RF design especially from potentially different PA design and different power consumption for the overall BS, the appropriate interference antenna modelling in the adjacent channel is extremely important to quantify the interference and figure out the required ACLR requirement.

Indeed, the spatial antenna modelling in the adjacent channel was discussed in the past and could be dated back to the AAS SI phase in Rel-12 phase. The composite radiation gain pattern for pure LOS channel can be expressed in logarithmical scale as captured in TR 37.840:

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2−1

Where ρ is the correlation level for the unwanted emission in the adjacent channel.

However after the Rel-12 LTE AAS SI, there were some contiguous discussions in both 3GPP and ITU-R WP5D on how to justify the more appropriate correlation level for the unwanted emissions for sharing study in ITU-R. During the Rel-19 SI for WP5D LS on IMT parameters of 4.4-4.8GHz, 7.125-8.4GHz and 14.8-15.35GHz, there were some further discussions on antenna modelling of unwanted emission in the adjacent channel, the array correlation factor roll-off model according to frequency offset away from the wanted signal carrier bandwidth edge is captured as following (extracted from TR 38.922).

Figure 2.3.5-1: Array correlation factor roll-off model. [TR 38.922]

Using the approach above the antenna model can be extended to capture array antenna gain outside the wanted carrier as:

AAθ,φ,f=Asubθ,φ+10log101+ρfm=1Mn=1Nwm,nvm,n2−1

In addition, the potential relationship between ACLR requirement and EVM requirement should be also considered. In the past, these two requirement was discussed separately in RAN4, however in the practical implementation, there are some implicit relationship e.g. more relaxed ACLR requirement will also result in the worse EVM requirement within the wanted carrier which will cause the negative impacts on the actual network performance.

- Proposal 21: for the ACLR requirement, propose to have some further study on the appropriate ACLR modelling to quantify more realistic interference modelling in the coexistence sharing study and define more proper ACLR requirement;

- Proposal 22: for the ACLR requirement, propose to consider the performance balance between ACLR and EVM requirement instead of treating these two requirement separately.

- 2.3.6. UL in-band blocking requirement

- For FR1 in-band blocking requirement -43d	Bm for FR1 WA BS, it could date back to UTRA phase considering the 99.99% of CDF curve of received power from UTRA UE UL transmission and 99% of CDF curve of received power from E-UTRA UE UL transmission, this requirement was reused for 5G directly without any further evaluation. However considering the various HPUE proposals (e.g. PC2, PC1.5, PC1, ATG UE, UAV HPUE UE) across FR1 TDD/FDD bands, the evolution of RF architecture for FR1 AAS BS (e.g. transition from BS type 1-C to digital beamforming with passive sub-array and hybrid beamforming as mentioned in clause 2.2.1) and increasing operating frequency for FR1 and FR3, therefore we propose to conduct the further study with more relevant coexistence assumptions to identify the appropriate requirement for 6GR BS.

- Proposal 23: for in-band blocking requirement of FR1 BS, propose to conduct the further study with more relevant coexistence assumptions to identify the appropriate requirement for 6GR BS.

- 2.3.7. UL out-of-band blocking requirement

For FR1 out-of-blocking requirement -15dBm, this could date back to UTRA phase due to the WiFi STA or GSM UE2UE CLI analysis e.g.30/33dBm-40dBc MCL-4/5dB body loss=-15dBm. This power level is then applicable for the out-of-blocking requirement from both UE and BS perspective and reused for both 4G and 5G. For 6G SI phase, we think that it is more appropriate to reevaluate this requirement with more realistic deployment assumption and the corresponding operating frequency and worst interference source (e.g. HPUE in UL or BS2BS CLI) causing this kind of blocking signal. It should be noted that the in-band blocking and out-of-blocking requirement play very important role of dimension the size of receiver’s analog filter which is very critical design for BS supporting massive MIMO operation.

- Proposal 24: for out-of-band blocking requirement of FR1 BS, propose to conduct the further study with more relevant coexistence assumptions to identify the appropriate requirement for 6GR BS.

- 2.3.8. Co-location requirement and coexistence spurious emission requirement

For the improvement of co-location requirement in FR1 high bands, it was discussed in Rel-19 and the normative work was postponed to Rel-20 BS RF enhancement [4]. For coexistence spurious emission requirement, this is also under the discussion in Rel-19 BS RF enhancement. From our understanding, the improvement for these two requirements in 5G-A phase can also apply for RF requirements of 6GR BS.

- Proposal 25: in order to avoid the parallel discussion on improvement on FR1 co-location requirements and coexistence spurious emission requirements between 5G-A and 6GR, propose to wait for the conclusion in 5G-A and make the necessary updates for RF requirements of 6GR BS.

- 2.3.9. The transition from BS type 1-H to BS type 1-O

For the existing BS type 1-H, there are only two radiated requirements specified e.g. EIRP and EIS. However the rest of conducted requirement might not reflect the overall radiated performance of the real operation of NR BS especially for EVM and ACLR requirements. In addition, for some bidding test of the operators, BS type 1-H also include some other radiated requirements e.g. EVM, ACLR, spurious emission, ACS, blocking. The details could be found in the following Table 2.3.9-1.

Table 2.3.9-1. the more inclusive radiated requirement on top of 5G NR BS type 1-H

|  | 5G BS type 1-H | - 6GR BS type 1-H |
| --- | --- | --- |
| Tx part |  |  |
| Base station output power | Conducted, OTA | - Conducted, OTA |
| Output power dynamics | Conducted | Conducted |
| Transmit ON/OFF power | Conducted | Conducted |
| Transmitted signal quality | Conducted | OTA |
| OBW | Conducted | Conducted |
| Tx intermodulation | Conducted | Conducted |
| ACLR | Conducted | - Conducted, OTA |
| Operating band unwanted emissions | Conducted | Conducted, OTA |
| Transmitter spurious emissions | Conducted | Conducted, OTA |
| Rx part |  |  |
| REFSENS | Conducted,OTA | Conducted, OTA |
| dynamic range | Conducted | Conducted |
| ACS | Conducted | Conducted, OTA |
| blocking | Conducted | Conducted, OTA |
| NBB | Conducted | Conducted |
| RX IMD | Conducted | Conducted |
| RX NB IMD | Conducted | Conducted |
| RX spurious emission | Conducted | Conducted |
| RX ICS | Conducted | Conducted |

Proposal 26: for the enhanced 6GR BS type 1-H, propose to discuss the additional OTA requirement in addition to EIRP and EIS requirement for BS type 1-H to reflect the radiated performance more precisely.

Conclusions

In this contribution, we shared some initial views on the 6GR BS RF study and proposals are made as following:

Proposal 1: for 6GR BS, propose to consider the following 5G BS types at least as starting point.

6GR TN

NR BS

Repeater, NCR for coverage extension with low cost

SBFD

Proposal 2: propose to consider the antenna modelling of hybrid beamforming in 6GR especially for around 7GHz 6GR BS.

Proposal 3: propose to consider the BS type 1-H at FR1 low bands as the evolution of the legacy BS type 1-C in 6G day1.

Proposal 4: propose to support the in-band NB-IoT operation for BS type 1-H and BS type 1-O in 6G day1.

Proposal 5: if LP-WUS signal is supported in 6G day1, propose to further discuss the impacts on potential EVM degradation of NR signal due to the simultaneous LP-WUS signal transmission.

Proposal 6: in order to achieve the more accurate beam steering direction in 6G day1. propose to have some study  the architecture and RF feasibility on the calibrated beamforming steering for multi-band operation BS or wide spectrum operation BS instead of defining the fractional bandwidth for different rated EIRP.

Proposal 7: for the multi-band operation, propose to consider the dynamic power sharing across different bands to improve the network capacity/coverage.

Proposal 8: for the efficient coverage extension, propose to consider the RIS deployment in 6G day1.

Proposal 9: for the sensing technology, propose to consider the 6G ISAC BS in 6G day1 at least.

Proposal 10: for minimum EVM requirement of existing 5G modulation order of 6GR, propose to apply the 5G BS EVM requirement as starting point for 6GR BS.

Proposal 11: for the optimal EVM requirement, propose to have some discussion on the necessity and evaluation method to figure out the optimal/enhanced EVM requirement to enable the achievable peak data rate if possible.

Proposal 12: for the relaxed EVM requirement for 6GR BS, propose to consider the AI or non-AI based DPoD compensation at UE side to improve the DL coverage.

Proposal 13: for the MIMO TAE requirement for 6GR BS, propose to conduct the relevant physical layer evaluation with the TAE and different MIMO layers taken into account to define the more appropriate TAE requirement;

Proposal 14: for TAE requirement among different transceivers for 6GR BS beamforming, propose to discuss the necessity and how to define the requirement for it in 6G day 1.

Proposal 15: For the intra-band contiguous CA for 6G BS, propose to consider the different requirements for different use case if necessary firstly e.g. the orthogonality to reduce the inter-carrier interference for communications, SSB-less for fast scell activation and spectrum aggregated positioning, then further discuss how and whether to specify the unified TAE requirement.

Proposal 16: For the intra-band non-contiguous CA for 6G BS, propose to discuss the necessary requirement from both with the consideration of NES with SSB-less operation in FR1 and the potential interruption due to Rx sweeping in FR2-1.

- Observation 1: the FR1 inter-band CA TAE requirement in co-located scenario as 260ns is feasible.

Proposal 17: for the inter-band co-located CA for 6G BS, propose to discuss the optimal TAE requirement considering the NES with SSB-less operation in FR1 and FR2 UE Common beam management (CBM) in FR2-1,etc.

Proposal 18: for the potential new spectrum utilization mechanism, RAN4 need to discuss and define the corresponding TAE requirement to enable the potential RRM measurement across different carriers.

- Proposal 19: for cell phase error requirement in 6G, propose to consider both the worst error requirement 3us and achievable cell phase error requirement under the normal operation mode to guide the relevant RRM requirement definition.

- Proposal 20: for cell phase requirement for CJT transmission, propose to have further discussion on the potential required phase alignment requirement with the associated UE measurement reporting information for timing misalignment, freq/ phase offset measurement and reporting. The accuracy of UE measurement reporting will also have direct impacts on the achievable performance at BS side.

- Proposal 21: for the ACLR requirement, propose to have some further study on the appropriate ACLR modelling to quantify more realistic interference modelling in the coexistence sharing study and define more proper ACLR requirement;

- Proposal 22: for the ACLR requirement, propose to consider the performance balance between ACLR and EVM requirement instead of treating these two requirement separately.

- Proposal 23: for in-band blocking requirement of FR1 BS, propose to conduct the further study with more relevant coexistence assumptions to identify the appropriate requirement for 6GR BS.

- Proposal 24: for out-of-band blocking requirement of FR1 BS, propose to conduct the further study with more relevant coexistence assumptions to identify the appropriate requirement for 6GR BS.

- Proposal 25: in order to avoid the parallel discussion on improvement on FR1 co-location requirements and coexistence spurious emission requirements between 5G-A and 6GR, propose to wait for the conclusion in 5G-A and make the necessary updates for RF requirements of 6GR BS.

Proposal 26: for the enhanced 6GR BS type 1-H, propose to discuss the additional OTA requirement in addition to EIRP and EIS requirement for BS type 1-H to reflect the radiated performance more precisely.

|  | 5G BS type 1-H | - 6GR BS type 1-H |
| --- | --- | --- |
| Tx part |  |  |
| Base station output power | Conducted, OTA | - Conducted, OTA |
| Output power dynamics | Conducted | Conducted |
| Transmit ON/OFF power | Conducted | Conducted |
| Transmitted signal quality | Conducted | OTA |
| OBW | Conducted | Conducted |
| Tx intermodulation | Conducted | Conducted |
| ACLR | Conducted | - Conducted, OTA |
| Operating band unwanted emissions | Conducted | Conducted, OTA |
| Transmitter spurious emissions | Conducted | Conducted, OTA |
| Rx part |  |  |
| REFSENS | Conducted,OTA | Conducted, OTA |
| dynamic range | Conducted | Conducted |
| ACS | Conducted | Conducted, OTA |
| blocking | Conducted | Conducted, OTA |
| NBB | Conducted | Conducted |
| RX IMD | Conducted | Conducted |
| RX NB IMD | Conducted | Conducted |
| RX spurious emission | Conducted | Conducted |
| RX ICS | Conducted | Conducted |

References

- RP-251881,New SID: Study on 6G Radio, Approved.

- R4-2511000,Further consideration on BS RF for Rel-19 LP-WUS, Noted

- RP-252952,New WID: UE RF enhancements for NR FR1, Phase 5, Approved.

- RP-25290, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, Approved

- RP-252578,Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, Approved.

- TR 38.922

- R4-2316538c,Further discussion on RF requirement impacts from NES perspective, ZTE corporation

- R4-2514638, WF on 6G BS RF and co-existence, Ericsson, approved.

Annex

| 5G BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.817-02 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Conducted requirements |  |  |  |
| Base station output power | X |  | Power accuracy and power threshold relevant requirement from 5G could be reused for 6G. |
| Output power dynamics |  |  |  |
| RE power control dynamic range |  | X | For the high modulation orders e.g. 64QAM, 256QAM and 1024QAM, this |
| Total power dynamic range |  | X | Due to the larger CBW and increasing PRB numbers, then this requirement should be updated accordingly. |
| Transmit ON/OFF | X |  |  |
| Transmitted signal quality |  |  |  |
| Frequency error | X |  | It is not expected to have implementation and deployment change |
| Modulation quality |  | X | Please find the detailed analysis in 2.3.1 and 2.3.2. |
| Time alignment error |  | X | Please find the detailed analysis in 2.3.3. |
| Unwanted emissions |  |  |  |
| Occupied bandwidth | X |  | This is from regulatory requirement and don’t see the necessity of change. |
| ACLR |  | X | This is highly dependent on the 6GR coexistence study. |
| Operating Band Unwanted Emissions |  | X | This is highly dependent on the 6GR coexistence study. |
| Transmitter spurious emission |  |  |  |
| General requirement | X |  | This is from regulatory requirement and don’t see the necessity of change. |
| Protection of BS receiver of own or different BS |  | X | For the protection of own BS receiver at the FDD band, this could be further discussed. E.g. REFSENS requirement for FDD bands with Tx on vs -96dBm/100kHz. |
| Additional spurious (coex) |  | X | For the protection of other BS’s receiver, this is dependent on the ongoing Rel-19 BS RF enhancement. |
| Co-location |  | X | For the co-location spurious emission requirement, this is also dependent on ongoing Rel-20 BS RF enhancement. |
| Transmitter intermodulation |  | X | MCL of Tx intermodulation requirement is highly dependent on ongoing Rel-20 BS RF enhancement. This requirement might be potentially impacted for FR1 high bands and candidate IMT bands between 7 to 24GHz. |
| Receiver sensitivity level |  | X | NF from 5G could be reuse for 6G. The current equation for REFSENS definition could be reused. FFS to discuss the FRC for larger channel bandwidth e.g. 200MHz or 400MHz in FR1 to simplify the conformance testing. |
| Dynamic range |  | X | The current equation for wanted signal and interfering signal could be reused. IoT level could be further checked in 6G coexistence study phase. |
| In-band selectivity and blocking |  |  |  |
| ACS |  | X |  |
| In-band blocking |  | X | Please find the detailed analysis in 2.3.7. |
| Out of band blocking |  | X | Please find the detailed analysis in 2.3.8. |
| Receiver spurious emissions | X |  |  |
| Receiver intermodulation |  | X | The power level of interference signal of Rx intermodulation might be updated to align with power level of interference signal according to the legacy practice. Propose to use the current equation to calculate the frequency offset of 6GR bandwidth. |
| In channel selectivity |  | X | The current equation for wanted signal and interfering signal could be reused. IoT level could be further checked in 6G coexistence study phase. |
| Radiated requirements |  |  |  |
| Radiated transmit power |  | X | EIRP related accuracy from 5G could be reused for 6G. Propose to have further discussion with fractional bandwidth in 6G and seek the improvement for this requirement. |
| OTA base station output power | X |  | TRP related accuracy from 5G could be reused for 6G. TRP based threshold for different 5G NR BS class could be reused for 6G. |
| OTA output power dynamics |  |  |  |
| OTA RE power control dynamic range |  | X | The same as conducted part |
| OTA Total power dynamic range |  | X | The same as conducted part |
| OTA transmit ON/OFF | X |  |  |
| OTA transmitted signal quality |  |  |  |
| OTA Frequency error | X |  |  |
| OTA Modulation quality |  | X | The same as conducted part |
| OTA Time alignment error |  | X | The same as conducted part |
| OTA Unwanted emissions |  |  |  |
| OTA Occupied bandwidth | X |  |  |
| OTA ACLR |  | X | The same as conducted part |
| OTA Operating Band Unwanted Emissions |  | X | The same as conducted part |
| OTA Transmitter spurious emission |  |  |  |
| OTA General requirement | X |  |  |
| OTA Protection of BS receiver of own or different BS |  |  | For the protection of own BS receiver at the FDD band, this could be further discussed. E.g. REFSENS requirement for FDD bands with Tx on vs -96dBm/100kHz. Whether to remove this requirement. |
| OTA Additional spurious (coex) |  | X | This is dependent on the ongoing Rel-19 BS RF enhancement. |
| OTA Co-location |  | X | This is dependent on the ongoing Rel-20 BS RF enhancement. |
| OTA transmitter intermodulation |  | X | MCL of Tx intermodulation requirement is highly dependent on ongoing Rel-20 BS RF enhancement. This requirement might be potentially impacted for FR1 high bands and candidate IMT bands between 7 to 24GHz. |
| OTA sensitivity | X |  |  |
| OTA receiver sensitivity level |  | X | Similar as conducted part |
| OTA dynamic range |  | X | Similar as conducted part |
| OTA in-band selectivity and blocking |  | X |  |
| OTA ACS |  | X | Similar as conducted part |
| OTA In-band blocking |  | X | Similar as conducted part |
| OTA out of band blocking |  | X | Similar as conducted part |
| OTA receiver spurious emissions | X |  |  |
| OTA receiver intermodulation |  | X | Similar as conducted part |
| OTA in channel selectivity |  | X | The current equation for wanted signal and interfering signal could be reused. IoT level could be further checked in 6G coexistence study phase. |