3GPP TSG-RAN WG4 Meeting#117                                R4-2521749

Dallas, USA, 17th – 21st, Nov. 2025

Agenda item:	8.4.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on 6GR coexistence study

Document for:	Approval

Introduction

In the RAN#108 meeting, a new WID on Study on 6G Radio [1] was approved. In the last RAN4 meeting, there were some initial discussions on 6G NR BS RF requirement and agreements were captured in WF [8].In this contribution, we would like to share some further understandings on 6GR coexistence study.

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - RAN4#1116bis agreement: Issue 2-1-1: Coexistence studies Way Forward: List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

Discussion

- 2.1. Coexistence study for 6GR study

- 2.1.1. Background information on coexistence study in 4G and 5G phase

- In the following table 1, we basically summarize the history of TN coexistence study in the past. In the following section, we want to further elaborate more on the detailed background of antenna modelling used for coexistence study in previous release.

- In the early stage of LTE phase e.g. Rel-8, only passive antenna with legacy RRU assumption inherited from UTRA phase is considered for the coexistence study. For elevation domain, there was no antenna pattern assumed even at the beginning. The antenna pattern for elevation domain was introduced at the late stage (probably during the AAS SI phase).

- With the BS antenna technology development, in Rel-11 LTE-A phase, one study item for AAS BS was approved at 2011 and then continued with 2 release extensive study. Antenna modelling without sub-array configuration is agreed firstly and captured in TR 38.840 and antenna modelling was further improved with typical sub-array configuration which was originated from the 6GHz and 10GHz coexistence study in Rel-17. It should be noted that in 6GHz and 10GHz coexistence study as captured in TR 38.921, antenna configuration is still based on assumption of without sub-array configuration. In the Rel-19, 15GHz coexistence study, Rel-19 U6GHz EEIRP mask evaluation and Rel-19 coexistence spurious limit evaluation, all of these evaluation cases has already considered the typical antenna configuration with the sub-array assumption.

- In Rel-14 NR SI, it was agreed to reuse the LTE ACLR/ACS requirement for NR without any further coexistence study directly. However as we shown in the following table, the coexistence study in LTE is focusing on 2GHz operating frequency and based on passive antenna at Rel-8 and based on AAS antenna modelling without sub-array configuration for UE specific beamforming at Rel-13. In addition, UE power class is also based on PC3 for eMBB UE in the past without considering PC2 as specified in FR1 FDD/TDD bands and PC1.5 for FR1 TDD bands.

- In addition, as extracted from TR 38.803, the simulation assumption for FR2 BS and UE (e.g. maximum transmission power assumption) is not fully correct which was pointed out at NR WI phase. In 6GR phase, it is reasonable to conduct more appropriate coexistence evaluation again to figure out the ACLR/ACS requirements.

Table 5.2.8-1: Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz | 200MHz | 200MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm | 23dBm | 43dBm | 33dBm |
| UE max TX power in dBm | 23dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | Note 1 | Note 1 | Note 1 |
| UE Noise figure in dB | Note 1 | Note 1 | Note 1 |
| Handover margin | 3dB | 3dB | 3dB |
| Note 1: For deriving ACIR/ACS values which are included in respond to WP5D, following NF are used in co-existence simulation study for both UE and BS. 30GHz: 9 and 11 dB, 45GHz: 11 and 13 dB, 70GHz: 13 and 15 dB. |  |  |  |

- Table 1. the summary for the legacy TN coexistence study from 4G to 5G-A

| - Release | - TR | - NOTE |
| --- | --- | --- |
| - Rel-8 | - TR 36.942 | - 2GHz Urban Macro and 900MHz Rural area, 43dBm for 5MHz and 46dBm for 10, 15, 20MHz |
| - Rel-12 | - TR 37.840 | - Single column 10x1 AAS BS without UE specific beamforming considered, 2GHz |
| - Rel-13 | - TR 37.842 | - Vertical cell splitting and horizontal cell splitting at 2GHz and UE specific beamforming |
| - Rel-14 | - TR 38.803 | - Focus on 30GHz, 45GHz and 70GHz mmWave coexistence study. |
| - Rel-17 | - TR 38.921 | - Focus on 6425-7125MHz and 10-10.5GHz with 16x8 and 8x8 antenna configuration without sub-array considered. Only PC3 UE and 43 BS transmission power is considered in the coexistence evaluation. |
| - Rel-18 | - TR 38.858 | - SBFD coexist with TDD NR in both FR1 and FR2-1 TDD bands |
| - Rel-19 | - TR 38.922 | - Focus on 15GHz evaluation and also capture the antenna configuration for 4.4-4.8GHz and 7.125-8.4GHz |

- Proposal 1: propose to consider the following key assumption difference between LTE/NR and 6GR to derive the 6GR coexistence study.

| - Operating frequency | - LTEG/5G | - 6GR difference |
| --- | --- | --- |
| - 700MHz | - LTE: Rural area evaluation only - NR: no evaluation | - AAS architecture - PC2 HPUE with single PC2 PA and PC1.5 HPUE with 2 PC2 PA |
| - 2GHz | - LTE: Urban Macro area evaluation only - (passive antenna in Rel-8 and AAS with UE specific beamforming in Rel-13, without sub-array configuration) - NR: No evaluation | - AAS architecture with sub-array configuration; - PC2 HPUE with single PC2 PA and PC1.5 HPUE with 2 PC2 PA |
| - 4GHz | - LTE: No evaluation - NR: No evaluation | - AAS architecture with sub-array configuration; - PC2 HPUE with single PC2 PA and PC1.5 HPUE with single PC1.5 PA or with 2 PC2 PA - (NOTE: there is no difference for coexistence study for PC1.5 with 1 PA or 2 PA) |
| - 7GHz | - LTE: No evaluation - NR: With evaluation - AAS without sub-array configuration (16x8), - BS transmission power 43dBm, - PC3 UE only | - AAS architecture with sub-array configuration (e.g. 1x3); - Larger antenna array size (e.g. 8x16 with sub-array as 1x3 in elevation domain) - Hybrid beamforming at BS side; - PC2 HPUE with single PC2 PA and PC1.5 HPUE with 2 PC2 PA (e.g. with certain beamforming gain using omini antenna); 4 PC3 PA with certain beamforming using the omini antenna |
| - 30GHz | - NR: With evaluation in Rel-14 | - More reasonable BS and UE transmission power assumption e.g. 30-40dBm for 512 antenna configuration or 22.4dBm for minimum EIRP for UE side; |

In addition, ACLR requirement is a critical RF requirement to guarantee the coexistence performance between adjacent carries of the same RAT or different 3GPP RAT. Meanwhile ACLR requirement is also one of the important parameters to dimension the RF design especially from potentially different PA design and different power consumption for the overall BS, the appropriate interference antenna modelling in the adjacent channel is extremely important to quantify the interference and figure out the required ACLR requirement.

Indeed, the spatial antenna modelling in the adjacent channel was discussed in the past and could be dated back to the AAS SI phase in Rel-12 phase. The composite radiation gain pattern for pure LOS channel can be expressed in logarithmical scale as captured in TR 37.840:

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2−1

Where ρ is the correlation level for the unwanted emission in the adjacent channel.

However after the Rel-12 LTE AAS SI, there were some contiguous discussions in both 3GPP and ITU-R WP5D on how to justify the more appropriate correlation level for the unwanted emissions for sharing study in ITU-R. During the Rel-19 SI for WP5D LS on IMT parameters of 4.4-4.8GHz, 7.125-8.4GHz and 14.8-15.35GHz, there were some further discussions on antenna modelling of unwanted emission in the adjacent channel, the array correlation factor roll-off model according to frequency offset away from the wanted signal carrier bandwidth edge is captured as following (extracted from TR 38.922).

Figure 2.3.5-1: Array correlation factor roll-off model. [TR 38.922]

Using the approach above the antenna model can be extended to capture array antenna gain outside the wanted carrier as:

AAθ,φ,f=Asubθ,φ+10log101+ρfm=1Mn=1Nwm,nvm,n2−1

In addition, the potential relationship between ACLR requirement and EVM requirement should be also considered. In the past, these two requirement was discussed separately in RAN4, however in the practical implementation, there are some implicit relationship e.g. more relaxed ACLR requirement will also result in the worse EVM requirement within the wanted carrier which will cause the negative impacts on the actual network performance.

- Proposal 2: for the ACLR requirement, propose to have some further study on the appropriate ACLR modelling to quantify more realistic interference modelling in the coexistence sharing study and define more proper ACLR requirement;

- 2.1.2. Coexistence cases for 6GR study

- As analyzed in section 2.1.1, RAN4 need to conduct the 6GR coexistence studies to identify the appropriate ACLR/ACS requirements. In the following Table 2.1.2-1, according to the confirmed IMT spectrum by ITU-R Radio regulation, we provide some initial understandings on the coexistence cases in 6GR study phase.

Table 2.1.2-1. coexistence case for 6GR study

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 low bands [e.g. 700MHz] | 6GR with AAS (e.g.up to 64 antenna elements) | 6GR with AAS (e.g. up to 64 antenna elements) |
| 2 | FR1 low bands [e.g. 700MHz] | 6GR with AAS (e.g.up to 64 antenna elements) | 5G with passive antenna |
| 3 | FR1 bands [e.g. 2GHz] | 6GR with AAS (e.g.up to 288 antenna elements) | 6GR with AAS (e.g.up to 288 antenna elements) |
| 4 | FR1 bands [e.g. 2GHz] | 6GR with AAS (e.g.up to 288 antenna elements) | 5G with legacy antenna array [192 antenna elements] |
| 5 | FR1 Mid bands [e.g. 4GHz] | 6GR with larger antenna array (e.g. Up to 576 Tx and Rx antenna elements) | 6GR with larger antenna array (e.g. Up to 576 Tx and Rx antenna elements) |
| 6 | FR1 Mid bands [e.g. 4GHz] | 6GR with larger antenna array (e.g. Up to 576 Tx and Rx antenna elements) | 5G with legacy antenna array [192 antenna elements] |
| 7 | Around 7GHz (NOTE1,2) | 6GR with larger antenna array (e.g. Up to 2304 Tx and Rx antenna elements) | 6GR with larger antenna array (e.g. Up to 2304 Tx and Rx antenna elements) |
| 8 | Around 7GHz (NOTE1,2) | 6GR with larger antenna array (e.g. Up to 2304 Tx and Rx antenna elements) | 5G with legacy antenna array [192 antenna elements] |
| 9 | FR2-1 30 GHz (NOTE 3) | 6GR with AAS | 6GR with AAS |
| 10 | FR2-1 30GHz (NOTE 3) | 6GR with AAS | 5G with AAS |
| NOTE 1: 6GR non-SBFD BS coexists with 6GR non-SBFD BS BS with larger antenna array than assumption in TR 38.921. NOTE 2: 6GR SBFD BS coexists with 6GR SBFD BS; NOTE 3: The simulation assumption captured in TR38.803 is too aggressive with BS maximum output power up to 43dBm. |  |  |  |

Proposal 3: for the 6GR coexistence study, propose to consider the coexistence cases as listed in Table 2.1.2-1.

- 2.1.3. Simulation assumption for 6GR study

- For the simulation assumption for 6GR study, we want to provide the following simulation assumptions as captured in Table 2.1.3-1 and Table 2.1.3-2 for the 6GR coexistence study.

- Table 2.1.3-1. general parameters for up to 7GHz coexistence study

| - FR | - 700MHz | - 2GHz | - 4GHz |
| --- | --- | --- | --- |
| - Network layout | - UMA, - Dense Urban, - Indoor | - UMA, - Dense Urban, - Indoor | - UMA, - Dense Urban, - Indoor |
| - Propagation model | - Refer to 38.901 | - Refer to 38.901 | - Refer to 38.901 |
| - BS Antenna configuration | - Around 700 MHz: Up to 64 Tx and Rx antenna elements | - Around 4 GHz: Up to 288 Tx and Rx antenna elements | - Around 4 GHz: Up to 576 Tx and Rx antenna elements |
| - UE Antenna configuration | - Omini antenna | - Omini antenna | - Omini antenna |
| - Transmission power control | - Refer to 38.922 | - Refer to 38.922 | - Refer to 38.922 |
| - ACLR/ACS modelling | - Depends ACLR modelling | - Depends ACLR modelling | - Depends ACLR modelling |
| - Link level performance for 6GR coexistence study | - Refer to 38.922 | - Refer to 38.922 | - Refer to 38.922 |
| - Coexistence simulation methodology | - Refer to 38.922 | - Refer to 38.922 | - Refer to 38.922 |

| - FR | - Around 7GHz |  |  |
| --- | --- | --- | --- |
| - Network layout | - UMA, - Dense Urban, - Indoor |  |  |
| - Propagation model | - Refer to 38.901 |  |  |
| - BS Antenna configuration | - Around 7 GHz: Up to 2304 Tx and Rx antenna elements |  |  |
| - UE Antenna configuration | - Omini antenna and - Beamforming with multiple omini antenna |  |  |
| - Transmission power control | - Refer to 38.922 |  |  |
| - ACLR/ACS modelling | - Depends ACLR modelling |  |  |
| - Link level performance for 6GR coexistence study | - Refer to 38.922 |  |  |
| - Coexistence simulation methodology | - Refer to 38.922 |  |  |

- Table 2.1.3-2. general parameters for 30GHz coexistence study.

| - FR | - 30GHz | - 40GHz | - Remarks |
| --- | --- | --- | --- |
| - Network layout | - UMA, - Dense Urban, - Indoor |  |  |
| - Propagation model | - Refer to 38.901 |  |  |
| - Antenna configuration | - Around 30 GHz: Up to 4096 Tx and Rx antenna elements |  |  |
| - Transmission power control | - Refer to 38.803 |  |  |
| - ACLR/ACS modelling | - Depends ACLR modelling |  |  |
| - Link level performance for 6GR coexistence study | - Refer to 38.922 |  |  |
| - Coexistence simulation methodology | - Refer to 38.922 |  |  |

Proposal 4: for the 6GR coexistence study, propose to consider the simulation assumptions as listed in Table Table 2.1.3-1 and Table 2.1.3-2.

Conclusions

In this contribution, we shared some initial views on the 6GR BS RF study and proposals are made as following:

- Proposal 1: propose to consider the following key assumption difference between LTE/NR and 6GR to derive the 6GR coexistence study.

| - Operating frequency | - LTEG/5G | - 6GR difference |
| --- | --- | --- |
| - 700MHz | - LTE: Rural area evaluation only - NR: no evaluation | - AAS architecture - PC2 HPUE with single PC2 PA and PC1.5 HPUE with 2 PC2 PA |
| - 2GHz | - LTE: Urban Macro area evaluation only - (passive antenna in Rel-8 and AAS with UE specific beamforming in Rel-13, without sub-array configuration) - NR: No evaluation | - AAS architecture with sub-array configuration; - PC2 HPUE with single PC2 PA and PC1.5 HPUE with 2 PC2 PA |
| - 4GHz | - LTE: No evaluation - NR: No evaluation | - AAS architecture with sub-array configuration; - PC2 HPUE with single PC2 PA and PC1.5 HPUE with single PC1.5 PA or with 2 PC2 PA - (NOTE: there is no difference for coexistence study for PC1.5 with 1 PA or 2 PA) |
| - 7GHz | - LTE: No evaluation - NR: With evaluation - AAS without sub-array configuration (16x8), - BS transmission power 43dBm, - PC3 UE only | - AAS architecture with sub-array configuration (e.g. 1x3); - Larger antenna array size (e.g. 8x16 with sub-array as 1x3 in elevation domain) - PC2 HPUE with single PC2 PA and PC1.5 HPUE with 2 PC2 PA (e.g. with certain beamforming gain using omini antenna); 4 PC3 PA with certain beamforming using the omini antenna |
| - 30GHz | - NR: With evaluation in Rel-14 | - More reasonable BS and UE transmission power assumption e.g. 30-40dBm for 512 antenna configuration or 22.4dBm for minimum EIRP for UE side; |

- Proposal 2: for the ACLR requirement, propose to have some further study on the appropriate ACLR modelling to quantify more realistic interference modelling in the coexistence sharing study and define more proper ACLR requirement;

Proposal 3: for the 6GR coexistence study, propose to consider the coexistence cases as listed in Table 2.1.2-1.

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 low bands [e.g. 700MHz] | 6GR with AAS (e.g.up to 64 antenna elements) | 6GR with AAS (e.g. up to 64 antenna elements) |
| 2 | FR1 low bands [e.g. 700MHz] | 6GR with AAS (e.g.up to 64 antenna elements) | 5G with passive antenna |
| 3 | FR1 bands [e.g. 2GHz] | 6GR with AAS (e.g.up to 288 antenna elements) | 6GR with AAS (e.g.up to 288 antenna elements) |
| 4 | FR1 bands [e.g. 2GHz] | 6GR with AAS (e.g.up to 288 antenna elements) | 5G with legacy antenna array [192 antenna elements] |
| 5 | FR1 Mid bands [e.g. 4GHz] | 6GR with larger antenna array (e.g. Up to 576 Tx and Rx antenna elements) | 6GR with larger antenna array (e.g. Up to 576 Tx and Rx antenna elements) |
| 6 | FR1 Mid bands [e.g. 4GHz] | 6GR with larger antenna array (e.g. Up to 576 Tx and Rx antenna elements) | 5G with legacy antenna array [192 antenna elements] |
| 7 | Around 7GHz (NOTE1,2) | 6GR with larger antenna array (e.g. Up to 2304 Tx and Rx antenna elements) | 6GR with larger antenna array (e.g. Up to 2304 Tx and Rx antenna elements) |
| 8 | Around 7GHz (NOTE1,2) | 6GR with larger antenna array (e.g. Up to 2304 Tx and Rx antenna elements) | 5G with legacy antenna array [192 antenna elements] |
| 9 | FR2-1 30 GHz (NOTE 3) | 6GR with AAS | 6GR with AAS |
| 10 | FR2-1 30GHz (NOTE 3) | 6GR with AAS | 5G with AAS |
| NOTE 1: 6GR non-SBFD BS coexists with 6GR non-SBFD BS BS with larger antenna array than assumption in TR 38.921. NOTE 2: 6GR SBFD BS coexists with 6GR SBFD BS; NOTE 3: The simulation assumption captured in TR38.803 is too aggressive with BS maximum output power up to 43dBm. |  |  |  |

Proposal 4: for the 6GR coexistence study, propose to consider the simulation assumptions as listed in Table Table 2.1.3-1 and Table 2.1.3-2.

- Table 2.1.3-1. general parameters for up to 7GHz coexistence study

| - FR | - 700MHz | - 2GHz | - 4GHz |
| --- | --- | --- | --- |
| - Network layout | - UMA, - Dense Urban, - Indoor | - UMA, - Dense Urban, - Indoor | - UMA, - Dense Urban, - Indoor |
| - Propagation model | - Refer to 38.901 | - Refer to 38.901 | - Refer to 38.901 |
| - BS Antenna configuration | - Around 700 MHz: Up to 64 Tx and Rx antenna elements | - Around 4 GHz: Up to 288 Tx and Rx antenna elements | - Around 4 GHz: Up to 576 Tx and Rx antenna elements |
| - UE Antenna configuration | - Omini antenna | - Omini antenna | - Omini antenna |
| - Transmission power control | - Refer to 38.922 | - Refer to 38.922 | - Refer to 38.922 |
| - ACLR/ACS modelling | - Depends ACLR modelling | - Depends ACLR modelling | - Depends ACLR modelling |
| - Link level performance for 6GR coexistence study | - Refer to 38.922 | - Refer to 38.922 | - Refer to 38.922 |
| - Coexistence simulation methodology | - Refer to 38.922 | - Refer to 38.922 | - Refer to 38.922 |

| - FR | - Around 7GHz |  |  |
| --- | --- | --- | --- |
| - Network layout | - UMA, - Dense Urban, - Indoor |  |  |
| - Propagation model | - Refer to 38.901 |  |  |
| - BS Antenna configuration | - Around 7 GHz: Up to 2304 Tx and Rx antenna elements |  |  |
| - UE Antenna configuration | - Omini antenna and - Beamforming with multiple omini antenna |  |  |
| - Transmission power control | - Refer to 38.922 |  |  |
| - ACLR/ACS modelling | - Depends ACLR modelling |  |  |
| - Link level performance for 6GR coexistence study | - Refer to 38.922 |  |  |
| - Coexistence simulation methodology | - Refer to 38.922 |  |  |

- Table 2.1.3-2. general parameters for 30GHz coexistence study.

| - FR | - 30GHz | - 40GHz | - Remarks |
| --- | --- | --- | --- |
| - Network layout | - UMA, - Dense Urban, - Indoor |  |  |
| - Propagation model | - Refer to 38.901 |  |  |
| - Antenna configuration | - Around 30 GHz: Up to 4096 Tx and Rx antenna elements |  |  |
| - Transmission power control | - Refer to 38.803 |  |  |
| - ACLR/ACS modelling | - Depends ACLR modelling |  |  |
| - Link level performance for 6GR coexistence study | - Refer to 38.922 |  |  |
| - Coexistence simulation methodology | - Refer to 38.922 |  |  |

References

- RP-251881,New SID: Study on 6G Radio, Approved.

- R4-2511000,Further consideration on BS RF for Rel-19 LP-WUS, Noted

- RP-252952,New WID: UE RF enhancements for NR FR1, Phase 5, Approved.

- RP-25290, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, Approved

- RP-252578,Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, Approved.

- TR 38.922

- R4-2316538c,Further discussion on RF requirement impacts from NES perspective, ZTE corporation

- R4-2514638, WF on 6G BS RF and co-existence, Ericsson, approved.

- RP-220335, CR to TR 38.803: Addition of array antenna model extension in subclause 5.2.3

Annex

The enhanced AAS antenna model for FR1 hybrid beamforming is proposed as following.

Wm3=1Msub3expj∙2πm3−1dv,subλ∙sinθsubtilt3

Sub-array pattern #3 with fixed down-tilt could be defined as follows:

Asub3θ,φ=AEθ,φ+10log10m3=1Msub3Wm3∙Vm32

where,

Vm3=expj∙2πm3−1dv,subλ∙cosθ

Sub-array pattern #2 with variable electrical down-tilt could be defined as follows:

Asub2θ,φ=Asub3θ,φ+10log10m2=1Msub2Wm2∙Vm22

where,

Wm2=1Msub2expj∙2πm2−1dv,sub∙Msub3λ∙sinθsubtilt2

Vm2=expj∙2πm2−1dv,sub∙Msub3λ∙cosθ

Composite antenna for the hybrid beamforming with digital beamforming could be defined as follows:

Acompositeθ,φ=Asub2θ,φ+10log10m1=1Mn=1NWm1,n∙Vm1,n2

where,

Wm1,n=1M∙Nexpj∙2πm−1dvλ∙sinθtilt−n−1dhλcosθtilt∙sinφscan

dv=α∙Msub3∙dv,sub

Vm1,n=expj∙2πm−1dvλ∙cosθ+n−1dhλsinθsinφ1