# 3GPP TSG-RAN WG4 Meeting #117	R4-2521752

# Dallas,TX, USA, November 17 – 21, 2025

Agenda item:			8.1

Source:	Feature lead (ZTE Corporation)

Title:	Running Summary of 6G Sensing

Document for:	Information

Introduction

Based on the latest approved RAN working group level SID on 6G Radio [RP-252912], RAN4 study on sensing including RF, coexistence, testability, with the detailed objective provided as:

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In this running summary, it is intended to include the following aspects relevant to the RAN4 study on 6G sensing:

- Status update for 6G RAN4 AI/ML issues, which is used to track the status of different objectives

- RAN4 key Tdocs and key agreements achieved before this meeting.

- RAN1 summary for relevant and essential agreements related to RAN4 6G Sensing

- RAN2 summary for relevant and essential agreements related to RAN4 6G Sensing

The information for feature lead is provided:

| Feature Lead | Company | Email address |
| --- | --- | --- |
| Fei Xue | ZTE Corporation | xue.fei25@zte.com.cn |

Status update

The following status is provided based on RAN4 discussion till the end of RAN#116bis:

- To have initial discussions on RAN4 agenda setting for sensing and RAN4 aspects which is less dependent with RAN1 design

RAN4 - Sensing related agreements summary

RAN4 #116bis meeting (Oct-2025)

Key Tdocs

| R4-2514516 | R4-2514516 Topic Summary for [116bis][109] 6G sensing |
| --- | --- |
| R4-2514592 | WF on 6G sensing |

Key agreements

General scope and timeline

Before April 2026, the agenda for sensing will be kept. RAN4 discussion will primarily focus on the following aspect

- Sensing related regulatory status and requirement survey

- Potential architecture consideration based on the RAN plenary use case study

- views sharing on the potential RAN4 scope based on the identified use cases out of RAN plenary study

- Identify less RAN1 design dependent RAN4 aspects if any

RAN – Sensing related agreements summary

Sensing in 6GR interface

RAN #109 (Sep-2025)

- Agreement:

Editor note:	More sensing use cases can be included depending on further discussion.

The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

| RP-252960 | pCR for 38.914 on Requirements of new and existing services for 6G |
| --- | --- |

RAN1 – Sensing related agreements summary

Sensing in 6GR interface

Guidance in RAN1 chair notes

Including PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…), aspects of integration with communication services.

Placeholder only and to be broken down. No contributions before RAN1#124b.

RAN1 #122bis (Oct-2025)

RAN1 #122 (Aug-2025)

Sensing in NR interface

RAN1 #122bis (Oct-2025)

Performance metrics

Agreement

On False alarm probability Type 2 (targets dropped in simulation area), the parameter K is the number of drops with at least one detected object.

Conclusion

Sensing resolution is not considered as a performance metric for evaluation of NR ISAC.

Agreement

The following performance objectives are adopted for evaluation purpose of NR ISAC.

| Metric | Value |
| --- | --- |
| Missed detection Probability | [5]% |
| False Alarm Probability Type 1 | [5]% |
| False Alarm Probability Type 2 | [5]% |
| Horizontal Positioning Accuracy | [10] m with confidence level 90% |
| Vertical Positioning Accuracy | [10] m with confidence level 90% |
| Velocity Accuracy | [5] m/s with confidence level 90% |

Simulation assumptions

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |  |
| --- | --- | --- |
| BS antenna configuration | For 4GHz, 4.9GHz: - Option 1 as optional - Tx: (12,16, 2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) - Option 2 - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) | For 6GHz: - Option 1 as optional - Tx: (16,16, 2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) - Option 2: - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) |
|  | Mandatory: dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization |  |

- Note: Tx antenna elements and Rx antenna elements are operating simultaneously

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 |
| BS antenna mechanic tilt (downtilt angle without electrical tilt) | 90° in GCS (pointing to horizontal direction) |
| BS antenna electrical tilt (downtilt angle after mechanic and electrical tilt) | Option 1: no electrical tilt Option 2: 102° in GCS |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| gNB-target link | gNB-aerial UE of UMa/RMa parameters in 36.777 with Alternative 3 |
| Concatenation of TX-target and target-RX links | Up to company choice between two options for concatenation defined in Step 9 in section 7.9.4.1, TR 38.901 |
| The power threshold for path dropping after concatenation for target channel | -25dB and -40dB are respectively used for the two options for concatenation |

Agreement

Up to company to model low power cluster in the evaluation of NR ISAC

Agreement

When Tx/Rx operates simultaneously (baseline case), the following parameters are used for the evaluation on NR ISAC:

| Antenna isolation | - Option 1: 65dB - Option 2: 80dB |
| --- | --- |
| Maximum BS Tx power | - Option 1: 37dBm - Option 2: 52dBm The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively |

- Optionally: companies should report the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived.

- Agreement

When Tx/Rx operates simultaneously, the following evaluation parameters are agreed for the evaluation on NR ISAC.

| Self-interference | The residual leakage interference/noise is modelled e.g. by additional additive white Gaussian noise, -94+X dBm in 100 MHz, X is up to company report. Companies to provide details on their modelling. Note: X = -Infinity corresponds to not modelling self-interference |
| --- | --- |

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| Co-site inter-sector interference | Not modelled |
| Adjacent channel interference | Not modelled |
| BS receiver noise figure | 5dB |

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| CPI (coherent processing interval) | Companies to report the CPI value assumed |
| Waveform | CP-OFDM as baseline, other waveform is up to companies report |

Agreement

For the evaluation on NR ISAC,

- 3 kinds of resources are defined

- Type 1: resources that are used for sensing signal transmission

- Type 2: part of Type 1 resources that are used for communication purpose

  - Note: it is possible the resource type 2 doesn’t exist

- Type 3: resources that are not used for sensing signal transmission, and cannot be used for communication purpose due to sensing operation

- Both options should be reported for sensing resource ratio

- Option 1: (Type_1 + Type_3) resources over all radio DL and UL resources

- Option 2: (Type_1 - Type_2 + Type_3) resources over all radio DL and UL resources

- Note: if Type 2 resource doesn’t exist, two options are the same

For the evaluation on NR ISAC,

- Company should report T/F RE mapping of sensing RS, assumed TDD UL/DL configuration if applicable.

- Company should report which sensing RS resources are considered as Type 2 resource and related reason.

Agreement

Companies are encouraged to report the high-level sensing signal/data processing method used in the evaluation, e.g., 2D FFT, MUSIC, and any other methods.

Agreement

Companies should report details of the Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP.

- The details of the Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP will be captured along with the corresponding evaluation results in the Rel-20 TR

RAN1 #122 (Aug-2025)

Performance metrics

Agreement

Horizontal/vertical positioning accuracy are agreed as performance metrics for NR ISAC.

  - It is defined as the absolute value of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target.

    - Note: in RAN1 evaluations, there should be only one estimated horizontal/vertical position corresponding to the true position of a sensing target

Agreement

For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions:

  - One true target is associated with at most one detected object

  - One detected object is associated with at most one true target

  - The same association applies to miss detection, false alarm probability Type 2 (if defined) and positioning/velocity accuracy

  - Companies to report the method used for association of the detected object(s) and the true target(s)

Agreement

- Missed detection probability is agreed as performance metric for NR ISAC.

  - It is defined as the conditional probability of not detecting the presence of a target when the target is actually present in the simulation area.

Pmd=n=0N−1DnMnN

Where,

      - Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object

      - Mn is the number of true targets in the drop n.

      - N is total number of drops with at least one target per drop

Agreement

- False alarm probability is agreed as performance metric for NR ISAC.

  - For cases without true target dropped in simulation area, False alarm probability Type 1 is computed and reported

  - For cases with true targets dropped in simulation area, False alarm probability Type 2 is computed and reported

  - Note: both False alarm probability Types are mandatory

  - KPI values for False alarm probability Type 1 and 2 can be discussed separately.

Definitions:

  - False alarm probability Type 1 (no target dropped in simulation area): An object is detected when there is no target present in simulation area is considered a false alarm.

Pf1=n=0N−1QnN

Where,

    - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0.

    - N is the total number of drops without targets in the simulation area.

  - False alarm probability Type 2 (targets dropped in simulation area): An object is detected but not associated with any true targets in the simulation area is considered as a false alarm.

Pf2=0≤n<NMn'≠0Dn'Mn'K

Where,

      - Dn' is the number of detected objects but not associated with any true targets in the drop n.

      - Mn' is the total number of detected objects in the drop n.

      - FFS:

        - Option 1: K is number of drops (N)

        - Option 2: K is number of drops with at least one detected object

Note: the number of targets should be reported by companies when providing False alarm probability Type 2

Agreement

- Velocity accuracy is agreed as performance metric for NR ISAC.

  - Velocity accuracy is defined as the absolute value of the difference between the estimated velocity and the corresponding true velocity of a sensing target.

  - For single TRP monostatic sensing,

    - The radial velocity accuracy can be estimated

      - The true radial velocity is the projection of true velocity on the direction from TRP to target for TRP monostatic.

    - The true velocity accuracy can be estimated.

Simulation assumptions

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| Scenario | UMa-AV, Optional RMa-AV |
| Carrier frequency | Mandatory: one value either 4 GHz or 4.9GHz. optional for FR1: 6 GHz [optional for FR2: 28 GHz] |
| System bandwidth | 100 MHz |
| Numerology | SCS = 30kHz |
| BS Layout | Hexagonal grid, 7 macro sites, 3 sectors per site. See Note1 3 sectors with 30, 150, 270 degrees |
| Inter-BS (2D) distance | Uma-AV: 500m, optional 1000m, RMa-AV: 1732m |
| BS antenna height | 25m for UMa-AV, 35m for RMa-AV |
| Wrap-round | No wrap-round |

Note1: target(s) are dropped only in the center site, and inter-BS interference is not modelled.

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| Target type | UAV with small size (0.3m x 0.4m x 0.2m) |
| Target distribution when target(s) are dropped | N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m |
| Mobility | horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h |

Note: N=0 will be discussed in a later proposal in relation to false alarm.

Agreement

The following evaluation parameters are agreed for the evaluation on NR ISAC.

| Parameters | Assumptions |
| --- | --- |
| Minimum BS-target 3D distance | 10 m |
| Minimum target-target (3D) distance | 10 m |
| Target outdoor/indoor proportion | 100% outdoor |
| LOS/NLOS | LOS and NLOS |
| Orientation | Random in horizontal domain |
| RCS model | RCS model 1 for UAV with small size |

RAN2 – Sensing related agreements summary

Sensing in 6GR interface

RAN2 #131bis (Oct-2025)

Agreement:

RAN2 will start sensing study in April 2026 .